3GPP TSG- RAN WG4 Meeting # 108bis	R4-2317372

# Xiamen, China,  Oct 09 - Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR |  | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.106 the introduction of BFD/BFR/RLM for NCR-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the agreement reached in R4-2310084,R4-2306360, it was agreed to introduce the BFD/BFR/RLM requirement for Local area NCR-MT only, however the corresponding RRM requirements are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the BFD/BFR/RLM requirement for Local area NCR-MT only |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BFD/BFR/RLM requirement for Local area NCR-MT is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.1, 14.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TR 25.942: "RF system scenarios".

[4]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”.

[7]	3GPP TS 38.115-1: “NR; Repeater conformance testing - Part 1: Conducted conformance testing”.

[8]	3GPP TS 38.115-2: “NR; Repeater conformance testing - Part 2: Radiated conformance testing”.

[9]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

[10]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[11]	Void

[12]	Void

[13]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[14]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”.

[15]	Void

[16]	Void

[17]	Void

[18]	Void

[19]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[20]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[21]	3GPP TS 38.133: “NR: Requirements for support of radio resource management”

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.213: "NR; Physical layer procedures for control".

< NEXT OF CHANGE>

14.3	Signalling Characteristics for NCR-MT

14.3.1	Radio Link Monitoring

14.3.1.1	Introduction

The requirements in clause 8.1 apply for radio link monitoring on:

-	PCell in SA NR,

The NCR-MT shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell as specified in TS 38.213 [TBD]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. NCR-MT is not required to perform RLM outside the active DL BWP.

On each RLM-RS resource, the NCR-MT shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell.

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout) as defined in Table 14.3.1-1. For SSB based radio link monitoring, Qout_SSB is derived based on the hypothetical PDCCH transmission parameters listed in Table 14.3.1.2.1-1. For CSI-RS based radio link monitoring, Qout_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 14.3.1.3.1-1.

The threshold Qin is defined as the level at which the downlink radio link quality can be received with significantly higher reliability than at Qout and shall correspond to the in-sync block error rate (BLERin) as defined in Table 14.3.1-1. For SSB based radio link monitoring, Qin_SSB is derived based on the hypothetical PDCCH transmission parameters listed in Table 14.3.1.2.1-2. For CSI-RS based radio link monitoring, Qin_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 14.3.1.3.1-2.

The out-of-sync block error rate (BLERout) and in-sync block error rate (BLERin) are determined from the network configuration via parameter rlmInSyncOutOfSyncThreshold signalled by higher layers. When NCR-MT is not configured with rlmInSyncOutOfSyncThreshold from the network, NCR-MT determines out-of-sync and in-sync block error rates from Configuration #0 in Table 14.3.1-1 by default. All requirements in clause 14.3.1 are applicable for BLER Configuration #0 in Table 14.3.1-1.

Table 14.3.1-1: Out-of-sync and in-sync block error rates

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |

NCR-MT shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number  of SSBs per half frame according to TS 38.213 [TBD], where NRLM is specified in Table 14.3.1-2  according TS 38.213 [TBD], and meet the requirements as specified in clause 14.3.1. NCR-MT is not required to meet the requirements in clause 14.3.1 if RLM-RS is not configured and no TCI state for PDCCH is activated.

Table 14.3.1-2: Maximum number of RLM-RS resources NRLM

| Carrier frequency range of PCell |  | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| FR2-1 | 64 | 8 |

14.3.1.2	Requirements for SSB based radio link monitoring

14.3.1.2.1	Introduction

The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell, provided that the SSB configured for RLM is actually transmitted within NCR-MT active DL BWP during the entire evaluation period specified in clause 14.3.1.2.2.

Table 14.3.1.2.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 14.3.1.2.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

14.3.1.2.2	Minimum requirement

NCR-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period.

NCR-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 14.3.1.2.2-1 for FR1 with scaling factor K1 = 5.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 14.3.1.2.2-2 for FR2 with scaling factor N=8 and K2 = 3.

For FR1,

-	P = 1

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when RLM-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

If the NCR-MT is not capable of 4 SMTC configurations per frequency [22], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [22], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the combination of RLM-RS resource and SMTC occasion configurations does not meet previous conditions.

Table 14.3.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200 K1, Ceil(10 P K1) TSSB) | Max(100 K1, Ceil(5 P K1) TSSB) |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. |  |  |

Table 14.3.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200 K2, Ceil(10 P N K2) TSSB) | Max(100 K2, Ceil(5 P N K2) TSSB) |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. |  |  |

14.3.1.2.3	Measurement restrictions for SSB based RLM

The NCR-MT is required to be capable of measuring SSB for RLM without measurement gaps. The NCR-MT is required to perform the SSB measurements with measurement restrictions as described in the following scenarios.

For FR1, when the SSB for RLM is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, NCR-MT shall be able to measure the SSB for RLM without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If NCR-MT supports simultaneousRxDataSSB-DiffNumerology, NCR-MT shall be able to measure the SSB for RLM without any restriction;

-	If NCR-MT does not support simultaneousRxDataSSB-DiffNumerology, NCR-MT is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

For FR2, when the SSB for RLM measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, NCR-MT is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

14.3.1.3	Requirements for CSI-RS based radio link monitoring

14.3.1.3.1	Introduction

The requirements in this clause apply for each CSI-RS based RLM-RS resource configured for PCell, provided that the CSI-RS configured for RLM is actually transmitted within NCR-MT active DL BWP during the entire evaluation period specified in clause 14.3.1.3.2. NCR-MT is not expected to perform radio link monitoring measurements on the CSI-RS configured as RLM-RS if the CSI-RS is not in the active TCI state of any CORESET configured in the NCR-MT active BWP.

Table 14.3.1.3.1-1: PDCCH transmission parameters for out-of-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 4dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 14.3.1.3.1-2: PDCCH transmission parameters for in-sync evaluation

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

14.3.1.3.2	Minimum requirement

NCR-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS [ms] period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS [ms] evaluation period.

NCR-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS [ms] period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS [ms] evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 14.3.1.3.2-1 for FR1 with scaling factor K1 = 5.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 14.3.1.3.2-2 for FR2 with scaling factor K2 = 3.

The requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For FR1,

-	P=1 .

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = 3, when the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

If the NCR-MT is not capable of 4 SMTC configurations per frequency [22], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [22], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

NOTE:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

Longer evaluation period would be expected if the combination of RLM-RS resource and SMTC occasion configurations does not meet previous conditions.

The values of Mout and Min used in Table 14.3.1.3.2-1 and Table 14.3.1.3.2-2 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [8, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

Table 14.3.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200 K1, Ceil(Mout×P K1)×TCSI-RS) | Max(100 K1, Ceil(Min×P K1) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. |  |  |

Table 14.3.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200 K2, Ceil(Mout×P K2)×TCSI-RS) | Max(100 K2, Ceil(Min×P K2) × TCSI-RS) |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. |  |  |

14.3.1.3.3	Measurement restrictions for CSI-RS based RLM

The NCR-MT is required to be capable of measuring CSI-RS for RLM without measurement gaps. The NCR-MT is required to perform the CSI-RS measurements with measurement restrictions as described in the following clauses.

For both FR1 and FR2, when the CSI-RS for RLM is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, NCR-MT is not required to receive CSI-RS for RLM in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD, or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for RLM, the NCR-MT shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for RLM, the NCR-MT shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the NCR-MT supports simultaneousRxDataSSB-DiffNumerology the NCR-MT shall be able to perform CSI-RS for RLM measurement without restrictions.

-	If the NCR-MT does not support simultaneousRxDataSSB-DiffNumerology, NCR-MT is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For FR1, when the CSI-RS for RLM is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, NCR-MT shall be able to measure the CSI-RS for RLM without any restriction.

For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, NCR-MT is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, NCR-MT is required to measure one of but not both CSI-RS for RLM and the other CSI-RS. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

-	The CSI-RS for RLM or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to NCR-MT,

-	Otherwise, NCR-MT shall be able to measure the CSI-RS for RLM without any restriction.

14.3.1.4	Minimum requirement for NCR-MT turning off the transmitter

The transmitter power of the NCR-MT in the monitored cell shall be turned off within 40ms after expiry of T310 timer as specified in TS 38.331 [2].

14.3.1.5	Minimum requirement for L1 indication

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, layer 1 of the NCR-MT shall send an out-of-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [22].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, layer 1 of the NCR-MT shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [22].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 [23]. Two successive indications from layer 1 shall be separated by at least TIndication_interval.

TIndication_interval is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 14.3.1.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 14.3.1.3 if the RLM-RS resource is CSI-RS.

14.3.1.6	Scheduling availability of NCR-MT during radio link monitoring

When the reference signal to be measured for RLM has different subcarrier spacing than PDSCH/PDCCH or is on frequency range 2, there are restrictions on the scheduling availability as described in the following clauses.

##### 14.3.1.6.1	Scheduling availability of NCR-MT performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to radio link monitoring performed with a same subcarrier spacing as PDSCH/PDCCH on FR1.

##### 14.3.1.6.2	Scheduling availability of NCR-MT performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1

For NCR-MTs which support simultaneousRxDataSSB-DiffNumerology [TBD] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For NCR-MTs which do not support simultaneousRxDataSSB-DiffNumerology [TBD] the following restrictions apply due to radio link monitoring based on SSB as RLM -RS.

-	The NCR-MT is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

##### 14.3.1.6.3	Scheduling availability of NCR-MT performing radio link monitoring on FR2

The following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell .

-	If the RLM-RS is CSI-RS which is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON,

-	There are no scheduling restrictions due to radio link monitoring based on the CSI-RS.

-	Otherwise

-	The NCR-MT is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

For FR2, if following conditions are met,

-	NCR-MT has been notified about system information update through paging,

-	The gap between NCR-MT’s reception of PDCCH that NCR-MT monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 3, NCR-MT is expected to receive the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for RLM; and

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 2, NCR-MT is expected to receive PDSCH that corresponds to the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for RLM.

14.3.2	Link Recovery Procedure

14.3.2.1	Introduction

The NCR-MT shall assess the downlink radio link quality of a serving cell based on the reference signal in the set  as specified in TS 38.213 [23] in order to detect beam failure on:

-	PCell in SA NR,,

The RS resource configurations in the set  on PCell can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set  on SCell shall be periodic CSI-RS. NCR-MT is not required to perform beam failure detection outside the active DL BWP. NCR-MT is not required to meet the requirements in clause 14.3.2.2 and 14.3.2.3  if NCR-MT does not have set .

On each RS resource configuration in the set , the NCR-MT shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell beams.

The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resource configuration on set  cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB is derived based on the hypothetical PDCCH transmission parameters listed in Table 14.3.2.2.1. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 14.3.2.3.1-1.

Upon request the NCR-MT shall deliver configuration indexes from the set as specified in TS 38.213 [23] , to higher layers,  and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR, which is indicated by higher layer parameter rsrp-ThresholdSSB. The NCR-MT applies the Qin_LR threshold to the L1-RSRP measurement obtained from an SSB. The NCR-MT applies the Qin_LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the set  can be periodic CSI-RS resources or SSBs or both SSB and CSI-RS resources. NCR-MT is not required to perform candidate beam detection outside the active DL BWP.

14.3.2.2	Requirements for SSB based beam failure detection

14.3.2.2.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the NCR-MT active DL BWP during the entire evaluation period specified in clause 14.3.2.2.2. The requirements in this clause could not be applicable if NCR-MT is required to perform beam failure detection on more than 1 serving cell per band.

Table 14.3.2.2.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

14.3.2.2.2	Minimum requirement

NCR-MT shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period.

The value of TEvaluate_BFD_SSB is defined in Table 14.3.2.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 14.3.2.2.2-2 for FR2 with scaling factor N= 8.

For FR1,

-	P=1.

For FR2,

-	P=1, when the BFD-RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when the BFD-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

If the NCR-MT is not capable of 4 SMTC configurations per frequency [22], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [22], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the combination of BFD-RS resource and SMTC occasion does not meet pervious conditions.

Table 14.3.2.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| Note: TSSB is the periodicity of SSB in the set . |  |

Table 14.3.2.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N) TSSB) |
| Note: TSSB is the periodicity of SSB in the set . |  |

14.3.2.2.3	Measurement restriction for SSB based beam failure detection

The NCR-MT is required to be capable of measuring SSB for BFD without measurement gaps. The NCR-MT is required to perform the SSB measurements with measurement restrictions as described in the following scenarios.

For FR1, when the SSB for BFD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, NCR-MT shall be able to measure the SSB for BFD measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If NCR-MT supports simultaneousRxDataSSB-DiffNumerology, NCR-MT shall be able to measure the SSB for BFD measurement without any restriction;

-	If NCR-MT does not support simultaneousRxDataSSB-DiffNumerology, NCR-MT is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

For FR2, when the SSB for BFD measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC in the same band, NCR-MT is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

14.3.2.3	Requirements for CSI-RS based beam failure detection

14.3.2.3.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set  of resource configurations for a serving cell, provided that the CSI-RS resource(s) in set for beam failure detection are actually transmitted within the NCR-MT active DL BWP during the entire evaluation period specified in clause 14.3.2.3.2. NCR-MT is not expected to perform beam failure detection measurements on the CSI-RS configured for BFD if the CSI-RS is not QCL-ed, with QCL-TypeD when applicable, with the RS in the active TCI state of any CORESET configured in the NCR-MT active BWP. The requirements in this clause apply when NCR-MT is required to perform beam failure detection on no more than 1 serving cell per band.

Table 14.3.2.3.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

14.3.2.3.2	Minimum requirement

NCR-MT shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS ms period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 14.3.2.3.2-1 for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in Table 14.3.2.3.2-2 for FR2 with N=1.

The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For FR1,

-	P = 1.

For FR2,

-	P = 1, when the BFD-RS resource is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	Psharing factor = 3.

If the NCR-MT is not capable of 4 SMTC configurations per frequency [22], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [22], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

NOTE:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the BFD-RS resource and SMTC occasion configurations does not meet pervious conditions.

The values of MBFD used in Table 14.3.2.3.2-1 and Table 14.3.2.3.2-2 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3.

Table 14.3.2.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P] TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

Table 14.3.2.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P N] TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

14.3.2.3.3	Measurement restrictions for CSI-RS based beam failure detection

The NCR-MT is required to be capable of measuring CSI-RS for BFD without measurement gaps. The NCR-MT is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, NCR-MT is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the NCR-MT shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the NCR-MT shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the NCR-MT supports simultaneousRxDataSSB-DiffNumerology the NCR-MT shall be able to perform CSI-RS measurement without restrictions.

-	If the NCR-MT does not support simultaneousRxDataSSB-DiffNumerology, NCR-MT is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, NCR-MT shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC in the same band, or in the same symbol as SSB for CBD measurement on the same CC in the same band when beam failure is detected, NCR-MT is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC in the same band,

-	In the following cases, NCR-MT is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in set  and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to NCR-MT,

-	Otherwise, NCR-MT shall be able to measure the CSI-RS for BFD measurement without any restriction.

14.3.2.4	Minimum requirement for L1 indication

When the radio link quality on all the RS resources in set  is worse than Qout_LR, layer 1 of the NCR-MT shall send a beam failure instance indication to the higher layers. A layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [22].

The beam failure instance evaluation for the RS resources in set  shall be performed as specified in clause 6 in TS 38.213 [23]. Two successive indications from layer 1 shall be separated by at least TIndication_interval_BFD.

TIndication_interval_BFD is max(2ms, TSSB-RS,M) ) or max(2ms, TCSI-RS,M), where TSSB-RS,M and TCSI-RS,M is the shortest periodicity of all RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB  in the set  or CSI-RS resource in the set .

14.3.2.5	Requirements for SSB based candidate beam detection

14.3.2.5.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within NCR-MT active DL BWP during the entire evaluation period specified in clause 14.3.2.5.2.

14.3.2.5.2	Minimum requirement

Upon request the NCR-MT shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB ms period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table in B.2.4.1 [21] for a corresponding band.

The NCR-MT shall monitor the configured SSB resources using the evaluation period in table 14.3.2.5.2-1 and 14.3.2.5.2-2 which is applicable to the non-DRX mode only.

The value of TEvaluate_CBD_SSB is defined in Table 14.3.2.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in Table 14.3.2.5.2-2 for FR2 with scaling factor N=8.

Where,

For FR1,,

-	P = 1.

For FR2,

-	P=1, when the candidate beam detection RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when candidate beam detection RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

If the NCR-MT is not capable of 4 SMTC configurations per frequency [22], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [22], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the combination of CBD-RS resource and SMTC occasion configurations does not meet pervious conditions.

Table 14.3.2.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | Ceil(3 P) TSSB |
| Note: TSSB is the periodicity of SSB in the set . |  |

Table 14.3.2.5.2-2: Evaluation period TEvaluate_CBD_SSB for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | Ceil(3 P N) TSSB |
| Note: TSSB is the periodicity of SSB in the set . |  |

14.3.2.5.3	Measurement restriction for SSB based candidate beam detection

For FR1, when the SSB for CBD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, NCR-MT shall be able to measure the SSB for CBD measurement without any restrictions;

-	If SSB and CSI-RS have different SCS-es,

-	If NCR-MT supports simultaneousRxDataSSB-DiffNumerology, NCR-MT shall be able to measure the SSB for CBD measurement without any restriction;

-	If NCR-MT does not support simultaneousRxDataSSB-DiffNumerology, NCR-MT is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, when the SSB for CBD measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC in the same band, NCR-MT is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

14.3.2.6	Requirements for CSI-RS based candidate beam detection

14.3.2.6.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set  configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within NCR-MT active DL BWP during the entire evaluation period specified in clause 14.3.2.6.2.

14.3.2.6.2	Minimum requirement

Upon request the NCR-MT shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table in B.2.4.2 [21] for a corresponding band.

The NCR-MT shall monitor the configured CSI-RS resources using the evaluation period in table 14.3.2.6.2-1 and 14.3.2.6.2-2 which is applicable to the non-DRX mode only.

The value of TEvaluate_CBD_CSI-RS is defined in Table 14.3.2.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in Table 14.3.2.6.2-2 for FR2 with scaling factor N=8.

For FR1,

-	P = 1.

For FR2,

-	P = 1, when candidate beam detection RS is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = 3, when candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

If the NCR-MT is not capable of 4 SMTC configurations per frequency [21], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [21], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the NCR-MT is capable of 4 SMTC configurations per frequency [21], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [21], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 14.3.2.6.3.

The values of MCBD used in Table 14.3.2.6.2-1 and Table 14.3.2.6.2-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Table 14.3.2.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | Max(25, Ceil(MCBD P) TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

Table 14.3.2.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | Max(25, Ceil(MCBD P N) TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . |  |

14.3.2.6.3	Measurement restriction for CSI-RS based candidate beam detection

For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, NCR-MT is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for CBD measurement, the NCR-MT shall be able to perform CSI-RS based CBD measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for CBD measurement, the NCR-MT shall be able to perform CSI-RS based CBD measurement with restrictions according to its capabilities:

-	If the NCR-MT supports simultaneousRxDataSSB-DiffNumerology the NCR-MT shall be able to perform CSI-RS based CBD measurement for without restrictions.

-	If the NCR-MT does not support simultaneousRxDataSSB-DiffNumerology, NCR-MT is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CBD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, NCR-MT shall be able to measure the CSI-RS for CBD measurement without any restriction.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement on the same CC in the same band, NCR-MT is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC in the same band, NCR-MT is required to measure one of but not both CSI-RS for CBD measurement and the other CSI-RS. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

14.3.2.7	Scheduling availability of NCR-MT during beam failure detection

Scheduling availability restrictions when the NCR-MT is performing beam failure detection are described in the following clauses.

##### 14.3.2.7.1	Scheduling availability of NCR-MT performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to beam failure detection performed on SSB and CSI-RS configured for BFD with the same SCS as PDSCH or PDCCH in FR1.

##### 14.3.2.7.2	Scheduling availability of NCR-MT performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1

For NCR-MTs which support simultaneousRxDataSSB-DiffNumerology [TBD] there are no restrictions on scheduling availability due to beam failure detection when SSB is configured as BFD. For NCR-MTs which do not support simultaneousRxDataSSB-DiffNumerology [TBD] the following restrictions apply due to beam failure detection when SSB is configured as BFD.

-	The NCR-MT is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

##### 14.3.2.7.3	Scheduling availability of NCR-MT performing beam failure detection on FR2

The following scheduling restriction applies due to beam failure detection.

-	For the case where no RSs are provided for BFD, or when CSI-RS is configured for BFD is explicitly configured and is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON.

-	There are no scheduling restrictions due to beam failure detection performed based on the CSI-RS.

-	Otherwise

-	The NCR-MT is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on BFD-RS resource symbols to be measured for beam failure detection.

For FR2, if following conditions are met,

-	NCR-MT has been notified about system information update through paging,

-	The gap between NCR-MT’s reception of PDCCH that NCR-MT monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, NCR-MT is expected to receive the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for BFD mesurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, NCR-MT is expected to receive PDSCH that corresponds to the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for BFD mesurement.

14.3.2.8	Scheduling availability of NCR-MT during candidate beam detection

Scheduling availability restrictions when the NCR-MT is performing L1-RSRP measurement for candidate beam detection are described in the following clauses.

##### 14.3.2.8.1	Scheduling availability of NCR-MT performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as link recovery detection resource with the same SCS as PDSCH or PDCCH in FR1.

##### 14.3.2.8.2	Scheduling availability of NCR-MT performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For NCR-MTs which support simultaneousRxDataSSB-DiffNumerology [TBD] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as link recovery detection resource. For NCR-MTs which do not support simultaneousRxDataSSB-DiffNumerology [TBD] the following restrictions apply due to L1-RSRP measurement based on SSB configured as link recovery detection resource.

-	The NCR-MT is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, TRS, CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for L1-RSRP.

##### 14.3.2.8.3	Scheduling availability of NCR-MT performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to candidate beam detection

-	The NCR-MT is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, CSI-RS for tracking or CSI-RS for CQI on reference symbols to be measured for candidate beam detection.

For FR2, if following conditions are met,

-	NCR-MT has been notified about system information update through paging,

-	The gap between NCR-MT’s reception of PDCCH that NCR-MT monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, NCR-MT is expected to receive the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for CBD measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, NCR-MT is expected to receive PDSCH that corresponds to the PDCCH that NCR-MT monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for CBD measurement.

< END OF CHANGE>