# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2317395

# Xiamen, China, October 09 – October 13, 2023

Agenda item:			6.6.5

Source:	Moderator (MediaTek inc.)

Title:	WF on R18 IoT NTN RRM requirements

Document for:	Approval

# Introduction

This document is to capture all agreements in email thread [108bis][231] IoT_NTN_enh.

# Topic #1: RRM core requirements (AI 6.6.4)

## Sub-Topic 1: K_satellite and other general issue

Agreement:

- For Idle mode, each frequency carrier

  - Ksatellite =2, if there are at least two NGSO satellites (including UE serving satellite) to be monitored by UE in this frequency carrier

  - Ksatellite =1, if there is only one NGSO satellite to be monitored in this frequency carrier

- For Connected mode, each frequency carrier

  - Ksatellite =2, if there are at least two NGSO satellites (including UE serving satellite) to be monitored by UE in this frequency carrier

  - Ksatellite =1, if there is only one NGSO satellite to be monitored in this frequency carrier

#### Issue 1-1-2: K_satellite for NGSO and t-ServiceStart(Neigh)

Agreement:

- For Ksatellite, all neighbor cell of neighboring NGSO satellite are not considered to be monitored before t-serviceStart(Neigh) if t-serviceStart(Neigh) is configured.

## Sub-Topic 2: IDLE mode measurements

#### Issue 2-2: Location-based triggering cell reselection measurements

Agreement:

- For IDLE mode neighbour cell measurements for eMTC and NB-IoT, the location-based measurement initiation condition for NR-NTN in R18 (in clause 4.2C.2.3 and 4.2C.2.4 in TS 38.133) can be reused with necessary modifications of parameters names

- Note:

  - Requirements apply for quasi-fixed cell and earth-moving cell

    - For defining the requirements for earth-moving cell, further discuss based on the agreement from R18 NR NTN.

  - margin for distanceThresh is discussed in other issue

## Sub-Topic 3: CONN mode measurements

#### Issue 3-0 (new): For NGSO, neighbor NGSO satellite measurement in connected mode

Discuss the following options until next meeting:

- Option 1: For NB-IoT, best effort measurement for inter-satellite neighbor NGSO satellite for intra-frequency measurement, similar to inter-frequency.

- Option 2: other options are not precluded.

#### Issue 3-1: K_satellite, update NB-IoT inter-frequency measurement in connected mode

Agreement:

- NTN NB-IoT inter-frequency measurement in connected mode, update Tdetect_inter NB1-NC  and Tmeasure _inter NB1-NC as

  - Tdetect_inter_NB1-NC=iN*K_satelliteMin(5000, Ta,i) ms

  - Tmeasure_inter_NB1-NC=iM*K_satelliteMin(5000, Tb,i) ms

#### Issue 3-2-1: Time-based triggering Neighbour cell measurements in connected mode for NB-IoT - requirement

Agreement:

- For NB-IoT, time-based (t-Service) measurement initiation requirements apply to earth fixed cell.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

- For earth moving cell, UE shall initiate the measurement before losing its coverage and this needs to reflected in RAN4 requirements.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

#### Issue 3-3-1: Time-based triggering Neighbour cell measurements in connected mode for eMTC – applicability

Agreement:

- For eMTC, time-based (t-Service) measurement initiation requirements apply to earth fixed cell.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

- For earth moving cell, UE shall initiate the measurement before losing its coverage and this needs to reflected in RAN4 requirements.

  - RAN4 understand that the exact time for UE to start the measurement is up to UE implementation according to RAN2.

  - FFS whether to capture the exact time for UE to start the measurement in RAN4 requirements.

- Further discuss for time-based neighbor cell measurement, requirements apply for the eMTC UE provided the measurement gaps are configured.

#### Issue 3-3-2: Time-based triggering Neighbour cell measurements in connected mode for eMTC – Trigger

Agreement:

- Trigger is defined as Max (Tidentify_intra_UE cat M1, Tidentify_inter_UE cat M1)

#### Issue 3-4-2: Location-based triggering Neighbour cell measurements in connected mode – requirement

FFS:

- For CONNECTED mode location-based triggering of neighbour cell measurements for eMTC and NB-IoT, add the location-conditions to the existing list of conditions to trigger the neighbor cell measurements. But the delay requirements remain same

  - Note: for eMTC, it needs to consider measurement gap. How to capture it should be discussed in the CR phase.

Agreement:

- For NB-IoT, CONNECTED mode, UE shall perform measurement when location condition is met. And the existing delay requirements apply.

FFS eMTC, whether measurement gap should be provided.

#### Issue 3-5 (new): Time-based triggering Neighbour cell measurements in connected mode for eMTC – impact on measurement gap

Discuss until the next meeting:

- If t-ServiceStartNeigh is provided, measurement gap pattern(s) configured for neighbour cell measurements are suspended until time t-ServiceStartNeigh.

## Sub-Topic 4: eMTC, CHO

#### Issue 4: For eMTC, CHO requirements

Agreement:

- Introduce CHO requirements for NTN eMTC with time and location-based trigger conditions. DCHO = TRRC + Tmeasure + TEvent_DU +Tinterrupt + TCHO_execution

Where:

  - TRRC is the RRC procedure delay

  - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until the time/location condition fulfilled.

  - Tmeasure is the measurements time. Tmeasure=0 if only condEventD1 or condEventT1 is configured.

  - TCHO_execution is the conditional execution preparation time

  - Tinterrupt is the interruption time.

  - FFS Update Tinterrupt to include Tsearch, based on handover interruption requirement as in

    - 5.5A.2.1.2	Interruption time for CEMode A

    - 5.5A.3.1.2	Interruption time for CEMode A

## Sub-Topic 5: GNSS re-acquisition gap in connected mode

#### Issue 5-1&5-2: GNSS gap, spec impact

Discuss the following options until next meeting.

- Option 1: add generic description that the measurement delay could be longer if GNSS fix happens during measurement period.

- Option 2: The measurement delay requirements are extended by the duration of the GNSS-MG.

  - When the UE triggers an early termination of the GNSS-MG, the measurement delay requirements are extended by the duration of the early-terminated GNSS-MG.

- Option 3: The measurement delay requirements are suspended until the termination of the GNSS-MG.

#### Issue 5-3: GNSS gap for eMTC, overlapping between MG and GNSS gap

Agreement:

- If gaps configured for reacquiring GNSS and gaps configured for mobility measurements at least partially overlaps in time with other, then UE shall suspend the gaps configured for mobility measurements and instead prioritize the use of GNSS gaps

  - Note: it considers as no overlapping between GNSS-MG and gaps configured for mobility measurements after the UE has performed early termination of GNSS-MG.

## Sub-Topic 6: CR split

| UE Cat | Requirement | Clause in TS36.133 | Volunteer |
| --- | --- | --- | --- |
| NB-IoT | IDLE mode measurements | 4.6A | MTK |
|  | CONNECTED mode measurement on neighbour cells | (New) 8.14A.X1 | Nokia |
|  | GNSS re-acqusition impact related, e.g. RLM, ... | … | Huawei |
| Cat M1 | IDLE mode measurements | 4.7A | Huawei |
|  | CONNECTED mode measurement on neighbour cells | 8.13A | Nokia |
|  | conditional HO for Cat-M1 | 5.5A.2.3/4 (CEModeA) 5.5A.3.3/4 (CEModeB) | CMCC |
|  | GNSS re-acqusition impact related, e.g. RLM, ... | ... | Ericsson |