3GPP TSG-RAN WG4 Meeting#108-bis 											R4-2317580

Xiamen, China, October 9 – 13

Title: 	WF on A-MPR for NS_50 with PC1.5

Agenda Item:	4.19.2

Source: 	Apple

Document for:	Approval

# A-MPR and regions for PC1.5 and NS_50

The WID RP-222351 objectives are to introduce RF requirements that are applicable to PC1.5 UE mobile device and FWA for NR TDD bands. The power class 1.5 shall be introduced to n39 and therefore NS_50 needs to be updated to accommodate the regulatory emission requirements.

## Evaluation on existing channel bandwidth

Below the A-MPR regions are displayed for CBW from 15 to 30MHz. The yellow region indicates the additional area for PC1.5 requiring power back-off.

Table 1: A-MPR regions for NS_50 (Power Class 1.5). Red colour indicates new regions with respect to PC2.

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
|  | < 12.42- LCRB*12*SCS | ≥ 2.7 < 2.7+2* RBstart*12*SCS ≥ 2* RBstart*12*SCS-1.08 | A10 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
|  | < 17.1- LCRB*12*SCS | ≥ 3.6 < 3.6+2* RBstart*12*SCS ≥ 2* RBstart*12*SCS-3.6 | A10 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
|  | > 1.8, ≤ 6.48 > LCRB *12*SCS -5 | > 3.6 | A10 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | > 1.8, ≤ 7.56 > LCRB *12*SCS -5 | > 3.6 | A10 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-4. |  |  |  |

Table 2: A-MPR regions for NS_50 (Power Class 1.5)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5+3 | ≤4.5 + 3 | ≤ 4 + 2.5 | ≤ 2.5 + 2.5 | ≤ 4 + 1 | ≤ 1 + 2.5 | ≤ 3.5 + 2 | ≤ 10+2.5 | ≤ 1 + 2.5 | ≤ 3.5 | ≤ 2.5 |
|  | QPSK | ≤ 11.5+3 | ≤ 5.5 + 3 | ≤ 4 + 2.5 | ≤ 2.5 + 2.5 | ≤ 4 + 1 | ≤ 1 + 2.5 | ≤ 3.5 + 2 | ≤ 10+2.5 | ≤ 1 + 2.5 | ≤ 3.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 11.5+3 | ≤ 5.5 + 3 | ≤ 4 + 2.5 | ≤ 2.5 + 2.5 | ≤ 4 + 1 | ≤ 1.5 + 2.5 | ≤ 3.5 + 2 | ≤ 10+2.5 | ≤ 1.5 + 2.5 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 11.5+3 | ≤ 5.5 + 3 | ≤ 4 + 2.5 | ≤ 5.0 | ≤ 4 + 1 |  | ≤ 5.5 | ≤ 10+2.5 |  | ≤ 3.5 | ≤ 2.5 |
|  | 256 QAM | ≤ 11.5+3 | ≤ 5.5 + 3 |  |  |  |  |  | ≤ 10+2.5 |  |  | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 12.5+3 | ≤ 7 + 3 | ≤ 5.5 + 3 | ≤ 4 + 3 | ≤ 4 + 2.5 | ≤ 2 + 3 | ≤ 5 + 2 | ≤ 11+2.5 | ≤ 2 + 3 | ≤ 5.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 12.5+3 | ≤ 7 + 3 | ≤ 5.5 + 3 | ≤ 4 + 3 | ≤ 4 + 2.5 | ≤ 5 | ≤ 5 + 2 | ≤ 11+2.5 | ≤ 5 | ≤ 5.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 12.5+3 | ≤ 7 + 3 | ≤ 5.5 + 3 | ≤ 4 + 3 | ≤ 4 + 2.5 |  | ≤ 5 + 2 | ≤ 11+2.5 |  | ≤ 5.0 | ≤ 4.0 |
|  | 256 QAM | ≤ 12.5+3 | ≤ 7 + 3 |  |  |  |  |  | ≤ 11+2.5 |  | ≤ 6.5 | ≤ 6.5 |

<Way forward>: Table 1 and Table 2 are considered for further discussion and companies are asked to provide additional insights next meeting.

## Consideration on 35MHz CBW

To complete the work on PC1.5 it is required to include 35MHz CBW. The specific channel bandwidth will be included in a future Rel-18 version. The current state of A-MPR and regions can be found in clause 6.2.3.19 of TS.38.101-1 (V18.3.0).

<Way forward>: Companies are asked to provide considerations and results on 35MHz CBW for NS_50 with PC1.5