3GPP TSG-RAN WG4 Meeting # 108bis                                                          R4-2317589

Xiamen, China, October 09 – October 13, 2023

Agenda item:			4.30.2

Source:	Moderator (CATT)

Title:	WF on NR_700800900_combo_enh

Document for:	Approval

- Introduction

This document provides the WF on the technical issues based on the discussion in topic WF on NR_700800900_combo_enh.

- Way Forward

2.1 CA_ n26(2A)

Issue 1-1:  Baseline RF architecture

- Assume dual PA architecture only.

Issue 1-2: Current MPR applicability for UL CA_n26(2A)

- Current 2PA MPR requirements can apply for UL CA_n26(2A)

Issue 1-3: NS_12/ 13/ 14/ 15, NS_100 applicability for UL CA_n26(2A)

- The following idea is the guideline for the discussion in next meeting.

- Define the CA NC NS_12/13/14/15/100 with relaxed values for AMPR

UE is allowed to apply any backoff to meet emission requirements

Issue 1-4: Coexistence requirements with neighboring bands

- Companies are encouraged to study how to meet this requirement for UL NC CA.

- Max Channel BW per CC for this BCS is 15MHz

Issue 1-5: MSD test points and value

- The general approach/framework for 2UL/2DL FDD NC CA MSD will be discussed.

- The format in the following table can be considered as a starting point.

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 5/5 | 816.5 | 839 | 12 (RBSTART = [0]) | 12(RBSTART = [3]) | 861.5 | 884 | TBD | TBD | FDD |

- The following principles can be considered and further discussed for the MSD requirement.

    - Channel allocation is 5MHz+5MHz

    - 15kHz SCS

    - Total of equally distributed RB is equivalent to the 10MHz UL configuration

    - PCC DL channel overlaps with the lowest order IMD product of the two CCs

    - RB start of each CC can be adjusted to better center the IMD product.

2.2 CA_ n5-n8

Issue 2-1: How to handle the outputs for 2U/2D CA_n5-n8 in this WI

- All the output of study about 2UL/2DL CA for CA_n5-n8 can be captured in the TR 38.872 for future reference.

Issue 2-2: How to add the UL restriction in 38.101 spec

- The draft CR R4-2317587 focuses on the wording of the note and the RF requirements.

- If any new approach is agreed for the UL restriction note, it can be fixed later.

2.3 CA_n5-n28-n105

Issue 3-2: Tib/Rib and MSD for CA_n5-n28-n105

- The following Tib/Rib are agreed

Table 7 ΔTIB,c and ΔRIB,c for CA_n5-n28-n105

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A-n105A | n5 | 0.7 | 0.2 |
|  | n28 | 1.0 | 0.7 |
|  | n105 | 1.0 | 0.7 |

- The following MSD table can be a starting point for next meeting’s discussion

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | [22.9] | FDD | IMD3 |

2.4 CA_ n26-n28

Issue 5-1: Requirements related to the protection of the band 28/n28

- The agreement is captured in the draft CR R4-2317588.

Reference

[1] R4-2317236, Topic summary for [108-bis][112] NR_700800900_combo_enh, CATT, RAN4#108b