3GPP TSG-RAN WG4 Meeting #108bis	R4-2317604

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on STxMP UE RF requirements

Agenda Item:	5.29.1

Source: 	Samsung

Document for:	Approval

<Topic 1> RF requirements for PCMAX,f,c,k

<Sub-topic 1-1> Overlapped beam handling

<Online agreement>

- Overlapping indication is not needed in Rel-18

  - Further discuss overlapping indications in the future release.

<Sub-topic 1-2> PUMAX,f,c,k

<Online agreement>

- Introduce PUMAX,f,c,k in the core requirements with minimal impacts in Rel-18

  - FFS of PUMAX,f,c  and PUMAX,f,c,k on the testability issue in future release

  - FFS on the minimal impacts

<Sub-topic 1-3> MPRf,c,k/A-MPRf,c,k

<Way forward>: MPRf,c,k/A-MPRf,c,k derivation without overlapped beam indication

-	Option 1: MAX(MPRf,c, A- MPRf,c,) + X dB, where X is

-	Option 1a: 10*log (number of UL TCI-states indicated for STxMP) dB

-	Option 1b: [3 dB] for STxMP

-	Option 2: MAX(X, MPRf,c, A- MPRf,c,), where X is

-	Option 2a: 10*log (number of UL TCI-states indicated for STxMP) dB

-	Option 2b: [3 dB] for STxMP

<Way forward>: Additional relaxation (TSTxMP)

-	Whether to leave additional relaxation, outside of MAX(MPR) to the lower bound, will be further discussed together with MPRf,c,k and/or for future implementation constraints

<Sub-topic 1-4> P-MPRf,c,k

<Agreement>: P-MPR and PHR enhancement for Rel-18 STxMP

-	Recommended WF: P-MPR value is completely left to UE implementation for MPE compliance

<Sub-topic 1-5> Testability

<Agreement>: Whether to send LS to RAN5, with following proposal

-	Do not send LS to RAN5 and stop further discussion in this release.