3GPP TSG-RAN4 Meeting #108bis	R4-2317615

# Xiamen, China,  9th Oct – 13th Oct, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38101-1 Introduction of Lower MSD requirements for inter-band CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, Skyworks, MediaTek, NTT DOCOMO Inc., Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_ RF_FR1_enh2 |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The feasiblity study on MSD improvement has been carried out, and the contributions from various companies have been captured in TR 38.881. RAN4 concludes that MSD improvement is feasible and the UE may optionally indicate the actual performance via a new UE capability of [LowerMSD]. The information to be conveyed in this new capability has been agreed in a series of way forwards, including: R4-2314923, R4-2310499, and etc. Additionally, such information has been communicated to RAN2 to facilitate the signalling design for the new UE capability via a number of LS, including R4-2312247, R4-2310276 and R4-2306594. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause 7.3A.7 is added to introduce the requirements on Lower MSD for inter-band CA. More explicitly, the lower-MSD capability classes are defined for a UE to report different levels of MSD performance. The conditions to indicate the [LowerMSD] capability are also specified. And an informative note is added for RAN5 to reduce the conformance test burden. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new feature of Lower MSD is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<Start of changes>>>

### 7.3A.7	Lower-MSD requirements for inter-band CA

A UE can report better MSD performance than the minimum requirements as specified in clause 7.3A.4, 7.3A.5 and 7.3A.6 by [LowerMSD] capability, except that the reporting for MSD caused by IMD with order higher than 5, IMD of UL intra-band CA or triple-beat is not supported in this release of the specification. The MSD performance after improvement is categorized into different lower-MSD capability classes, which are defined in Table 7.3A.7-1.

Table 7.3A.7-1: Lower-MSD capability classes

| Lower-MSD capability class | Maximum allowed actual MSD (i.e. Threshold) | Remark |
| --- | --- | --- |
| I | 0 dB | Actual MSD ≤ 0dB |
| II | 3 dB | Actual MSD ≤ 3dB |
| III | 6 dB | Actual MSD ≤ 6dB |
| IV | 9 dB | Actual MSD ≤ 9dB |
| V | 12 dB | Actual MSD ≤ 12dB |
| VI | 15 dB | Actual MSD ≤ 15dB |
| VII | 18 dB | Actual MSD ≤ 18dB |
| VIII | 22 dB | Actual MSD ≤ 22dB |

The reported lower-MSD capability classes are subject to the same uplink/downlink configurations as defined for the minimum MSD requirements in clause 7.3A.4, 7.3A.5 and 7.3A.6. If a UE can support more than one test points for a given REFSENS exception case, the reported lower-MSD capability class is applicable for the test point having the largest specified MSD value. Otherwise, it’s only applicable for the test point which can be supported by the UE.

If the minimum requirement for a given REFSENS exception case is tightly bounded by the range of a lower-MSD capability class (i.e, MSD ≤ Thresholdi dB, where i is the lowest possible lower-MSD capability class), the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thresholdi-1 dB, where (i-1) is a lower level lower-MSD capability class) in order for the UE to report [LowerMSD] capability. If the actual MSD is larger than the maximum threshold (corresponding to lower-MSD capability class VIII), the UE cannot report [LowerMSD] capability for this REFSENS exception case.

NOTE: 	The [LowerMSD] capability is verified by reusing the RAN4 MSD test point parameters and only replacing the minimum MSD requirement value by the threshold of the reported lower-MSD capability class. And, similar to the specified MSD, only the highest supported power class or the power class required by the certification/regulation body per UL configuration is verified.

<<<End of changes>>>