3GPP TSG-RAN WG4 Meeting #xxx	R4-2317656

<Meeting Location>, <Date> ‒ <Date>, 2023

Title: 	WF on Cross-band MSD test point for FDD bands with Intra-band contiguous UL CA

Agenda Item:	4.1.1.1

Source: 	Qualcomm France

Document for:	Approval

## <Sub-topic 1-1 channel and RB allocation rules for inter-band MSD test FDD contiguous intra-band ULCA>

<Way forward/Agreement>:

- Equal CC BW’s for aggressor band, or if equal BW’s are not possible then PCC is 5MHz narrower

- Aggregated RB amount is the RB amount used for the single CC equaling the aggregated BW

  - E.g. 10+10 MHz => Aggregated UL RB allocation according to 20MHz single CC

- Use equal PSD to select UL RB balance between CC’s

- Evaluate two cases and specify MSD which is larger

  - Use RBstart’s which result in lowest IMD order full hit to DL, if applicable

  - Use RBstart’s which result in lowest IMD order partial hit to DL

- [IMD13] is considered as highest IMD order

- In addition to evaluating victim DL MSD, the impact to Intra-band UL CA band DL’s should be evaluated

  - To check the MSD is not higher than that of standalone Intra-band UL CA MSD

  - To give information to RAN5 that DL’s of the Intra-band contiguous UL CA will see desense

NOTE: If the IMD order for full hit is lower than the IMD order for partial hit, the partial hit does not need to be evaluated

# <Topic 1-2 CA_n5B_n12, CA_n5B_n14, and CA_n5B_n29 test points for evaluation >

<Way forward>:

The following test points are used for evaluation in next meeting:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | TBD | FDD | IMD9 |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | TBD | FDD | IMD11 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | TBD | FDD | IMD7 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | TBD | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | TBD | FDD | IMD11 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=37) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | TBD | FDD | IMD13 |
| NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified |  |  |  |  |  |  |  |  |

NOTE: Only one MSD testpoint for each NR CA combination is specified based on the evaluation