3GPP TSG-RAN WG4 Meeting #108bis-e                                                    	R4-2317672

Xiamen, China, October 09 – 13, 2023

Source:	Huawei, HiSilicon

Title:	TP for TR 37.718-21-11 DC_3C-28A_n1A

Agenda item:			4.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-21-11 to include DC_3C-28A_n1A.

2. Text Proposal

<Start of Text Proposal>

5.x	DC_3-28_n1

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink configuration |
| --- | --- |
| DC_3C-28A_n1A | DC_3A_n1A DC_3C_n1A DC_28A_n1A |

Note that UL DC_3A_n1A, DC_28A_n1A are already specified for this configuration in the specification.

5.x.2	Co-existence studies

The 2nd, 3rd and 4th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products had already been analyzed and specified for UL configuration DC_3A_n1A and DC_28A_n1A, respectively.

Regarding UL configuration DC_3C_n1A, the following analysis is presented.

Table 5.x.2-3: IMD (due to triple beats) analysis for UL configuration DC_3C_n1A

| UE UL carriers | Band 3 case #1 |  | Band 3 case #2 |  | Band n1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | fx1_low | fx2_low | fx1_high | fx2_high | fy_low | fy_high |
| UL frequency (MHz) | 1710 | 1750 | 1745 | 1785 | 1920 | 1980 |
| TB1 | ¦fx1_low-fx2_low+fy_low ¦ | ¦fx1_low-fx2_low+fy_high ¦ | ¦fx1_high-fx2_high+fy_low ¦ | ¦fx1_high-fx2_high+fy_high ¦ |  |  |
|  | 1880 | 1940 | 1880 | 1940 |  |  |
| TB2 | ¦-fx1_low+fx2_low+fy_low ¦ | ¦-fx1_low+fx2_low+fy_high ¦ | ¦-fx1_high+fx2_high+fy_low ¦ | ¦-fx1_high+fx2_high+fy_high¦ |  |  |
|  | 1960 | 2020 | 1960 | 2020 |  |  |
| TB3 | ¦-fx1_low-fx2_low+fy_low ¦ | ¦-fx1_low-fx2_low+fy_high ¦ | ¦-fx1_high-fx2_high+fy_low ¦ | ¦-fx1_high-fx2_high+fy_high ¦ |  |  |
|  | 1540 | 1480 | 1610 | 1550 |  |  |
| TB4 | ¦fx1_low+fx2_low+fy_low ¦ | ¦fx1_low+fx2_low+fy_high ¦ | ¦fx1_high+fx2_high+fy_low ¦ | ¦fx1_high+fx2_high+fy_high ¦ |  |  |
|  | 5380 | 5440 | 5450 | 5510 |  |  |

As we can see from the above table, there is no IMD to band 3, 28 or n1.

5.x.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values for DC_3-28_n1 have already been specified in TS 38.101-3.

5.x.4	Reference sensitivity exceptions

Compared to its fallback modes, no additional MSD requirements for this band combination are needed.

<End of Text Proposal>