3GPP TSG-RAN Meeting #108-bis	R4-2317690

# Xiamen, China, 09-13 Oct, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to add PC3 missing fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, TELUS, Bell Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_3BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2023-09-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following inter-band NR-CA combination with PC3 needs to be added as the fallback of CA_n5A-n66A-n77(3A), which is missing. No new MSD, so draft CR is adopted. DL_n5A-n66A-n77(2A)_UL_n77(2A) All PC3 fallbacks have been specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the requested inter-band NR-CA combination with PC3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requested inter-band NR-CA combinations with PC3, which is the fallback of CA_n5A-n66A-n77(3A), is not included in current spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGE >>>

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 |  |  | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n5A | CA_n1A-n3A CA_n1A-n5A CA_n3A-n5A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n3A-n7A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  | - |  |  | n1 |  | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n1A-n3A-n7B | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
| CA_n1A-n3(2A)-n7A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1(2A)-n3A-n7A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1A-n3B-n7A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1(2A)-n3B-n7A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1(2A)-n3(2A)-n7A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1A-n3B-n7B | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
| CA_n1A-n3A-n8A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n3A-n18A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
| CA_n1A-n3A-n20A | CA_n1A-n3A CA_n1A-n20A CA_n3A-n20A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n20 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n3A-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n3A-n26(2A) | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n1A-n3B-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
| CA_n1A-n3B-n26(2A) | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n1A-n3A-n28A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 202 |  |  |
|  | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 201, 301 |  |  |
| CA_n1A-n3B-n28A | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n3A-n38A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1A-n3B-n38A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1(2A)-n3A-n38A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1(2A)-n3B-n38A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1A-n3(2A)-n38A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1(2A)-n3(2A)-n38A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1A-n3A-n40A | CA_n1A-n3A CA_n1A-n40A CA_n3A-n40A |  |  | n1 |  | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 30, 35, 40, 45, 50 |  |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n3A-n41A | n417 CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n3A-n67A | CA_n1A-n3A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n3A-n75A | - |  |  | n1 |  | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n1A-n3A-n77A | n777 CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35,40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n3A-n77(2A) | n777 CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n1A-n3A-n77(3A) | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7 |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n78 |  | n78 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n1A-n3A-n78(2A) | CA_n78(2A) CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n3A-n78C | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n1A-n3B-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n3B-n78(2A) | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A CA_n3A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1(2A)-n3A-n79A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n3A-n79C | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1(2A)-n3A-n79C | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1A-n3B-n79A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n3B-n79C | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1(2A)-n3B-n79A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1(2A)-n3B-n79C | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3B_BCS0 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1A-n3(2A)-n79A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n3(2A)-n79C | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1(2A)-n3(2A)-n79A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1(2A)-n3(2A)-n79C | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n3 |  | CA_n3(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1A-n3A-n105A | CA_n1A-n3A CA_n1A-n105A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n3A-n105A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
| CA_n1A-n5A-n7A | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1A-n5A-n7B | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A CA_n7B |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
| CA_n1A-n5A-n28A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n1A-n7A-n8A | CA_n1A-n7A CA_n1A-n8A CA_n7A-n8A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n7A-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n7A-n26(2A) | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n1A-n7B-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A CA_n7B |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n7B-n26(2A) | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n1A-n7A-n38A10 | n1A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1(2A)-n7A-n38A10 | n1A |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1A-n7A-n40A | CA_n1A-n7A CA_n1A-n40A CA_n7A-n40A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
| CA_n1A-n7A-n67A | CA_n1A-n7A |  |  | n1 |  | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n7A-n75A | - |  |  | n1 |  | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n1A-n7A-n79A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n7A-n79C | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1(2A)-n7A-n79A | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1(2A)-n7A-n79C | - |  |  | n1 |  | CA_n1(2A)_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n1A-n8A-n28A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n28 |  | 10, 15, 20 |  |  |
| CA_n1A-n8A-n40A | CA_n1A-n8A CA_n1A-n40A CA_n8A-n40A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
| CA_n1A-n8A-n77A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n8A-n77(2A) | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n7B-n28A | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A CA_n7B |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 901, 100 |  |  |
|  | - |  |  | n1 |  | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n78 |  | n78 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n1A-n7B-n78A | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n1A-n7B-n78(2A) | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
|  | CA_n78(2A) CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n7A-n78C | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  | - |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n8A-n78(2A) | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS1 |  |  |
| CA_n1A-n8A-n79A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n18A-n28A | CA_n1A-n18A CA_n1A-n28A CA_n18A-n28A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
| CA_n1A-n18A-n41A | CA_n1A-n18A CA_n1A-n41A CA_n18A-n41A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n18A-n77A | CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n18A-n77(2A) | CA_n1A-n18A CA_n1A-n77A CA_n18A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n1A-n20A-n67A | CA_n1A-n20A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n20 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
| CA_n1A-n20A-n78A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n20 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n26A-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n1A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n1A-n28A-n38A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n1A-n28A-n40A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n28A-n40B | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n40 |  | CA_n40B_BCS0 |  |  |
| CA_n1A-n28A-n41A | n417 CA_n1A-n28A CA_n1A-n41A CA_n28A-n41A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n28A-n46A | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n46 |  | 10, 20, 40, 60, 80 |  |  |
| CA_n1A-n28A-n46C | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n46 |  | CA_n46C_BCS0 |  |  |
| CA_n1A-n28A-n46D | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n46 |  | CA_n46D_BCS0 |  |  |
| CA_n1A-n28A-n46(2A) | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
| CA_n1A-n28A-n75A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n75 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n1A-n28A-n77A | n777 CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n28A-n77(2A) | n777 CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n1A-n28A-n77(3A) | CA_n1A-n28A CA_n1A-n77A CA_n28A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 202 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n28A-n78(2A) | CA_n78(2A) CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n28A-n78C | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n28A-n102A | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n1A-n28A-n102B | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n1A-n28A-n102C | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n1A-n28A-n102D | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n1A-n28A-n102E | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n1A-n28A-n102(2A) | CA_n1A-n28A CA_n1A-n102A CA_n28A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n1A-n38A-n78A | - |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n40A-n77A | CA_n1A-n40A CA_n1A-n77A CA_n40A-n77A |  |  | n1 |  | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n40A-n77(2A) | CA_n1A-n40A CA_n1A-n77A CA_n40A-n77A |  |  | n1 |  | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n1A-n40A-n78A | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A |  |  | n1 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n40B-n78A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n40 |  | CA_n40B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n40A-n105A | CA_n1A-n40A CA_n1A-n105A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n40A-n105A |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
| CA_n1A-n41A-n77A | n417 n777 CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n1A-n41A-n77(2A) | n417 n777 CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n1A-n41A-n77(3A) | CA_n1A-n41A CA_n1A-n77A CA_n41A-n77A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n1A-n41A-n79A | CA_n1A-n41A CA_n1A-n79A CA_n41A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n46A-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | 10, 20, 40, 60, 80 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n46C-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46C_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n46D-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46D_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n46(2A)-n78A | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n46A-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n78(2A) |  |  | n46 |  | 10, 20, 40, 60, 80 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n46C-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n78(2A) |  |  | n46 |  | CA_n46C_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n46D-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n78(2A) |  |  | n46 |  | CA_n46D_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n46(2A)-n78(2A) | CA_n1A-n46A CA_n1A-n78A CA_n46A-n78A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n78(2A) |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n67A-n78A | CA_n1A-n78A |  |  | n1 |  | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n1A-n67A-n78(2A) | CA_n1A-n78A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20, 30, 40, 45, 50 | 0 |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n1A-n75A-n78A | - |  |  | n1 |  | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n78 |  | n78 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n77(2A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n77(3A)-n79A4 | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n78A-n79A5 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n1 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n78(2A)-n79A | - |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n1A-n78A-n102A | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n1A-n78A-n102B | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n1A-n78A-n102C | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n1A-n78A-n102D | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n1A-n78A-n102E | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n1A-n78A-n102(2A) | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n1A-n78(2A)-n102A | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n1A-n78(2A)-n102B | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n1A-n78(2A)-n102C | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n1A-n78(2A)-n102D | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n1A-n78(2A)-n102E | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n1A-n78(2A)-n102(2A) | CA_n1A-n78A CA_n1A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n1A-n78A-n105A | CA_n1A-n78A CA_n1A-n105A |  |  | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n78A-n105A |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
| CA_n2A-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2A-n5A-n41A | CA_n2A_n5A CA_n2A_n41A CA_n5A_n41A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 25 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n5A-n48B | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A CA_n48B |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 |  |  |
| CA_n2A-n5A-n48(2A) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
| CA_n2A-n5A-n48(A-B) | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 |  |  |
|  |  |  |  | n48 |  | CA_n48(A-B)_BCS0 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 |  |  |
|  |  |  |  | n48 |  | CA_n48(A-B)_BCS1 |  |  |
| CA_n2(2A)-n5A-n30A | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2A-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2(2A)-n5A-n66A | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2(2A)-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2A-n5A-n66(2A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
| CA_n2A-n5A-n66(3A) | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n2A-n5A-n77A | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n5A-n77C | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 CA_n77C |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n2A-n5A-n77(2A) | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n5A-n77A | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n5A-n77(2A) | n777 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n7A-n12A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
| CA_n2A-n7A-n66A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n2A-n7A-n71A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n2A-n7A-n77A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2(2A)-n12A-n30A | CA_n2A-n12A CA_n2A-n30A CA_n12A-n30A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2A-n12A-n41A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n2A-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2(2A)-n12A-n66A | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2A-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2(2A)-n12A-n66(2A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2A-n12A-n66(3A) | CA_n2A-n12A CA_n2A-n66A CA_n12A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n2A-n12A-n77A | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n12A-n77A | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n12A-n77(2A) | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n12A-n77(2A) | n777 CA_n2A-n12A CA_n2A-n77A7 CA_n12A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2(2A)-n14A-n30A | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2A-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2(2A)-n14A-n66A | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2(2A)-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2A-n14A-n66(2A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2A-n14A-n66(3A) | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n2A-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n14A-n77(2A) | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n14A-n77A | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n14A-n77(2A) | n777 CA_n2A-n14A CA_n2A-n77A7 CA_n14A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n29A-n30A | CA_n2A-n30A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2(2A)-n29A-n30A | CA_n2A-n30A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
| CA_n2A-n29A-n66A | CA_n2A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2(2A)-n29A-n66A | CA_n2A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2A-n29A-n66(2A) | CA_n2A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2(2A)-n29A-n66(2A) | CA_n2A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2A-n29A-n77A | n777 CA_n2A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n29A-n77A | n777 CA_n2A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n29A-n77(2A) | n777 CA_n2A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n29A-n77(2A) | n777 CA_n2A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n2(2A)-n30A-n66A | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n2(2A)-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n2A-n30A-n66(2A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
| CA_n2A-n30A-n66(3A) | CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n2A-n30A-n77A | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n30A-n77(2A) | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n30A-n77A | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n30A-n77(2A) | n777 CA_n2A-n30A CA_n2A-n77A7 CA_n30A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n41A-n66A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n2A-n41A-n71A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2A-n48(A-B)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(A-B)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(A-B)_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2A-n48B-n66A | CA_n48B CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2A-n48(2A)-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n2A-n48A-n77A | n777,9 CA_n2A-n48A CA_n2A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n48A-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A CA_n77C |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n2A-n48(2A)-n77C | CA_n2A-n48A CA_n2A-n77A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n2A-n48B-n77C | CA_n48B CA_n2A-n48A CA_n2A-n77A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n2A-n48B-n77A | CA_n48B CA_n2A-n48A CA_n2A-n77A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n48(2A)-n77A | CA_n2A-n48A CA_n2A-n77A |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n66A-n71A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n2A-n66A-n77A | n777, 9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n66A-n77A | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n66(2A)-n77A | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n66A-n77C | n777, 9 CA_n2A-n66A CA_n2A-n77A CA_n66A-n77A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n2A-n66A-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n66(2A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2(2A)-n66(2A)-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2(2A)-n66A-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 |  |  | n2 |  | CA_n2(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n66(2A)-n77(2A) | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n66(3A)-n77A | n777 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n66(3A)-n77(2A) | n777 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n66A-n78A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n66A-n78(2A) | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n2A-n71A-n77A | CA_n2A-n71A CA_n2A-n77A CA_n71A-n77A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n71A-n77(2A) | CA_n2A-n71A CA_n2A-n77A CA_n71A-n77A |  |  | n2 |  | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n2A-n71A-n78A | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n2A-n71A-n78(2A) | - |  |  | n2 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n3A-n5A-n7A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
| CA_n3A-n5A-n7B | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
| CA_n3A-n5A-n28A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n7A-n8A | CA_n3A-n7A CA_n3A-n8A CA_n7A-n8A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20, 35 |  |  |
| CA_n3A-n7A-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
| CA_n3A-n7A-n26(2A) | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n3A-n7B-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A CA_n7B |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
| CA_n3A-n7B-n26(2A) | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n3B-n7A-n26A | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
| CA_n3B-n7A-n26(2A) | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n3B-n7B-n26A | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A CA_n7B |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
| CA_n3B-n7B-n26(2A) | CA_n3A-n7A CA_n3A-n26A CA_n7A-n26A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
| CA_n3A-n7A-n28A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n3A-n7B-n28A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n3B-n7A-n28A | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n3B-n7B-n28A | CA_n7B CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
| CA_n3A-n7A-n38A10 | n3A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n3B-n7A-n38A10 | n3A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n3(2A)-n7A-n38A10 | n3A |  |  | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n3A-n7A-n67A | CA_n3A-n7A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
| CA_n3A-n7A-n75A | - |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n3A-n7A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n78 |  | n78 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n3A-n7A-n78C | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n3A-n7B-n78A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n3A-n7B-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3A-n7A-n78(2A) | CA_n78(2A) CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n3B-n7A-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3B-n7A-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3B-n7B-n78A | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3B-n7B-n78(2A) | CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A CA_n7B |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3A-n7A-n79A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3B-n7A-n79A | - |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3(2A)-n7A-n79A | - |  |  | n3 |  | CA_n3(2A)_BCS0 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n8A-n28A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20, 35 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n8A-n77A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n8A-n77(2A) | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n18A-n28A | CA_n3A-n18A CA_n3A-n28A CA_n18A-n28A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n18A-n77A | CA_n3A-n18A CA_n3A-n77A CA_n18A-n77A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n18A-n77(2A) | CA_n3A-n18A CA_n3A-n77A CA_n18A-n77A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n18 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n3A-n20A-n67A | CA_n3A-n20A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n20 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
| CA_n3A-n20A-n28A | CA_n3A-n20A CA_n3A-n28A CA_n20A-n28A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n20 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
| CA_n3A-n20A-n78A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n20 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3B-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n3A-n28A-n38A | - |  |  | n3 |  | 5, 10, 15, 20, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 30, 40 |  |  |
| CA_n3A-n28A-n40A | CA_n3A-n28A CA_n3A-n40A CA_n28A-n40A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n40 |  | 20, 40 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35,40 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n28A-n41A | n417 CA_n3A-n28A CA_n3A-n41A7 CA_n28A-n41A7 |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n28A-n41B | CA_n3A-n28A CA_n3A-n41A CA_n28A-n41A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n41 |  | CA_n41B_BCS0 |  |  |
| CA_n3A-n28A-n77A | n777 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 35,40 | 2 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n28A-n77(2A) | n777 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  | CA_n77(2A) |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
| CA_n3A-n28A-n77(3A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 202 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 202 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 2 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n28A-n78C | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 202 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  | CA_n78(2A) CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 2 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n3B-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3B-n28A-n78(2A) | CA_n78(2A) CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n3A-n28A-n79A | CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n79 |  | 40, 50, 80, 100 |  |  |
| CA_n3A-n38A-n40A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n38A-n78A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n40A-n78A | CA_n3A-n40A CA_n3A-n78A CA_n40A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n40A-n105A | CA_n3A-n40A CA_n3A-n105A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n40A-n105A |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n3A-n77(3A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n3 |  | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n3A-n40A-n41C | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A |  |  | n3 |  | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n41 |  | CA_n41C_BCS4 and 5 |  |  |
| CA_n3A-n40A-n77A | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A |  |  | n3 |  | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n40A-n77(2A) | CA_n3A-n40A CA_n3A-n77A CA_n40A-n77A |  |  | n3 |  | 5, 10, 15, 20, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n3A-n41A-n77A | n417 n777 CA_n3A-n41A7 |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  | CA_n3A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n41A-n77A7 |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n41B-n77A | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | CA_n41B_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n41A-n77(2A) | n417 n777 CA_n3A-n41A7 |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  | CA_n3A-n77A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n41A-n77A |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
| CA_n3A-n41A-n77(3A) | CA_n3A-n41A CA_n3A-n77A CA_n41A-n77A |  |  | n3 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n3A-n41A-n78A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | CA_n3A-n41A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  | CA_n3A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n41A-n78A |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  | CA_n3A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n41A-n78A |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n3A-n41A-n79A | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n3 |  | 5, 10, 15, 20, 25, 30 | 2 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n3 |  | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n79 |  | See n79 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n3A-n41C-n79A | CA_n41C CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A |  |  | n3 |  | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS4 and 5 |  |  |
|  |  |  |  | n79 |  | See n79 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n3A-n67A-n78A | CA_n3A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n3A-n67A-n78(2A) | CA_n78(2A) CA_n3A-n78A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n3A-n75A-n78A | - |  |  | n3 |  | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n78 |  | n78 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n3A-n78A-n79A | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n3A-n78A-n79C | - |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n3B-n78A-n79A | - |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n3B-n78A-n79C | - |  |  | n3 |  | CA_n3B_BCS0 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n3(2A)-n78A-n79A | - |  |  | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n3(2A)-n78A-n79C | - |  |  | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n3A-n78A-n105A | CA_n3A-n78A CA_n3A-n105A |  |  | n3 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n78A-n105A |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
| CA_n5A-n7A-n28A | - |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 25, 30, 40, 50 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
| CA_n5A-n7A-n77A | CA_n5A-n7A CA_n5A-n77A CA_n7A-n77A |  |  | n5 |  | 5, 10, 15, 20, 25 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n7A-n77(2A) | CA_n77(2A) CA_n5A-n7A CA_n5A-n77A CA_n7A-n77A |  |  | n5 |  | 5, 10, 15, 20, 25 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
| CA_n5A-n7A-n77(3A) | CA_n77(2A) CA_n5A-n7A CA_n5A-n77A CA_n7A-n77A |  |  | n5 |  | 5, 10, 15, 20, 25 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
| CA_n5A-n7A-n78A | CA_n5A-n78A7 CA_n7A-n78A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n5A-n7B-n78A | CA_n5A-n78A7 CA_n7A-n78A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n7 |  | CA_n7B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n5A-n12A-n77A | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n12A-n77(2A) | n777 CA_n5A-n12A CA_n5A-n77A7 CA_n12A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n14A-n77A | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n14A-n77(2A) | n777 CA_n5A-n14A CA_n5A-n77A7 CA_n14A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n14 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n5A-n25A-n77A | CA_n5A-n25A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  | CA_n5A-n77A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  | CA_n25A-n77A |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n25(2A)-n77A | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n25A-n77(2A) | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n25A-n77(3A) | CA_n77(2A) CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n5A-n25(2A)-n77(2A) | CA_n5A-n25A CA_n5A-n77A CA_n25A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n25A-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n25(2A)-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n25A-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n5A-n25(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n5A-n29A-n77A | n777 CA_n5A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n29A-n77(2A) | n777 CA_n5A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n30A-n66A | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n5A-n30A-n66(2A) | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
| CA_n5A-n30A-n66(3A) | CA_n5A-n30A CA_n5A-n66A CA_n30A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n5A-n30A-n77A | n777 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n30A-n77(2A) | n777 CA_n5A-n30A CA_n5A-n77A7 CA_n30A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n40A-n78A | CA_n5A-n40A CA_n5A-n78A CA_n40A-n78A |  |  | n5 |  | 5, 10, 15, 20, 251 | 0 |  |
|  |  |  |  | n40 |  | 58, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |  |
| CA_n5A-n41A-n66A | CA_n5A-n41A CA_n5A-n66A CA_n41A-n66A |  |  | n5 |  | 5, 10, 15, 20, 25 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n5A-n48(A-B)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A |  |  | n5 |  | 5, 10, 15, 20, 251 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(A-B)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(A-B)_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n5A-n48B-n66A | CA_n48B CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n5A-n48(2A)-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n5A-n48A-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n48A-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A CA_n77C |  |  | n5 |  | 5, 10, 15, 20, 251 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 | 1 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n5A-n48B-n77A | CA_n5A-n48A CA_n5A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n48B-n77C | CA_n5A-n48A CA_n5A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77C BCS1 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77C BCS0 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 3 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77C BCS1 |  |  |
| CA_n5A-n48(2A)-n77A | CA_n5A-n48A CA_n5A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n48(2A)-n77C | CA_n5A-n48A CA_n5A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 2 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 3 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n5A-n66A-n77A | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n66(2A)-n77A | n777 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n66(2A)-n77(2A) | n777 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n66(3A)-n77A | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n66(3A)-n77(2A) | n777 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n5A-n66A-n77C | CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A |  |  | n5 |  | 5, 10, 15, 20, 251 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20, 251 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n5A-n66A-n77(2A) | n777 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 CA_n77(2A) |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  |  |  | n5 |  |  | 5, 10, 15, 20 |  | 1 |  |  |
|  |  |  |  |  |  | n66 |  |  | 5, 10, 15, 20, 30, 40 |  |  |  |  |
|  |  |  |  |  |  | n77 |  |  | CA_n77(2A)_BCS4 and 5 |  |  |  |  |
| CA_n5A-n66A-n77(3A) | CA_n77(2A) CA_n5A-n66A CA_n5A-n77A CA_n66A-n77A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n5 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n66(2A)-n78A | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n5A-n66A-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n5A-n66(2A)-n78(2A) | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  |  | n5 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n8A-n28A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |
| CA_n7A-n8A-n40A | CA_n7A-n8A CA_n7A-n40A CA_n8A-n40A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
| CA_n7A-n8A-n78A | CA_n7A-n8A CA_n7A-n78A CA_n8A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n12A-n25A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n7A-n12A-n66A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |  |
| CA_n7A-n12A-n77A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n12 |  | 5, 10, 15 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n7A-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n7A-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n7A-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n7(2A)-n25A-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n7(2A)-n25(2A)-n66A | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n7(2A)-n25A-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n7(2A)-n25(2A)-n66(2A) | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n7A-n25A-n71A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n7A-n25A-n77A | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n25(2A)-n77A | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n25A-n77(2A) | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7A-n25A-n77(3A) | CA_n77(2A) CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n7A-n25(2A)-n77(2A) | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7(2A)-n25A-n77A | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7(2A)-n25(2A)-n77A | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7(2A)-n25A-n77(2A) | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7(2A)-n25(2A)-n77(2A) | CA_n7A-n25A CA_n7A-n77A CA_n25A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7A-n25A-n78A | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |  |
| CA_n7(2A)-n25A-n78A | - |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |  |
| CA_n7A-n25(2A)-n78A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |  |
| CA_n7(2A)-n25(2A)-n78A | - |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |  |
| CA_n7A-n25A-n78(2A) | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7(2A)-n25A-n78(2A) | - |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7A-n25(2A)-n78(2A) | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7(2A)-n25(2A)-n78(2A) | - |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7A-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n26A-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7A-n26(2A)-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n26(2A)-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7B-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B |  |  | n7 |  | CA_n7B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7B-n26A-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n7B CA_n26A-n78A |  |  | n7 |  | CA_n7B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7B-n26(2A)-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A |  |  | n7 |  | CA_n7B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7B-n26(2A)-n78(2A) | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A |  |  | n7 |  | CA_n7B_BCS0 | 0 |  |
|  |  |  |  | n26 |  | CA_n26(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS0 |  |  |
| CA_n7A-n28A-n38A11 | n28 |  |  | n7 |  | 5, 10, 15, 20, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 30, 40 |  |  |
| CA_n7A-n28A-n78(2A) | CA_n78(2A) CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n28A-n78A | CA_n7A-n78A7 |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  | CA_n28A-n78A7 |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n7A-n28A-n78C | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n7B-n28A-n78A | CA_n7A-n78A7 CA_n28A-n78A7 |  |  | n7 |  | CA_n7B_BCS0 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A CA_n7B |  |  | n7 |  | CA_n7B_BCS0 | 1 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |  |
| CA_n7B-n28A-n78(2A) | CA_n7B CA_n78(2A) CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A |  |  | n7 |  | CA_n7B_BCS0 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n38A-n78A10 | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n40A-n78A | CA_n7A-n40A CA_n7A-n78A CA_n40A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n46A-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | 20, 40, 60, 80 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n46C-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46C_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n46D-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46D_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n46(2A)-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n46(2A)-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n46A-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | 20, 40, 60, 80 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n46C-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46C_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n46D-n78(2A) | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n46 |  | CA_n46D_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n66A-n71A | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n7A-n66A-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n66(2A)-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n66A-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7A-n66A-n77(3A) | CA_n77(2A) CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n7A-n66(2A)-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7(2A)-n66A-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7(2A)-n66(2A)-n77A | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7(2A)-n66A-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7(2A)-n66(2A)-n77(2A) | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n7A-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS1 |  |  |
|  |  |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7(2A)-n66A-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7(2A)-n66(2A)-n78A | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n66(2A)-n78(2A) | - |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7(2A)-n66A-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7(2A)-n66(2A)-n78(2A) | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A |  |  | n7 |  | CA_n7(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n67A-n78A | CA_n7A-n78A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n67A-n78(2A) | CA_n7A-n78A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n67 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n7A-n71A-n77A | CA_n7A-n71A CA_n7A-n77A CA_n71A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n7A-n71A-n77(2A) | CA_n77(2A) CA_n7A-n71A CA_n7A-n77A CA_n71A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
| CA_n7A-n71A-n77(3A) | CA_n77(2A) CA_n7A-n71A CA_n7A-n77A CA_n71A-n77A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
| CA_n7A-n75A-n78A | - |  |  | n7 |  | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n78 |  | n78 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n7A-n78A-n102A | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n7A-n78A-n102B | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n7A-n78A-n102C | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n7A-n78A-n102D | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n7A-n78A-n102E | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n7A-n78A-n102(2A) | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n7A-n78(2A)-n102A | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n7A-n78(2A)-n102B | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n7A-n78(2A)-n102C | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n7A-n78(2A)-n102D | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n7A-n78(2A)-n102E | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n7A-n78(2A)-n102(2A) | CA_n7A-n78A CA_n7A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n8A-n20A-n75A |  |  |  | n8 |  | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n20 |  | n20 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n8A-n28A-n75A |  |  |  | n8 |  | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n28 |  | n28 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n75 |  | n75 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n8A-n28A-n78A | - |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n8A-n38A-n40A | - |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  |
|  | - |  |  | n8 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60 |  |  |
| CA_n8A-n39A-n79A | CA_n8A-n39A CA_n8A-n79A CA_n39A-n79A |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n8 |  | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n8A-n40A-n41C | CA_n41C CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A |  |  | n8 |  | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n41 |  | CA_n41C_BCS4 and 5 |  |  |
| CA_n8A-n40A-n78A | CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n8A-n41A-n79A | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n8 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n8 |  | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n79 |  | See n79 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n8A-n78A-n79A | - |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n8A-n78(2A)-n79A | - |  |  | n8 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n12A-n25A-n41A | - |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n12A-n25A-n66A | - |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n12A-n30A-n66A | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n12A-n30A-n66(2A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n12A-n30A-n66(3A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n12A-n30A-n77A | n777 CA_n12A-n30A, CA_n12A-n77A7 CA_n30A-n77A7 |  |  | n12 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n12A-n30A-n77(2A) | n777 CA_n12A-n30A CA_n12A-n77A7 CA_n30A-n77A7 |  |  | n12 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n12A-n41A-n66A | - |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
| CA_n12A-n41A-n77A | - |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n12A-n66A-n77A | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n12A-n66(2A)-n77A | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n12A-n66A-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n12A-n66(2A)-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n12A-n66(3A)-n77A | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n12A-n66(3A)-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 |  |  | n12 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n13A-n25A-n66A | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A |  |  | n13 |  | 5, 10 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n13A-n25A-n77A | CA_n13A-n25A CA_n13A-n77A CA_n25A-n77A |  |  | n13 |  | 5, 10 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n13A-n25A-n77(2A) | CA_n77(2A) CA_n13A-n25A CA_n13A-n77A CA_n25A-n77A |  |  | n13 |  | 5, 10 | 0 |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n13A-n66A-n77A | n777, 9 CA_n13A-n66A CA_n13A-n77A CA_n66A-n77A |  |  | n13 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n13A-n66A-n77(2A) | CA_n77(2A) CA_n13A-n66A CA_n13A-n77A CA_n66A-n77A |  |  | n13 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n14A-n30A-n66(3A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
| CA_n14A-n30A-n77A | n777 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n14A-n30A-n77(2A) | n777 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n14A-n66A-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n14A-n66(2A)-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n14A-n66A-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n14A-n66(2A)-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n14A-n66(3A)-n77A | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n14A-n66(3A)-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 |  |  | n14 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n18A-n28A-n41A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A |  |  | n18 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n18A-n28A-n77A | CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A |  |  | n18 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n18A-n28A-n77(2A) | CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A |  |  | n18 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n28 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n18A-n41A-n77A | CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A |  |  | n18 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n18A-n41A-n77(2A) | CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A |  |  | n18 |  | 5, 10, 15 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n20A-n28A-n75A | - |  |  | n20 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n75 |  | 5, 10, 15, 20 |  |  |
| CA_n20A-n28A-n78A | - |  |  | n20 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n20A-n28A-n78C | - |  |  | n20 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n28 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78C_BCS1 |  |  |
| CA_n24A-n41A-n48A | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n24A-n41(2A)-n48A | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS1 |  |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n24A-n41A-n48(2A) | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A) BCS0 |  |  |
| CA_n24A-n41(2A)-n48(2A) | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS1 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A) BCS0 |  |  |
| CA_n24A-n41A-n77A | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n24A-n41(2A)-n77A | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n24 |  | 5, 10 | 1 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n24A-n41A-n77(2A) | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n24 |  | 5, 10 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS0 |  |  |
| CA_n24A-n41(2A)-n77(2A) | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n24 |  | 5, 10 | 1 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS0 |  |  |
| CA_n24A-n48A-n77A |  |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n24A-n48(2A)-n77A |  |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A) BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n24A-n48A-n77(2A) |  |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS0 |  |  |
| CA_n24A-n48(2A)-n77(2A) |  |  |  | n24 |  | 5, 10 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A) BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS0 |  |  |
| CA_n25A-n29A-n66A | CA_n25A-n66A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n29 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n25A-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n25(2A)-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n25(2A)-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n25A-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n25A-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25A-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25(2A)-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25(2A)-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25A-n41A-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41A-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41C-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41(2A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41(2A)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41(3A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(3A) BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41C-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41(A-C)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(A-C) BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41A-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 |  |  | n25 |  | CA_n25(2A)_BCS1 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS1 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 30, 40 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41A-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
| CA_n25(2A)-n41(2A)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41(2A)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n41C-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41C-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41A-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41A-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n25A-n41A-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41(2A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41(2A)-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n25A-n41(2A)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41(3A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(3A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41C-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS0 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41C-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n25A-n41C-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41(A-C)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(A-C) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41A-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 |  |  | n25 |  | CA_n25(2A)_BCS1 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS1 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41A-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
| CA_n25(2A)-n41A-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
| CA_n25(2A)-n41(2A)-n71A | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41(2A)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n41(2A)-n71B |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
| CA_n25(2A)-n41C-n71A | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41C-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n41C-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
| CA_n25A-n41A-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41(2A)-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41(3A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(3A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41A-n77(2A) | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41(2A)-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n41A-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 |  |  | n25 |  | CA_n25(2A)_BCS1 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41A-n77(2A) | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n41C-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n41(2A)-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41C-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS2 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41(A-C)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A CA_n41C |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41(A-C) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n41C-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A CA_n41C |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25A-n41A-n78A | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25A-n41A-n78(2A) | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25A-n41A-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n48A-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 40, 50 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n25A-n48(2A)-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n25A-n48C-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n25A-n66A-n71A | - |  |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n25A-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n25A-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n66(2A)-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
| CA_n25A-n66(2A)-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
| CA_n25(2A)-n66A-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | CA_n25(2A)_BCS1 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
| CA_n25(2A)-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
| CA_n25A-n66A-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n66(2A)-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n66A-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25A-n66A-n77(3A) | CA_n77(2A) CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n25A-n66(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n66(2A)-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n66(2A)-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25(2A)-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25A-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25A-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25(2A)-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25(2A)-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25A-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25(2A)-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A |  |  | n25 |  | CA_n25(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25A-n66A-n85A | CA_n25A-n66A CA_n25A-n85A CA_n66A-n85A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n71A-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n71A-n77(2A) | n777,9 CA_n77(2A) CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS1 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25A-n71A-n77(3A) | CA_n77(2A) CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A) BCS1 |  |  |
| CA_n25A-n71B-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n71B-n77(2A) | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25A-n71(2A)-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n71(2A)-n77(2A) | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n71A-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 |  |  | n25 |  | CA_n25(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n71A-n77(2A) | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n71B-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n71B-n77(2A) | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25(2A)-n71(2A)-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25(2A)-n71(2A)-n77(2A) | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A |  |  | n25 |  | CA_n25(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n25A-n71A-n78A | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n25A-n71A-n78(2A) | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n25A-n71A-n85A | CA_n25A-n71A CA_n25A-n85A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n25A-n77A-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A |  |  | n26 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A |  |  | n26 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n28A-n38A-n78A | - |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n39A-n40A | CA_n28A-n39A CA_n28A-n40A CA_n39A-n40A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n39A-n41A | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n39A-n41C | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 |  |  |
| CA_n28A-n39A-n79A | CA_n28A-n39A CA_n28A-n79A CA_n39A-n79A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n40A-n41A | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n40A-n41C | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | CA_n41C_BCS0 |  |  |
| CA_n28A-n40A-n78A | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 1 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n28A-n40A-n77A | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A |  |  | n28 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n40A-n77(2A) | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A |  |  | n28 |  | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n28A-n40B-n78A | - |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n40 |  | CA_n40B_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n41A-n77A | n417 n777 CA_n28A-n41A7 |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  | CA_n28A-n77A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n41A-n77A |  |  | n77 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n41B-n77A | CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A |  |  | n28 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | CA_n41B_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n41A-n77(2A) | n417 n777 CA_n28A-n41A7 |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  | CA_n28A-n77A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  | CA_n41A-n77A |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
| CA_n28A-n41A-n77(3A) | CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A |  |  | n28 |  | 5, 10 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 90, 100 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n41A-n79C | - |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n28A-n41C-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n41C-n79C | - |  |  | n28 |  | 5, 10, 15, 20, 30 | 0 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 |  |  |
|  |  |  |  | n79 |  | CA_n79C_BCS0 |  |  |
| CA_n28A-n46A-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | 20, 40, 60, 80 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n46C-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | CA_n46C_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n46D-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | CA_n46D_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n46(2A)-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n28A-n46(2A)-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | CA_n46(2A)_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n28A-n46A-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | 20, 40, 60, 80 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n28A-n46C-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | CA_n46C_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n28A-n46D-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n46 |  | CA_n46D_BCS0 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n28A-n75A-n78A | - |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n75 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n77(3A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n78A-n79A | CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n28A-n78A-n102A | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n28A-n78A-n102B | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n28A-n78A-n102C | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n28A-n78A-n102D | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n28A-n78A-n102E | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n28A-n78A-n102(2A) | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n28A-n78(2A)-n102A | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |
| CA_n28A-n78(2A)-n102B | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102B_BCS0 |  |  |
| CA_n28A-n78(2A)-n102C | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102C_BCS0 |  |  |
| CA_n28A-n78(2A)-n102D | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102D_BCS0 |  |  |
| CA_n28A-n78(2A)-n102E | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102E_BCS0 |  |  |
| CA_n28A-n78(2A)-n102(2A) | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) |  |  | n28 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |
| CA_n29A-n30A-n66A | CA_n30A-n66A |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
| CA_n29A-n30A-n66(2A) | CA_n30A-n66A |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
| CA_n29A-n30A-n77A | n777 CA_n30A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n29A-n30A-n77(2A) | n777 CA_n30A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n30 |  | 5, 10 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n29A-n66A-n70A | - |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n29A-n66B-n70A | - |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66B_BCS0 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n29A-n66(2A)-n70A | - |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n29A-n66A-n71A | CA_n66A-n71A |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n29A-n66(2A)-n71A | CA_n66A-n71A |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n29A-n66A-n77A | n777 CA_n66A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n29A-n66(2A)-n77A | n777 CA_n66A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n29A-n66A-n77(2A) | n777 CA_n66A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n29A-n66(3A)-n77A | n777 CA_n66A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n29A-n66(2A)-n77(2A) | n777 CA_n66A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n29A-n66(3A)-n77(2A) | n777 CA_n66A-n77A7 |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n29A-n70A-n71A | CA_n70A-n71A |  |  | n29 |  | 5, 10 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n30A-n66A-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 |  |  | n30 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n30A-n66(2A)-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 |  |  | n30 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n30A-n66A-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 |  |  | n30 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n30A-n66(2A)-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 |  |  | n30 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n30A-n66(3A)-n77A | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 |  |  | n30 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n30A-n66(3A)-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 |  |  | n30 |  | 5, 10 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(3A)_BCS0 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
| CA_n38A-n66A-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n38A-n66A-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n38A-n66(2A)-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n38A-n66(2A)-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A |  |  | n38 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n39A-n41A-n79A | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n39 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n79 |  | , 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n40 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60 |  |  |
|  |  |  |  | n79 |  | , 40, 50, 60, 80, 100 |  |  |
|  |  |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n79 |  | See n79 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n40A-n41C-n79A | CA_n41C CA_n41A-n79A CA_n40A-n41A CA_n40A-n79A |  |  | n40 |  | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS4 and 5 |  |  |
|  |  |  |  | n79 |  | See n79 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n40A-n78A-n105A | CA_n40A-n78A CA_n40A-n105A |  |  | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  | CA_n78A-n105A |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
| CA_n41A-n66A-n70A | CA_n41A-n66A CA_n41A-n70A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n41A-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n41A-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n41A-n66(2A)-n71A | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n66(2A)-n71B | CA_n41A-n66A CA_n66A-n71A CA_n41A-n71A |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
| CA_n41A-n66(2A)-n71(2A) | CA_n41A-n66A CA_n66A-n71A CA_n41A-n71A |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
| CA_n41(2A)-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A |  |  | n41 |  | CA_n41(2A)_BCS1 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | CA_n41(2A)_BCS1 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n66A-n71B | CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n41(2A)-n66A-n71(2A) | CA_n41A-n66A CA_n41A-n71A CA_n66A-n71A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n41(2A)-n66(2A)-n71A | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n66(2A)-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n41(2A)-n66(2A)-n71B | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n41(3A)-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A |  |  | n41 |  | CA_n41(3A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A |  |  | n41 |  | CA_n41C_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n66A-n71B | CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n41C-n66A-n71(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n41C-n66(2A)-n71A | CA_n41A-n71A CA_n41A-n66A CA_n41C CA_n66A-n71A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n66(2A)-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n41C CA_n66A-n71A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
| CA_n41C-n66(2A)-n71B | CA_n41A-n71A CA_n41A-n66A CA_n41C CA_n66A-n71A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
| CA_n41(A-C)-n66A-n71A | n417,9 CA_n41C7 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A |  |  | n41 |  | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n77A7 CA_n41A-n66A7 CA_n66A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41A-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n66(2A)-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41(2A)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 |  |  | n41 |  | CA_n41(2A)_BCS1 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n66A-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41(3A)-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A |  |  | n41 |  | CA_n41(3A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 |  |  | n41 |  | CA_n41C_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n66A-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41(A-C)-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A |  |  | n41 |  | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n66A-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n41A-n66A-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n41A-n66(2A)-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n41A-n66(2A)-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS1 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n41A-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n70A-n78A | CA_n41A-n70A CA_n41A-n78A CA_n70A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 20, 25 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n41A-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n71B-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS 4 and 5 |  |  |
| CA_n41A-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41(2A)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | CA_n41(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41(2A)-n71A-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A |  |  | n41 |  | CA_n41(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41(3A)-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A |  |  | n41 |  | CA_n41(3A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n71(2A)-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS 4 and 5 |  |  |
| CA_n41C-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 |  |  | n41 |  | CA_n41C_BCS0 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 |  |  | n41 |  | CA_n41C_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 |  |  | n41 |  | CA_n41C_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41C-n71A-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A |  |  | n41 |  | CA_n41C BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n41(A-C)-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A |  |  | n41 |  | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n71A-n78A | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n41A-n71A-n78(2A) | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n41A-n71A-n85A | CA_n41A-n71A CA_n41A-n85A |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n41A-n77A-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n41A-n77(2A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n41A-n77(3A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS0 |  |  |
|  |  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
| CA_n41A-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A |  |  | n41 |  | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n46A-n48A-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46B-n48A-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46C-n48A-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46D-n48A-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46M-n48A-n96A | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46N-n48A-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46A-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46B-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46C-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46D-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46M-n48B-n96A |  |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46N-n48B-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46A-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46B-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46C-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46D-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46M-n48C-n96A | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46N-n48C-n96A | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46A-n48A-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46B-n48A-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46C-n48A-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46D-n48A-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46M-n48A-n96B | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46N-n48A-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46A-n48A-n96C | - |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46B-n48A-n96C | - |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46C-n48A-n96C | - |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46D-n48A-n96C | - |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46M-n48A-n96C | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46N-n48A-n96C | - |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46A-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46B-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46C-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46D-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46M-n48B-n96C | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46N-n48B-n96C | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48B_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46A-n48C-n96C | - |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46B-n48C-n96C | - |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46C-n48C-n96C | - |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46D-n48C-n96C | - |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46M-n48C-n96C | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46N-n48C-n96C | - |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46A-n48A-n96D | - |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46B-n48A-n96D | - |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46C-n48A-n96D | - |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46D-n48A-n96D | - |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46M-n48A-n96D | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46N-n48A-n96D | - |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46A-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46B-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46C-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46D-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46M-n48C-n96D | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46N-n48C-n96D | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46A-n48A-n96E | - |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46B-n48A-n96E | - |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46C-n48A-n96E | - |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46D-n48A-n96E | - |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46M-n48A-n96E | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46N-n48A-n96E | - |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46A-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46B-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46C-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46D-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46M-n48C-n96E | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46N-n48C-n96E | CA_n46A-n48A CA_n46A-n48B CA_n48A-n96A CA_n48B CA_n48B-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48C_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46A-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46B-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46C-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46D-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46M-n48(2A)-n96A | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46N-n48(2A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46A-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46B-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46C-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46D-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46M-n48(2A)-n96B | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46N-n48(2A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46A-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46B-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46C-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46D-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46M-n48(2A)-n96C | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46N-n48(2A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46A-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46B-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46C-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46D-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46M-n48(2A)-n96D | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46N-n48(2A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46A-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46B-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46C-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46D-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46M-n48(2A)-n96E | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46N-n48(2A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46A-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46B-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46C-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46D-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46M-n48(3A)-n96A | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46N-n48(3A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46A-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46B-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46C-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46D-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46M-n48(3A)-n96B | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46N-n48(3A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46A-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46B-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46C-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46D-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46M-n48(3A)-n96C | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46N-n48(3A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46A-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46B-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46C-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46D-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46M-n48(3A)-n96D | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46N-n48(3A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46A-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46B-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46C-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46D-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46M-n48(3A)-n96E | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46N-n48(3A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(3A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46A-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46B-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46C-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46D-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46M-n48(4A)-n96A | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46N-n48(4A)-n96A | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | 20, 40, 60, 80 |  |  |
| CA_n46A-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46B-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46C-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46D-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46M-n48(4A)-n96B | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46N-n48(4A)-n96B | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96B_BCS0 |  |  |
| CA_n46A-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46B-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46C-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46D-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46M-n48(4A)-n96C | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46N-n48(4A)-n96C | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96C_BCS0 |  |  |
| CA_n46A-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46B-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46C-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46D-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46M-n48(4A)-n96D |  |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46N-n48(4A)-n96D | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96D_BCS0 |  |  |
| CA_n46A-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | 10, 20, 40, 60, 80 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46B-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46B_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46C-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46C_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46D-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46D_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46M-n48(4A)-n96E | - |  |  | n46 |  | CA_n46M_BCS0 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46N-n48(4A)-n96E | CA_n46A-n48A CA_n48A-n96A |  |  | n46 |  | CA_n46N_BCS1 | 0 |  |
|  |  |  |  | n48 |  | CA_n48(4A)_BCS0 |  |  |
|  |  |  |  | n96 |  | CA_n96E_BCS0 |  |  |
| CA_n46A-n78A-n102A |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46A-n78A-n102B |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46A-n78A-n102C |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46A-n78A-n102D |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46A-n78A-n102E |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46A-n78A-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46(2A)-n78A-n102A |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46(2A)-n78A-n102B |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46(2A)-n78A-n102C |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46(2A)-n78A-n102D |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46(2A)-n78A-n102E |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46(2A)-n78A-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46C-n78A-n102A |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46C-n78A-n102B |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46C-n78A-n102C |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46C-n78A-n102D |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46C-n78A-n102E |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n48C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46C-n78A-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46D-n78A-n102A |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46D-n78A-n102B |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46D-n78A-n102C |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46D-n78A-n102D |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46D-n78A-n102E |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46D-n78A-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46A-n78(2A)-n102A |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46A-n78(2A)-n102B |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46A-n78(2A)-n102C |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46A-n78(2A)-n102D |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46A-n78(2A)-n102E |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46A-n78(2A)-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | 10,20, 40, 60, 80, 100 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46(2A)-n78(2A)-n102A |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46(2A)-n78(2A)-n102B |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46(2A)-n78(2A)-n102C |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46(2A)-n78(2A)-n102D |  | CA_n46A-n78A CA_n78A-n102A | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  | CA_n78(2A) | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46(2A)-n78(2A)-n102E |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46(2A)-n78(2A)-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46(2A)_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46C-n78(2A)-n102A |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46C-n78(2A)-n102B |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46C-n78(2A)-n102C |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46C-n78(2A)-n102D |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46C-n78(2A)-n102E |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46C-n78(2A)-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46C_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n46D-n78(2A)-n102A |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | 20, 40, 60, 80, 100 |  |  |  |  |
| CA_n46D-n78(2A)-n102B |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102B_BCS0 |  |  |  |  |
| CA_n46D-n78(2A)-n102C |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102C_BCS0 |  |  |  |  |
| CA_n46D-n78(2A)-n102D |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102D_BCS0 |  |  |  |  |
| CA_n46D-n78(2A)-n102E |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102E_BCS0 |  |  |  |  |
| CA_n46D-n78(2A)-n102(2A) |  | CA_n46A-n78A CA_n78A-n102A CA_n78(2A) | n46 |  | CA_n46D_BCS0 |  |  | 0 |  |
|  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |  |  |
|  |  |  | n102 |  | CA_n102(2A)_BCS0 |  |  |  |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48A-n70A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48A-n70A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48A-n70A |  |  | n48 |  | CA_n48(2A)_BCS1 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48A-n70A |  |  | n48 |  | CA_n48B_BCS2 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
| CA_n48A-n66A-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A |  |  | n48 |  | CA_n48(2A)_BCS1 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48B-n66A-n71A | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A |  |  | n48 |  | CA_n48B_BCS2 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n66A CA_n48A-n71A CA_n66A-n71A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
| CA_n48A-n66A-n77A | n777,9 CA_n48A-n66A CA_n66A-n77A7 |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48A-n66(2A)-n77A | CA_n48A-n66A CA_n66A-n77A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |  |
|  |  |  |  | n66 |  | CA_n66(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48A-n66A-n77C | n777,9 CA_n48A-n66A CA_n66A-n77A CA_n77C |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n48B-n66A-n77C | CA_n48A-n66A CA_n66A-n77A CA_n77C |  |  | n48 |  | CA_n48B_BCS2 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n48B-n66A-n77A | CA_n48A-n66A CA_n66A-n77A |  |  | n48 |  | CA_n48B_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n48 |  | CA_n48B_BCS1 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n48 |  | CA_n48B_BCS2 | 2 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48(2A)-n66A-n77A | CA_n48A-n66A CA_n66A-n77A |  |  | n48 |  | CA_n48(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48(2A)-n66A-n77C | CA_n48A-n66A CA_n66A-n77A |  |  | n48 |  | CA_n48(2A)_BCS0 | 0 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS0 | 1 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 | 2 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS0 |  |  |
|  |  |  |  | n48 |  | CA_n48(2A)_BCS1 | 3 |  |
|  |  |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n77 |  | CA_n77C_BCS1 |  |  |
| CA_n48A-n70A-n71A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A |  |  | n48 |  | CA_n48(2A)_BCS1 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48B-n70A-n71A | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A |  |  | n48 |  | CA_n48B_BCS2 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n70A CA_n48A-n71A CA_n70A-n71A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
| CA_n48A-n70A-n77A | CA_n48A-n70A CA_n70A-n77A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 20, 25 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48(2A)-n70A-n77A | CA_n48A-n70A CA_n70A-n77A |  |  | n48 |  | CA_n48(2A)_BCS1 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 20, 25 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48(2A)-n71A-n77A | CA_n48A-n71A CA_n71A-n77A |  |  | n48 |  | CA_n48(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n48A-n71A-n77A | CA_n48A-n71A CA_n71A-n77A |  |  | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A |  |  | n66 |  | 5, 10, 15, 20, 40 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n66A-n70A-n78A | CA_n66A-n78A CA_n70A-n78A |  |  | n66 |  | 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A |  |  | n66 |  | CA_n66B_BCS0 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A |  |  | n66 |  | CA_n66(2A)_BCS0 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 201, 251 |  |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
| CA_n66A-n70A-n77A | CA_n66A-n77A CA_n70A-n77A |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 20, 25 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n66(2A)-n70A-n77A | CA_n66A-n77A CA_n70A-n77A |  |  | n66 |  | CA_n66(2A)_BCS0 | 0 |  |
|  |  |  |  | n70 |  | 5, 10, 15, 20, 25 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n66A-n71A-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66A-n71B-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS2 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66A-n71B-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n66A-n71(2A)-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66A-n71(2A)-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A) BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n66(2A)-n71A-n77A | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 |  |  | n66 |  | CA_n66(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66A-n71A-n77(2A) | n777,9 CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A7 |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n66A-n71A-n77(3A) | CA_n77(2A) CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(3A)_BCS1 |  |  |
| CA_n66(2A)-n71B-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66(2A)-n71B-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71B_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n66(2A)-n71(2A)-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66(2A)-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n66(2A)-n71(2A)-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A |  |  | n66 |  | CA_n66(2A) BCS 4 and 5 | 4 and 5 |  |
|  |  |  |  | n71 |  | CA_n71(2A)_BCS 4 and 5 |  |  |
|  |  |  |  | n77 |  | CA_n77(2A) BCS 4 and 5 |  |  |
| CA_n66A-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n66A-n71A-n78(2A) | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A |  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n66(2A)-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A |  |  | n66 |  | CA_n66(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| CA_n66(2A)-n71A-n78(2A) | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A |  |  | n66 |  | CA_n66(2A)_BCS1 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
| CA_n66A-n71A-n85A | CA_n66A-n71A CA_n66A-n85A |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n66A-n77A-n85A | CA_n66A-n77A CA_n66A-n85A CA_n77A-n85A |  |  | n66 |  | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  |  |  | n85 |  | n85 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n70A-n71A-n77A | CA_n70A-n71A CA_n70A-n77A CA_n71A-n77A |  |  | n70 |  | 5, 10, 15, 20, 25 | 0 |  |
|  |  |  |  | n71 |  | 5, 10, 15, 20 |  |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 6: Only single uplink carriers with power class other than PC3 are listed. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 8: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination NOTE 10: For a band combination which include band n7 and n38 simultaneously, carriers in band n7 and n38 can only be configured as downlink carriers. Power imbalance between downlink carriers on Band n7 and Band n38 is assumed to be within 6dB. NOTE 11: UL carrier shall be supported in Band n28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |  |  |  |  |  |  |  |

<<< END OF CHANGES >>>