3GPP TSG-RAN WG4 Meeting #108-bis	R4-2317709

Xiamen, China, 9th October – 13th October 2023

Source:	Ericsson, BT plc

Title:	TP for 38718-02-01 adding CA_n20A-n78(2A) and DC_n20A-n78(2A)

Agenda item:			4.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 adding CA_n20A-n78(2A) and DC_n20A-n78(2A).

# 2. Text Proposal

# ---Start of changes---

## 5.x		CA_n20-n78

### 5.x.1		Specific for 1 bands UL CA

#### 5.x.1.1 Operating bands for CA

CA_n20-n78 has already been specified and this section does not need to be revisited.

#### 5.x.1.2	Channel bandwidths per operating band for CA

DL CA_n20A-n78(2A) with UL CA_n78(2A) need to be added to in the existing configuration table of 38.101-1, see Table 5.x.1-1 below.

Table 5.x.1.2-2: Supported bandwidths per CA band combination of band n20+n78

| CA_n20A-n78(2A) | CA_n20A-n78A CA_n78(2A) | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.2.2-3 gives a non-contiguous uplink IMD interference analysis

Table 5.x.2.2-3: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.x.2.2-3, there are IMD4 and IMD6 products falling inside n20 DL, which may occur in simultaneous RX/TX configurations.

#### 5.x.1.4	∆TIB and ∆RIB values

CA_n20-n78 has already been specified and this section does not need to be revisited.

#### 5.x.1.5	REFSENs requirements

Based on the co-existence studies there is a need to define MSD values. IMD4 MSD value band n20 is reused from CA_n5-n77, CA_n67-n78 and CA_n71-n77.

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n78 | n20 | N/A | 5 | N/A | 800 | 8.6 | FDD | IMD415 |
|  | n7812 | 3350 | 10 | 1 RBSTART=25 | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 RBSTART=25 | 3750 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

---Text omitted---

### 7.x.1	Operating bands for DC_n20-n78

Table 7.x.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n20-n78 | n20, n78 |

### 7.x.2   Configurations for DC_n20-n78

Table 7.x.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n20A-n78(2A) | DC_n20A-n78A |

### 7.x.3   Maximum output power for NR-DC

Table 7.x.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n20A-n78A | 23 | +2/-3 |

---End of changes---

# Reference

[1] 		RP-231115, Revised WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE

[2]		R4-2308181, TR 38.718-02-01 v0.6.0: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE Corporation