3GPP TSG-RAN WG4 Meeting #108bis		R4-2317726

# Xiamen, China, Oct 9 – Oct 13, 2023

Agenda item:			5.30.2.4 Co-channel coexistence for LTE sidelink and NR sidelink

Source:	LG Electronics

Title:	TP for TR 38.786 On Co-channel coexistence for LTE SL and NR SL

Document for:	Approval

Introduction

This contribution provides text proposal for the co-channel coexistence of LTE SL and NR SL based on the proposals in [1].

Text Proposal

----- START of TEXT PROPOSAL for TR38.786 -----

- 8	Co-channel coexistence between LTE Sidelink and NR Sidelink

- 8.1.	Configured transmitted power for V2X UE supporting co-channel coexistence with LTE SL

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied. When NR V2X UE is configured to co-channel coexistence operation with LTE V2X and NR SCS is configured to 30kHz, the evaluation period for PUMAX,c for NR V2X sidelink is the first slot of NR SL slots overlapping with an LTE SL subframe.

- 8.2.	Relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL

Relative slot power tolerance is defined for NR V2X supporting co-channel coexistence to ensure the RX performance of the LTE V2X UE operating within the same channel. This requirement applies only to transmissions with 30kHz SCS.

The relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL is the ability of the NR V2X UE operating with 30kHz SCS to control the output power of transmitted slots during PSCCH/PSSCH transmission consisting of two slots overlapping with an LTE SL subframe (500us). The reference slot is the 1st slot overlapping with LTE SL subframe and target slot is the subsequent NR SL slot overlapping with the LTE SL subframe. The measurement period is one NR SL slot with guard symbol omitted.

----- END of TEXT PROPOSAL for TR38.786 -----

References

[1]	R4-2315549, On Co-channel coexistence for LTE SL and NR SL, LG electronics

[2] TR 38.786 v1.0.0