3GPP TSG-RAN WG4 Meeting #108bis	R4-2317727

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on NR_SL_enh2_UERF_part2

Agenda Item:	5.30.4

Source: 	LG Electronics

Document for:	Approval

1-1: Tx requirements of con-current operation

Issue 1-1-1: MOP for Uu@Licensed and SL@Un-licensed

- Proposals

  - Option 1: Defines MOP per UE as the total transmitted power from each operating band (LGE, Meta)

  - Option 2: Define MOP per each operating band in Rel-18 (vivo)

  - Option 3: Use PC3 and the MOP limit for Rel-18 inter-band concurrent operation as PC5@unlicensed band and PC3@licensed band (Oppo)

- WF

  - Defines MOP per UE as the total transmitted power from each operating band

Issue 1-1-3: PCMAX,L & PCMAX,H of configured transmitted power

- Proposals

  - Option 1:  Introduce the Ppowerclass,concurrent (Oppo)

PCMAX_L (p,q) =  min{10 log10 [pCMAX_L,c,NR (p) + pCMAX_L,c,V2X (q)] - ΔTIB,c, Ppowerclass,concurrent}

PCMAX_H (p,q) = min{10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,V2X (q)], Ppowerclass,concurrent}

  - Option 2: (LGE) :  Introduce the Ppowerclass,concurrent and PEMAX,con-current

PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p) + pCMAX_L,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL(q)], PPowerClass,con-current, PEMAX,con-current}

  - Option 3:  (Meta)

PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p)+ pCMAX_L,c,SL (q)], PPowerClass_CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL (q)], PPowerClass_CA, PEMAX,CA}

  - Option 4:  (Huawei)

Reuse Rel-17 inter-band con-current Pcmax for V2X

- WF

  - Reuse Rel-17 inter-band con-current Pcmax for V2X

2-1: LTE SL and NR SL co-channel coexistence scenarios

Issue 2-1-1: RF requirement impact due to NR 2nd slot power limitation of RAN1 agreement for LTE SL and NR SL co-channel coexistence scenario)

- Proposals

  - Option 1: Define and clarify the related UE RF requirements (LGE) (R4-2315549)

    - Define Relative Power Tolerance requirement for NR V2X UE supporting co-channel coexistence with LTE V2X

    - Clarify the reference to sub-clause 6.2.4 in 6.2E.4.1 General

    - Clarify the sub-clause 6.2E.4.1 General and reference to clause  6.2.4

  - Option 2: Not define UE RF requirement and capture RAN1 constraint in TR (Oppo)

  - Option 3:  If RAN4 is necessary to apply the RAN1 agreements in the configured Tx power, then we prefer to measure the 1st slot only for PUMAX,f,c. (Meta)

- WF

  - Continue discussion on the related UE RF requirements in next meeting