3GPP TSG-RAN WG4 #108BIS Meeting 	R4- 2317728

Xiamen, China, October, 09th – 13th, 2023

Agenda Item:	5.30.2.3

Title:	TP for TR 38.786 on the Remaining RF requirements for NR SL CA operation

Source: 	Meta Ireland

Document for:	Approval

Background

This contribution provides text proposal to update RF requirements for NR SL CA operation based on the our contribution [1].

Text Proposal

<<<<<<<<<<< Start of changes in section 6.3 >>>>>>>>>>

6.3	Tx requirements for NR SL CA operation

<<<<<<<<<<< Unchanged sections are omitted >>>>>>>>>>

6.3.4	Configured transmitted power for NR SL CA operation

For intra-band contiguous SL CA operation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.3.2 and subclause 6.3.4 respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤  PCMAX,  ≤  PCMAX_H

For SL transmission of intra-band contiguous CA when same slot pattern is used in all aggregated component carriers.

PCMAX_L = MIN{10 log10 ∑ pEMAX,C - TC, PPowerClass, SL_CA – MAX(MAX(MPR,  A-MPR) + ΔTIB,c+TC, P-MPR ), PRegulatory, PEMAX, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,C , PPowerClass, SL_CA, PRegulatory, PEMAX, CA }

where

-	For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,CA is the value given by [the sum of IE sl-maxTransPower from each CC or new IE for maximum transmitted power of SL CA], defined by TS 38.331;

-	PPowerClass, SL_CA is the maximum UE power specified in Table 6.3.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.3.2 and subclause 6.3.3 respectively;

-	TIB,c  and P-MPR are specified in clause 6.2.4 in TS38.101-1;

-	TC is the highest value TC,c among all component carriers c in the subframe over both timeslots. TC,c = 1.5 dB when NOTE 3 in Table 6.2.1-1 in TS38.101-1 applies, otherwise TC,c = 0 dB;

-	PRegulatory= 10 - Gpost connector dBm when V2X UE is within the protected zone in ETSI TS 102 792 of CEN DSRC tolling system and operating in Band n47; PRegulatory= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For intra-band SL CA operation, when at least one different numerology/slot pattern is used in aggregated cells, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

The measured configured maximum output power PUMAX,c for sidelink CA operation, when at least one slot has a different transmission numerology or slot pattern, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

<<<<<<<<<<< End of changed in TR38.786 >>>>>>>>>>

References

- R4-2311529, “Consideration on UE RF requirements for Intra-band contiguous SL CA in ITS spectrum,” Meta Ireland

- R4-2312759, “TR38.786 v0.3.0,” OPPO