3GPP TSG-RAN WG4 Meeting #108bis	R4-2317748

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on NR_ATG_UERF_part2

Agenda Item:	5.13.8

Source: 	Huawei

Document for:	Approval

Way forward

Issue 2-1-1: Minimum output power ATG UE

<Way forward>:

For Omni-directional antenna:

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

For phased array antenna:

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -19 | -19+10log10 (BWChannel /20) | -19+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Issue 2-2-1: SEM requirements

<Way forward>:

If ATG UE output power is less than or equal to 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 apply.

If ATG UE output power is larger than 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 shall be relaxed by the scaling factor (Pout – 31) dB.

NOTE: This scaling factor is only applicable to ATG airborne UE. (This sentence will be captured into the specification.)

Issue 3-1-2: Maximum input level

<Way forward>:

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Receiver characteristics specified at the antenna connector(s) | Receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -42 | -30 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -44 | -32 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table [7.3.2-3] with PCMAX_L,f,c as defined in clause [6.2.4]. |  |  |  |  |

Issue 3-2-1: Out-of-band blocking/ Spurious response

Agreement:

- Use the existing out-of-band blocking requirements in TS 38.101-1 for ATG UE and capture the following note into specification.

  - In 3GPP, the ATG UE OOB specification is defined to ensure the telecommunication link and there may be other sources of interference and regulatory issues that need to be considered when designing ATG UE, i.e. avionic equipment.

Agreement: Use the existing receiver spurious response requirement (-44dBm) in TS 38.101-1 for ATG UE.

Issue 3-2-1: Intermodulation characteristics

<Way forward>:

To reuse Intermodulation characteristics requirements specified in TS 38.101-1 for ATG UE.

Table 7.8.2-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter |  | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Pw in Transmission Bandwidth Configuration, per CC5 |  | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| PInterferer 1 (CW) |  | dBm |  | -46 |  |  |
| PInterferer 2 (Modulated) |  | dBm |  | -46 |  |  |
| BWInterferer 2 |  | MHz |  | 5 |  |  |
| FInterferer 1 (Offset) |  | MHz |  | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |  |  |
| FInterferer 2 (Offset) |  | MHz |  | 2*FInterferer 1 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. NOTE 5: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |  |

Table 7.8.2-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | BWChannel |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |