3GPP TSG-RAN WG4 Meeting #108bis												R4-2317766

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.20.5

Source:	vivo

Title:	WF on UE RF part for LP_WUS

Document for:	Approval

Topic #1: LP-WUR architectures

Sub-topic 1-1 updated Guard RB for ACS/ASCS

Issue 1-1-1: Updated number of guard RBs for LP-WUS ACS

Agreements:

- Capture simulations results contributed this meeting as company input to TR.

- Keep current agreement in reply LS as it is and add a sub-bullet to say that the required guard RBs might be updated/confirmed next meeting based on data considering additional/combined RF impairments, if needed.

Issue 1-1-2: Updated number of guard RBs for LP-WUS ASCS

Agreements:

- Update the required number of guard RBs for ASCS to 0RB~1RB for 30kHz SCS, capture in TR as RAN4 evaluation outcome.

Sub-topic 1-2 Guard RB placement within NR channel

Issue 1-2-1: Guard RBs placement for LP-WUS ACS case

Agreements:

- For ASCS, the guard RBs belongs to “WUS carrier”, the overall RBs should within “WUS carrier” bandwidth.

- For ACS,

  - Option 1: the required RBs are RB offset between WUS carrier bandwidth edge and channel edge,

    - RBs within the offset may not be blanked.

    - Guard RBs within WUS carrier bandwidth should be taken into account in addition to required RBs

  - Option 2: the required guard RBs are RBs within WUS carrier bandwidth.

    - FFS on how to allocate guard RBs for ASCS and ACS purpose

  - NOTE: for evaluation purpose, WUS carrier bandwidth is the bandwidth of WUS signal plus guard RBs, e.g., 25PRBs for 15Khz SCS and 14PRBs for 30KHz SCS

Sub-topic 1-3 UE Noise Figure

Issue 1-3-1: Noise Figure range for LP-WUR (LR)

Agreements:

- Regarding the NF in RAN1 evaluation, RAN4 will derive RF requirement based on updated NF which is feasible from coverage and implementation perspective in WI phase.

- For LP-WUS evaluation, RAN4 could use ~9dB NF and X dB SNR (FFS channel) as an example assumption for MR coverage discussion.

- Encourage companies to share assumed NF of each architecture of LR next meeting.

- RAN4 will focus on sensitivity evaluation instead of specific NF value in WI phase.

Sub-topic 1-4 WUS power range

Issue 1-4-1: Possible LP-WUS power range

Agreements:

Manufacture could declare power boosting for WUS signal is supported and the boosting level from 0 dB to [x]dB. Final [x] will be decided in WI phase based on further analysis.

- Encourage companies to provide analysis in RAN4#109 for upper bound of power boosting level.

Sub-topic 1-5 LP-WUS operation band

Issue 1-5-1: Band operation for LP-WUS

Agreements:

RAN4 can consider the following scenarios and analyse RF impacts in WI phase:

- The IMT band for LR and MR is the same, e.g., WUS within a NR band for LP-WUR, WUS and NR DL could be TDM/FDM mode.

- The IMT band for LR and MR can be different, e.g., WUS located within a NR band for LP-WUR, and another NR band for MR.

Sub-topic 1-6 New methodology for LP-WUR RF requirements

Issue 1-6-1: Performance metric for LP-WUR RF requirements

Agreements:

RAN4 should discuss a new the methodology for WUR requirements in WI phase. Following can be considered:

- BLER

- Misdetection ratio, including paging failure

- others

Issue 1-6-2: LP-WUR Sensitivity

Agreements:

- FFS, sensitivity requirement depends on RAN-P decision on LP-WUS coverage, which can be discussed in WI phase.

Issue 1-6-3: LP-WUR test cases

Agreements:

- The testability issue of LP-WUR RF requirements should be discussed in WI phase.