3GPP TSG-RAN WG4 Meeting #109	R4-2318015

Chicago, USA, 13 – 17 November 2023

3GPP TSG RAN WG1 #114bis	                                                                R1-2310649

# Xiamen, China, October 9th – 13th, 2023

# Title:	Draft reply LS on report quantity parameter setting for CQI reporting with 1Tx

# Response to:	R4-2313998

# Release:	Release 15

# Work Item:	NR_newRAT-Perf

# Source:	RAN1

# To:	RAN4

# CC:	-

# Contact Person:

Name: 	               Bo Gao

E-mail Address:	gao.bo1@zte.com.cn

# 1. Overall Description:

RAN1 would like to thank RAN4 for LS R4-2313998 on the question on report quantity parameter setting with 1Tx and would like to provide the following response.

In RAN4 #108 RAN4 discussed on a test parameter called reportQuantity for demodulation CQI reporting with 1Tx and noticed that PMI specification is not defined in TS 38.214 cl.5.2.2.2.1 in a case of the 1 antenna port. It is not clear on how PMI for Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured. After the further investigation in RAN1 specification TS 38.212, we noticed that there is a description at clause 6.3.1.1.2 about CSI sequence generation in UCI over PUCCH. According to the sentence, the bitwidth for PMI with 1 CSI-RS port is 0. Therefore, there may be a chance that we configure the report quantity as ‘cri-RI-PMI-CQI’ and UE will not report the PMI. Though, the intended behavior for UEs not supporting the “csi-ReportWithoutPMI” capability when dealing with zero length PMI is not clear, and neither is it clear why the zero length PMI is not specified in UCI over PUSCH. Furthermore, another issue was identified with TS38.331 on codebookConfig IE that it is undefined for one CSI/antenna port. Since the codebookConfig IE is optional according to TS 38.331, there is a possibility that under 1Tx scenario where no precoder is applied, gNB can omit the codebookconfig field, which means no signaling of precoder Type information and codebook restriction. RAN4 also analyzed that at least for PUSCH part, codebook type will be undefined if CodebookConfig is not signalled (as it is conditioned on codebookType) and consequently PMI bitwidth will be undefined. We do note that all the CSI tests with 1Tx in the latest RAN4 UE demodulation performance requirements assume the CSI report via PUCCH, and all the test cases do not configure codebook information. However, it may not be aligned with RAN1’s intention as codebook config is conditioned on codebookType. Question RAN4 kindly asks RAN1 to take the above information into account and clarify how PMI for Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured with 1Tx. Including what are the consequences of not configuring the optional codebookConfig IE.

Answers to the Question:

RAN1 confirms that according to the current specifications, if 1-port CSI-RS is configured as channel measurement resource, bitwidth of PMI is 0 for UCI on PUCCH. In such case, codebookConfig in the CSI-ReportConfig should NOT be configured.

# 2. Actions:

# To RAN4 group

RAN1 respectfully requests RAN4 to take the above information into consideration.

# 3. Date of Next TSG-RAN WG1 Meeting:

TSG-RAN WG1 Meeting #115	November 13th – 17th, 2023		Chicago, US

TSG-RAN WG1 Meeting #116	February 26th – March 1st, 2024	Athens, GR