3GPP TSG-RAN WG4 Meeting # 109		R4-2318033

# Chicago, US, November 13 – 17, 2023

Agenda Item:	8.3.1.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On interpretation of requirements for 4Tx coherent UL MIMO

Document for:	Approval

# Introduction

This paper is just to confirm the interpretation of the proposed requirements for 4Tx coherent UL MIMO.

# Discussion

## Premise

Before discussing two alternaive proposals [1, 2],  we share out understanding of the current RAN4 requirements for power difference and relative phase error for coherent UL MIMO for 2Tx.

Figure 1: An example of a current coherent requirement in terms of power

## Discussion

A proposal from [1] from Qualcomm is as follows.

This can be interpreted for 4Tx that the said requirements, i.e., difference, in the section 2.1 shall be met between

(p0, p1)@n and (p0, p1)@n+X

(p0, p2)@n and (p0, p2)@n+X

(p0, p3)@n and (p0, p3)@n+X

(p1, p2)@n and (p1, p2)@n+X

(p1, p3)@n and (p1, p3)@n+X

(p2, p3)@n and (p2, p3)@n+X

Another proposal [2] from vivo is as shown below.

This requires following cases on top of the six cases from 1st proposal [1].

(p0, p1, p2)@n and (p0, p1, p2)@n+X

(p0, p1, p3)@n and (p0, p1, p3)@n+X

(p0, p2, p3)@n and (p0, p2, p3)@n+X

(p1, p2, p3)@n and (p1, p2, p3)@n+X

(p0, p1, p2, p3)@n and (p0, p1, p2, p3)@n+X

During RAN4 meeting, it seems that companies preference is the 1st proposal. If we go with this way, as we shared in RAN4 reflector, a following text would be better since the scheduled ports must not be two given that TDMed is not allowed for 4Tx SRS. And the amendment would be compatible with the case for 8Tx if introduced.

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between any two ports out of the scheduled ports for UL transmission at their respective antenna connectors in any slot within the specified time window from the last transmitted SRS on the same antenna connectors, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS.

# Conclusion

We shared out understanding of proposals by [1, 2], respectively. In addition, we shared the reason of our amendment of the proposal by [1].

Proposal: if our interpretation of the requirements of [1] is correct, adopt the following text.

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between any two ports out of the scheduled ports for UL transmission at their respective antenna connectors in any slot within the specified time window from the last transmitted SRS on the same antenna connectors, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS.

# Reference

- R4-2315307, CR on Requirements for coherent UL MIMO, Qualcomm, TSG-RAN WG4 #108bis

- R4-2315830, Remaining issues of 4Tx requirements, vivo, TSG-RAN WG4 #108bis