3GPP TSG-RAN WG4 Meeting # 109		R4-2318037

# Chicago, US, November 13 – 17, 2023

Agenda Item:	11.2.5.

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Draft LS to R2-2311441 on higherPowerLimit-r17

Document for:	Approval

# Introduction

This paper addresses a following question enclosed in RAN2 LS of [1]. A draft LS can be seen in an Annex.

# Discussion

## Background

Firstly, current TS38.306 defines higherPowerLimit-r17 in a very simple way as follows.

From the above, it seems that there is no restriction to apply higherPowerLimit-r17 to any types of band combinations like NR CA, EN-DC etc.

It is, however, TS38.331 has not covered MR-DC like EN-DC for higherPowerLimit-r17 at all, while TS38.331 has covered NR CA and NR-DC for higherPowerLimit-r17. Thus, RAN4 LS of [2] was sent to RAN2 in RAN4#108.

## Answer on the question

In our view, “whether the higher power limit is applicable to NR-DC” has two aspects. One is as a feature, whether is higherPowerLimit-r17 technically applicable to NR-DC or not? The other is whether is there higherPowerLimit-r17 related requirements for NR-DC in TS38.101-1 or not?

For the 1st aspect, there is no technical reason to preclude NR-DC from applying higherPowerLimit-r17 to it as a feature.

For the 2nd aspect, there is no higherPowerLimit-r17 related requirements for NR-DC corresponding to a NOTE 7 in Table 6.2A.1.3-1 for UE maximum output power for Inter-band CA and higherPowerLimit-r17 related requirements for Configured transmitted power for Inter-band CA in 6.2A.4.1.3 in TS38.101-1.

Observation 1: The question has following two aspects.

- One is as a feature, whether is higherPowerLimit-r17 technically applicable to NR-DC or not?

- The other is whether is there higherPowerLimit-r17 related requirements for NR-DC in TS38.101-1 or not?

Observation 2: Observations for the respective aspects.

- There is no technical reason to preclude NR-DC from applying higherPowerLimit-r17 to it.

- On the other hand, there is no higherPowerLimit-r17 related requirements for NR-DC corresponding to a NOTE 7 in Table 6.2A.1.3-1 for UE maximum output power for Inter-band CA and higherPowerLimit-r17 related requirements for Configured transmitted power for Inter-band CA in 6.2A.4.1.3 in TS38.101-1.

We propose to simply share the above facts with RAN2. It is up to RAN2 on what to do for TS38.331 and TS38.306 with the consideration of the information.

Proposal: A reply LS should simply share the above observations without specific actions.

# Conclusion

This paper addressed a question enclosed in RAN2 LS of [1] and obtained following two observations and one proposal. A companion LS is drafted in an Annex.

Observation 1: The question has following two aspects.

- One is as a feature, whether is higherPowerLimit-r17 technically applicable to NR-DC or not?

- The other is whether is there higherPowerLimit-r17 related requirements for NR-DC in TS38.101-1 or not?

Observation 2: Observations for the respective aspects.

- There is no technical reason to preclude NR-DC from applying higherPowerLimit-r17 to it.

- On the other hand, there is no higherPowerLimit-r17 related requirements for NR-DC corresponding to a NOTE 7 in Table 6.2A.1.3-1 for UE maximum output power for Inter-band CA and higherPowerLimit-r17 related requirements for Configured transmitted power for Inter-band CA in 6.2A.4.1.3 in TS38.101-1.

We propose to simply share the above facts with RAN2. It is up to RAN2 on what to do for TS38.331 and TS38.306 with the consideration of the information.

Proposal: A reply LS should simply share the above observations without specific actions.

# Reference

- R2-2311441, Reply LS on higher power limit capability for inter-band UL DC, RAN2, TSG-RAN WG2 #123bis

- R4-2314886, [Power_Limit_CA_DC] Draft LS on higher power limit capability for inter-band UL DC, TSG-RAN WG4 #108

# Annex

3GPP TSG-RAN WG4 Meeting #109	R4-23xxxxx

# Chicago, US, November 13 – 17, 2023

# Title:	Reply LS on higher power limit capability for inter-band UL DC

# Response to:	R2-2311441/ R4-2318024

# Release:	Rel-17

# Work Item:	Power_Limit_CA_DC

Source:	RAN4

To:	RAN2

Cc:	NONE

Contact Person:

#### Name: Hiromasa Umeda

#### E-mail Address: hiromasa.umeda@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

RAN4 discussed the question included in R2-2311441 and would like to share the answer as follows.

- Q1:  For clarifying the applicability of the parameter “higherPowerLimit-r17”, RAN2 would like to confirm whether the higher power limit is applicable to NR-DC?

- A1: There is no technical reason to preclude NR-DC from applying higherPowerLimit-r17 to it. There is , however, no higherPowerLimit-r17 related requirements for NR-DC corresponding to a NOTE 7 in Table 6.2A.1.3-1 for UE maximum output power for Inter-band CA and higherPowerLimit-r17 related requirements for Configured transmitted power for Inter-band CA in 6.2A.4.1.3 in TS38.101-1.

2. Actions:

To RAN2 group.

ACTION: RAN4 asks RAN2 to consider the above in its further work.

3. Date of Next RAN WG4 Meetings:

3GPP RAN4#110		26 Feb – 01 March 2024		  Athens, GR

3GPP RAN4#110-bis		15 – 19 April 2024				  	 TBD, CN