3GPP TSG-RAN WG4 Meeting #109	R4-2318053

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Supporting Simulations for 256QAM UL Demodulation

Agenda item:	8.6.4.1

Document for:	Discussion

Introduction

At RAN#98 meeting, the WI was approved to study the feasibility to support UL256 QAM [1] with the following objectives of the performance part.

UL 256QAM - Specify the BS demodulation performance

In this paper, we present Nokia’s simulation results for 256QAM UL demodulation.

Discussion

Background

This paper presents the simulation results to support our corresponding discussion paper for RAN4#108 regarding UL256QAM [5]. The simulation results are shown broken down according to TxEVM and Phase Noise to represent the open issues identified at RAN4#108bis.

All simulations are with Rank 1 and measured at 70% of maximum Throughput.

Phase noise impact is included in impairment results.

CP-OFDM, Mapping Type B Results

Table 1 : 256QAM UL, With Phase Noise impact included in Impairment Result

| PT-RS | SCS, CBW | MCS | Channel | DMRS | Ideal Result | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- |
| PT-RS Enabled | 60kHz, 50MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 16.48 | 20.37 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 16.58 | 20.45 |
|  | 120kHz, 50MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 16.52 | 20.36 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 16.87 | 20.75 |
|  | 120kHz, 100MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 16.59 | 20.49 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 16.76 | 20.66 |
|  | 120kHz, 200MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 16.58 | 20.51 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 16.72 | 20.71 |
| PT-RS Disabled | 60kHz, 50MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 15.73 | 19.66 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 15.88 | 20.26 |
|  | 120kHz, 50MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 15.67 | 19.49 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 16.01 | 20.15 |
|  | 120kHz, 100MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 15.8 | 19.76 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 15.99 | 20.22 |
|  | 120kHz, 200MHz | MCS20 | TDLD30-35 | DMRS 1+1 | 15.82 | 19.89 |
|  |  | MCS20 | TDLD30-35 | DMRS 1+0 | 16.01 | 20.53 |

Conclusion

Within this contribution we present simulation results for UL 256 QAM.

In the paper, no observations were made.

- References

- RP-222909 Revised WID NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, RAN#98-e, December 2022

- TS 38.104 v18.1.0. Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 18). March 2023

- TS 38.808 v17.0.0. Technical Specification Group Radio Access Network; Study on supporting NR from 52.6 GHz to 71 GHz (Release 17). March 2021

- R1-162885, On the phase noise model for 5G New Radio evaluation, RAN1#84, Busan, Korea, April 2016

- R4-2315044 “Discussion on 256QAM UL Demodulation”, Nokia, Nokia Shanghai Bell, RAN WG4 #108bis, Xiamen