3GPP TSG-RAN WG4 Meeting #109	R4-2318057

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulations for Coverage Enhancement BS Demodulation

Agenda item:	8.27.2

Document for:	Discussion

Introduction

This document introduces to RAN4 the features associated with coverage enhancement under the Work Item “Further NR coverage enhancements” and the impact of the newly introduced features on Base Station demodulation within RAN4.

Specifically, within this paper we provide simulation results for the PRACH repetition feature introduced in RAN1 and its impact for BS demodulation performance requirements.

Discussion

PRACH Repetitions

CDL-A

The following section presents the simulation results of Nokia’s simulation campaign on PRACH repetitions and the impact for coverage enhancement.

Table 1 and Table 2 present the parameters used in our simulation campaign and Figure 1 and Figure 2 represent the missed detection probability for a 2x2 and 1x4 array respectively.

Table 1. General parameters for FR2

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Urban: 28GHz (TDD) |
| Frame structure for TDD | DDSUU (S: 10D:2G:2U) Other frame structures can be reported by companies. |
| BWP | 100MHz |
| Channel model for link-level simulation | CDL-A |
| Delay spread | Urban scenario: 100ns |
| UE velocity | Urban scenario: 3km/h |
| Number of antenna elements for BS | Urban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Note: only one polarization is used |
| Number of TxRUs for BS | 2 Note: Analog beamforming is used. Note: only one TxRU is used |
| Number of UE antenna elements | 4, one panel: (M, N, P) = (2,2,2) Note: only one polarization is used 4, one panel: (M, N, P) = (1,4,1) |

Table 2. Channel-specific parameters for PRACH simulations for FR2

| Parameter | Value |
| --- | --- |
| Format | Format B4 |
| SCS | 60 kHz |
| Performance metric | 0.1% false alarm, 1% miss-detection |
| Number of UE Tx chains | 1T |

Figure 1. Comparison of 4 PRACH transmission with wide beam and 4 PRACH transmissions with 2x2 antenna array and different beam sweeping configurations.

Figure 2. Comparison of 4 PRACH transmission with wide beam and 4 PRACH transmissions with 1x4 antenna array and different beam sweeping configurations.

PUSCH with FDSS

Table 3 : PUSCH Performance with different FDSS Filters

| Propagation Environment | DMRS | SCS | CBW | MCS (Table 1) | FD Filter | SNR @ 70% TPUT | Loss to Baseline |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | None | -7.15 |  |
|  |  |  |  |  | (1, 0.28) | -7.25 | -0.10 |
|  |  |  |  | 2 | (0.28, 1, 0.28) | -6.91 | 0.24 |
|  |  |  |  |  | (0.335, 1, 0.335) | -6.62 | 0.53 |
|  |  | 15 kHz | 5 MHz |  | (0.5, 0.1667) | -7.24 | -0.09 |
|  |  |  |  |  | None | -1.81 |  |
|  |  |  |  |  | (1, 0.28) | -1.03 | 0.78 |
|  |  |  |  | 8 | (0.28, 1, 0.28) | 1.20 | 3.01 |
|  |  |  |  |  | (0.335, 1, 0.335) | 2.44 | 4.25 |
| TDLB 100-400 | (1+0) |  |  |  | (0.5, 0.1667) | -0.25 | 1.55 |
|  |  |  |  |  | None | -6.55 |  |
|  |  |  |  |  | (1, 0.28) | -6.73 | -0.18 |
|  |  |  |  | 2 | (0.28, 1, 0.28) | -6.48 | 0.07 |
|  |  |  |  |  | (0.335, 1, 0.335) | -6.28 | 0.26 |
|  |  | 30 kHz | 10 MHz |  | (0.5, 0.1667) | -6.76 | -0.21 |
|  |  |  |  |  | None | -1.00 |  |
|  |  |  |  |  | (1, 0.28) | -0.28 | 0.72 |
|  |  |  |  | 8 | (0.28, 1, 0.28) | 2.13 | 3.13 |
|  |  |  |  |  | (0.335, 1, 0.335) | 3.50 | 4.50 |
|  |  |  |  |  | (0.5, 0.1667) | 0.48 | 1.48 |

.

Figure 3  : FDSS Impact on PUSCH for 30 KHz SCS 10 MHz CBW

Figure 4 : FDSS Impact on PUSCH for 15 KHz SCS 5 MHz CB

Conclusion

This contribution provides initial results on our simulation campaign for PRACH repetitions on Coverage Enhancement and it’s impact on BS demodulation. No observations were made:

References

- RP-221858 “Further NR coverage enhancements” RAN #96 Budapest, Hungary, June 2022

- R4-2318056 “Discussion on coverage enhancement BS Demodulation”, Nokia, Nokia Shanghai Bell, RAN WG4 #109, Chicago, Nov, 2023

- R1-2302880 “PRACH coverage enhancements” Nokia, Nokia Shanghai Bell, RAN WG1 #112-bis-e, Online, April, 2023

- R1-2211595 “PRACH coverage enhancements” Nokia, Nokia Shanghai Bell, RAN WG1 #111, Toulouse, November, 2022

- TS 38.104 v18.1.0. Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 18). March 2023