3GPP TSG-RAN WG4 Meeting # 109														R4-2318134

Chicago, USA, November 13 –  November 17, 2023

Agenda item:			8.6.5

Source:	Hisashi Onozawa (Nokia)

Title:	Topic summary for [109][128] FR2_enh_req_Ph3_part1

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

# Topic #1: Beam correspondence requirements for RRC_INACTIVE and initial access

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2319135 | Xiaomi, Nokia | Reserved for TR 38.891 v 0.8.0 for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 |
| R4-2318466 | Huawei, HiSilicon | Draft CR for beam correspondence for IDLE and INACTIVE |
| R4-2318483 | Nokia, Nokia Shanghai Bell | Text Proposal for TR 38.891 on Beam Correspondence Requirements |
| R4-2318486 Introducing beam correspondence requirement for initial access and RRC_INACTIVE | Nokia, Nokia Shanghai Bell | Withdrawn |
| R4-2318631 | Apple | Withdrawn |
| R4-2318632 | Apple | CR to TS 38.101-3 On beam correspondence requirement for EN-DC/NE-DC |
| R4-2318633 | Apple | TP for TR 38.891: Specification impact |
| R4-2318878 Discussion on beam correspondence requirements for RRC_INACTIVE and initial access | Xiaomi | Proposal 1: Introduce the tolerance for spherical coverage requirement for BC in initial access into Spec, as below Table 2-1: UE spherical coverage for power class 3 Operating band; Min EIRP at 50 %-tile CDF (dBm). n257; 11.5. n258; 11.5. n259; 5.8. n260; 8. n261; 11.5. n262; 2.9. n263; 2.3. NOTE 1: Minimum EIRP at 50 %-tile CDF in RRC_CONNECTED is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 4: Minimum EIRP at 50 %-tile CDF in initial access and RRC INACTIVE shall be reduced the lower tolerance limit by 2 dB.. |
| R4-2318981 Further evaluation on the impact of power tolerence | vivo | Observation: Under the assumed worst case, i.e., ∆P~U (-3.5, 0), the spherical coverage degradation is 1.9 dB. Proposal：Capture following highlighted wording in the spec: Table 6.2.1.3-3: UE spherical coverage for power class 3 Operating band; Min EIRP at 50 %-tile CDF (dBm). n257; 11.5. n258; 11.5. n259; 5.8. n260; 8. n261; 11.5. n262; 2.9. n263; 2.3. NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance in RRC_CONNECTED state. NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 4: The requirements in this table minus 2 dB tolerance apply to PRACH. |
| R4-2318982 | vivo | TP for TR 38.891 on impact of power control tolerance |
| R4-2319195 | ZTE Corporation | Draft CR to TS38.101-2 introduction of beam correspondence requirement for RRC_INACTIVE and initial access |
| R4-2319269 Discussion on PRACH requirements handling | Samsung | Observation 1: existing PRACH requirements covers MPR, time mask, power control and EVM which are required to be verified at beam peak direction, but there is no beam peak search according to previous agreement (no min peak EIRP requirement) Proposal 1: RAN4 to discuss following two options to resolve this issue: - Option 1: remove PRACH requirements for MPR, time mask, power control and EVM in core specification - Option 2: send LS to RAN5 to inform RAN4 guidance on test verification perspective |
| R4-2320637 | Nokia, Nokia Shanghai Bell | Withdrawn |
| R4-2320638 | Nokia, Nokia Shanghai Bell | CR to 38.101-2 Introducing beam correspondence requirement for initial access and RRC_INACTIVE |
| R4-2320969 | Apple | CR to 38.101-2 On beam correspondence requirement for EN-DC/NE-DC |
| R4-2318484 | Nokia, Nokia Shanghai Bell | Text Proposal for TR 38.891 on Implementation impact to UE |
| R4-2318485 | Nokia, Nokia Shanghai Bell | Text Proposal for TR 38.891 on UE testing impacts |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 PRACH requirement handling

Sub-topic description:

According to R4-2319269, existing PRACH requirements covers MPR, time mask, power control and EVM which are required to be verified at beam peak direction, but there is no beam peak search according to previous agreement (no min peak EIRP requirement). It proposes that RAN4 discuss following two options to resolve this issue.

Moderator note: This proposal may have impacts to legacy releases.

Open issues and candidate options before meeting:

Issue 1-1: PRACH requirements handling

- Proposals

  - Option 1: remove PRACH requirements for MPR, time mask, power control and EVM in core specification

  - Option 2: send LS to RAN5 to inform RAN4 guidance on test verification perspective

- Recommended WF

  - TBA

### Sub-topic 1-2 Power class

Sub-topic description

Currently, beam correspondence requirements are specified not only for power class 3 but also power class 5, 6 and 7. Is the agreement so far made for initial access and RRC inactive applicable for all these power classes?

Open issues and candidate options before meeting:

Issue 1-2: Power class

- Proposals

  - Option 1: Introduce the new beam correspondence requirement not only for power class 3 but also for power class 5, 6 and 7 with 2 dB tolerance in spherical coverage EIRP.

  - Option 2: FFS for power class 5, 6 and 7.

- Recommended WF

  - Option 1

### Sub-topic 1-3 Spec impact to TS 38.101-3

Sub-topic description

The current WID does not cover TS 38.101-3. It is for further discussion if the CR is necessary.

Open issues and candidate options before meeting:

Issue 1-3: Spec impact to TS 38.101-3

- Proposals

  - Option 1: No CR to TS 38.101-3

  - Option 2: Agree R4-2318632 and revise the WID in the next plenary.

- Recommended WF

  - FFS

### Sub-topic 1-4 Text Proposals

Sub-topic description

Open issues and candidate options before meeting:

Issue 1-4: Text proposals

- Proposals

  - R4-2318483 Nokia

  - R4-2318633 Apple

  - R4-2318982 vivo

  - R4-2318484 Nokia

  - R4-2318485 Nokia

- Recommended WF

  - FFS

### Sub-topic 1-5 CR/draftCR

Sub-topic description

Open issues and candidate options before meeting:

Issue 1-5: CR/draftCR to 38.101-2

- Proposals

  - R4-2318466 Huawei

  - R4-2319195 ZTE

  - R4-2320638 Nokia

  - R4-2320969 Apple

- Recommended WF

  - FFS