3GPP TSG-RAN WG4 Meeting # 109										              R4-2318175

Chicago, USA, November 13-17, 2023

Agenda item:			8.22.4

Source:	Moderator (Ericsson)

Title:	Topic summary for [109][219] NR_pos_enh2_part1

Document for:	Information

# Introduction

The document contains discussion related to the positioning core requirements for the following 4 main topics:

- Topic # 1: General (AI 8.22.2.1)

- Topic # 2: RedCap positioning (AI 8.22.2.4)

- Topic # 3: PRS/SRS bandwidth aggregation (AI 8.22.2.5)

- Topic # 4: RRM performance requirements (AI 8.22.3)

NOTE: Draft CRs are not included as they will be discussed during AH and online sessions after resolving open issues.

# Topic #1: General

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2319942 | Intel Corporation | - 37. NR_pos_enh2 (FG 37-1) Support of reduced number of samples for PRS based positioning measurements with frequency hopping - 37. NR_pos_enh2 (FG 37-2) Support of reduced number of samples in positioning measurements with PRS/SRS bandwidth aggregation |
| R4-2320366 | Ericsson | Observation 1: Agreement from the last meeting already considers the scenario highlighted by RAN1 in the LS. Proposal 1: No further RAN4 action needed to address the issue highlighted in R1-2310478. Proposal 2: Reply LS to R1-2310478 is not needed. |
| R4-2320700 | Ericsson | Updated work Split on RRM Core Requirements for Positioning |
| R4-2320701 | Ericsson | Updated Big Draft CR on Skeleton for RRM Core Requirements for Positioning |
| R4-2318278 | CATT | Proposal 8: The following UE feature-related signalling should be defined to indicate UE to perform the corresponding measurements: - Support of reduced sample positioning measurement for RedCap UE in RRC_IDLE state. |
| R4-2318279 | CATT | Proposal 9: All UE features are discussed in RAN1. RAN4 does not need to discuss it parallel. |
| R4-2320808 | Nokia, Nokia Shanghai Bell | Proposal 4: RAN4 to send Reply LS to RAN1 asking clarification, if the number of measurement samples can be independent of the two PFL groups. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: RAN1 LS on measurement definitions for positioning with bandwidth aggregation

RAN4 has received RAN1 LS in R1-2310478/R4-2318003 containing the following agreements:

Agreement Configuring up to two PFL combinations is supported (e.g. PFL1 aggregated with PFL2 and PFL3 aggregated with PFL4). - Send an LS to RAN4 (CC to RAN2 and RAN3) to inform them with the above agreement and specify corre-sponding requirements. - Note: more than one combinations are measured in TDMed manner

Issue 1-1-1: LS on PRS bandwidth aggregation

- Proposals:

  - Option 1: Ericsson

    - No further RAN4 action needed to address the issue highlighted in R1-2310478.

    - Reply LS to R1-2310478 is not needed.

  - Option 2: Nokia

    - RAN4 to send Reply LS to RAN1 asking clarification, if the number of measurement samples can be independent of the two PFL groups.

  - Option 3: CATT

    - Measurement requirements for one PFL combination also apply to the cases when two PFL combinations are configured. No extra RAN4 work is required.

- Recommended WF

  - Discuss the options and prepare a draft LS response if there is consensus to ask RAN1 for further clarification.

### Sub-topic 1-2: Work split and timeline for RRM core requirements

The RRM core requirements for Rel-18 WI on Expanded and improved NR positioning are supposed to be completed in RAN4#109 (November 2023) in accordance with the timeline in the latest approved WID,

Issue 1-2-1: Rapporteur input on updated work split between companies

- Proposals:

  - Option 1: Ericsson (rapporteur)

    - Following new draft CRs are included in the updated work split:

      - Draft CRs # 2A, # 4A, # 7A and # 14A.

Table 1: Updated work split on major topics and corresponding RRM core requirements for positioning measurements.

| DraftCR No. | Requirements for feature | Detail of requirements | New or impacted section in TS 38.133 | Volunteer Company |
| --- | --- | --- | --- | --- |
| 1 | All | Draft Big CR for RRM Core Requirements for Positioning Enhancement | All relevant sections in TS 38.133 | Ericsson (Rapporteur) |
| 2 | PRS measurement requirements in RRC idle state | General aspects: introduction | 4.x1.1 | Qualcomm |
|  |  | RSTD measurement requirements | 4.x1.3 |  |
| 2A |  | Cell re-selection for positioning | 4.x1.2 | Huawei |
| 3 |  | PRS-RSRP measurement requirements | 4.x1.4 | OPPO |
|  |  | PRS-RSRPP measurement requirements | 4.x1.5 |  |
| 4 | PRS measurement requirements for RedCap in RRC idle state | General aspects: introduction | 4.x1A.1 | Xiaomi |
|  |  | RSTD measurement requirements | 4.x1A.3 |  |
| 4A |  | Cell re-selection for positioning | 4.x1A.2 | Huawei |
| 5 |  | PRS-RSRP measurement requirements | 4.x1A.4 | CATT |
|  |  | PRS-RSRPP measurement requirements | 4.x1A.5 |  |
| 6 | PRS measurement requirements in RRC inactive state (LPHAP related updates) | General aspects: introduction (including general aspects of PRS measurements with bandwidth aggregation and CPP) | 5.6.1 (existing section) | Ericsson |
| 7 |  | Cell re-selection for positioning | 5.6.1A | Huawei |
| 7A |  | TA validation requirements for positioning measurements | 5.6.1B | Ericsson |
| 8 |  | RSTD measurement requirements | 5.6.2 (existing section) | CMCC |
|  |  | PRS-RSRP measurement requirements | 5.6.3 (existing section) |  |
| 9 |  | UE Rx-Tx time difference measurement requirements | 5.6.4 (existing section) | CATT |
|  |  | PRS-RSRPP measurement requirements | 5.6.5 (existing section) |  |
| 10 | PRS measurement requirements with bandwidth aggregation in RRC inactive state | RSTD measurement requirements | 5.6.2.x1 | Huawei |
|  |  | UE Rx-Tx time difference measurement requirements | 5.6.4.x1 |  |
| 11 | CPP: DL RSCP and DL RSCPD in RRC inactive state | Measurement requirements for DL RSCPD reported with RSTD | 5.6.x1 | Nokia |
| 12 |  | Measurement requirements for DL RSCP reported with UE Rx-Tx time difference | 5.6.x2 | Nokia |
| 13 | PRS measurement requirements for RedCap in RRC inactive state | General aspects: introduction | 5.6A.1 | Xiaomi |
| 14 |  | Cell re-selection for positioning | 5.6A.2 | Huawei |
| 14A |  | TA validation requirements for positioning measurements | 5.6A.3 | Ericsson |
| 15 |  | RSTD measurement requirements | 5.6A.4 | Ericsson |
|  |  | PRS-RSRP measurement requirements | 5.6A.5 |  |
| 16 |  | UE Rx-Tx time difference measurement requirements | 5.6A.6 | Xiaomi |
|  |  | PRS-RSRPP measurement requirements | 5.6A.7 |  |
| 17 | UE autonomous timing adjustment at cell reselection in RRC inactive | UE transmit timing for positioning measurements | 7.1E | LG |
| 18 | PRS measurement requirements with bandwidth aggregation in RRC connected state | General aspects: introduction (include also general aspects of CPP) | 9.9.1 (existing section) | ZTE |
| 19 |  | RSTD measurement requirements | 9.9.2.x1 | Ericsson |
|  |  | UE Rx-Tx time difference measurement requirements | 9.9.4.x1 |  |
| 20 | PRS measurement requirements for RedCap in RRC connected state | General aspects: introduction | 9.9A.1 | ZTE |
| 21 |  | RSTD measurement requirements | 9.9A.2 | Mediatek |
|  |  | PRS-RSRP measurement requirements | 9.9A.3 |  |
| 22 |  | UE Rx-Tx time difference measurement requirements | 9.9A.4 | Huawei |
|  |  | PRS-RSRPP measurement requirements | 9.9A.5 |  |
| 23 | CPP: DL RSCP and DL RSCPD in RRC connected state | Measurement requirements for DL RSCPD reported with RSTD | 9.9.x1 | Ericsson |
| 24 |  | Measurement requirements for DL RSCP reported with UE Rx-Tx time difference | 9.9.x2 | Nokia |
| 25 | NR sidelink positioning measurements | General aspects: introduction | 12A.1 | Ericsson |
| 26 |  | SL-RSTD measurement requirements | 12A.2 | Ericsson |
|  |  | SL-RSRP measurement requirements | 12A.3 |  |
| 27 |  | SL-Rx-Tx time difference measurement requirements | 12A.4 | CATT |
|  |  | SL-RSRPP measurement requirements | 12A.5 |  |
| 28 |  | SL-AoA measurement requirements | 12A.6 | Vivo |
|  |  | SL-RTOA measurement requirements | 12A.7 |  |
| NOTE: Company providing draft CR to include the draftCR No. in the draft CR title e.g. Draft CR # X: Title |  |  |  |  |

- Recommended WF

  - Discuss the updated work split.

### Sub-topic 1-3: Big Draft CR template and structure for RRM core part

The big draft CR template was endorsed in R4-2317387 at RAN4#108bis to include new clauses for defining RRM core requirements for positioning in Rel-18.

Issue 1-3-1: Rapporteur input on updated Big Draft CR structure for RRM core requirements

- Proposals:

  - Option 1: Ericsson (rapporteur)

- Recommended WF

  - Discuss the proposed updated Big Draft CR structure in R4-2320701 to account for new draft CRs (# 2A, # 4A, # 7A and # 14A) in R4-2320700 (updated work split).

### Sub-topic 1-4: UE feature list

Issue 1-4-1: General discussion on UE feature

- Proposal:

  - Option 1: CATT

    - All UE features are discussed in RAN1. RAN4 does not need to discuss it parallel.

- Recommended WF

  - Discuss the option.

Issue 1-4-2: Feature on reduced number of samples for PRS measurement in RRC_IDLE for RedCap

- Proposal:

  - Option 1: CATT

    - The following UE feature-related signalling should be defined to indicate UE to perform the corresponding measurements:

      - Support of reduced sample positioning measurement for RedCap UE in RRC_IDLE state.

- Recommended WF

  - Discuss the option.

Issue 1-4-3: Features on PRS measurements with FH for RedCap (FG 37-1)

- Proposal:

  - Option 1: Intel

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 | Support of reduced number of samples for PRS based positioning measurements with frequency hopping | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 45-5-1 | Yes | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false | Optional with capability signalling |

- Recommended WF

  - Discuss any RAN4 impact on feature 37-1 in R4-2319942.

Issue 1-4-4: Features on PRS measurements with PRS/SRS BW aggregation (FG 37-2)

- Proposal:

  - Option 1: Intel

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-2 | Support of reduced number of samples in positioning measurements with PRS/SRS bandwidth aggregation | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation 2. Support of reduced number of samples in positioning measurements with SRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 Component 2 RAN1 feature 41-4-6 | Yes | NA | UE does not support reduced number of samples in positioning measurements with PRS and/or SRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/false # Component 2 candidate value: true/false | Optional with capability signalling |

- Recommended WF

  - Discuss any RAN4 impact on feature 37-2 in R4-2319942.

# Topic #2: RedCap positioning

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2318278 | CATT | Proposal 1: Whether to support gapless based PRS measurements with FH is up to RAN1 and no RAN4 work until RAN1 decides to support such case. Proposal 3: The side conditions for measurements without FH can be reused for measurements with FH. Proposal 4: Measurement sample under FH can be defined as a PRS measurement over multiple hops within a single measurement gap. Proposal 5: The Rx beam sweeping factor definition in Rel-17 is reused for PRS measurements with FH. Proposal 6: Explicit request by LMF for measurements with FH should be one of the applicability conditions for measurement requirements. Proposal 8: The following UE feature-related signalling should be defined to indicate UE to perform the corresponding measurements: - Support of reduced sample positioning measurement for RedCap UE in RRC_IDLE state. |
| R4-2318857 | Xiaomi | Requirements in RRC_IDLE Proposal 1: The requirements of RedCap positioning without FH in RRC_IDLE/RRC_INACTIVE can be firstly implemented in draftCR. Proposal 2: The following requirements of RedCap Pos in RRC_IDLE can be specified: - RSTD - PRS-RSRP and PRS-RSRPP Measurement reporting period requirements Observation 1: The different accuracy measurement requirements will be expected when UE reporting the single measurement result based on the multiple hops or single hop. Observation 2: The requirement for measurements based on both the multiple hopes and single hop within a gap are expected to be defined in RAN4. These two different types of requirements will be applied for UE upon the indication on the number of hops associated with the measurement reporting. Observation 3: There is some ambiguity on the “measurement samples” used to define the measurement requirements in current spec [4] when UE reporting measurements over the hops within a gap. Proposal 3: The “measurement samples” used to define the measurement requirements in current spec [4] shall be revisited in case of UE reporting measurements over the hops within a gap. UE complexity impacts Observation 4: UE soft buffer potentially needs to be enlarged to support PRS RX hopping. Proposal 4: In order to support RX hoping PRS in Redcap UE, the additional UE capability with enlarged soft buffer size shall be considered. PRS measurement requirements without gap Proposal 5: RAN4 can FFS measurement without gap for RedCap UE’s positioning after RAN1’s design stable. Frequency hopping in UL Observation 5: The hopping configuration for SRS will impact on how to define UE Rx-Tx time difference requirements Observation 6: The uncertainty of UL SRS transmission time will be larger than that of without FH. The total requirements for UE Rx-Tx time difference will be impacted. Proposal 6: RAN4 can discuss UE Rx-Tx time difference requirements for Redcap UE with frequency hoping SRS upon RAN1’s conclusion, e.g. - The legacy requirements on UE Rx-Tx time deference needs to be relaxed due to the additional switching time. - Scheduling restriction because of the collision due to SRS TX hopped Observation 7: From UE behavior and RRM requirements itself, the measurement on the outside of UL active BWP with the different numerology as UL active BWP needs additional UE capability supported. |
| R4-2319483 | OPPO | Proposal 1: For RedCap UE with FH, gapless based PRS measurement is not supported. Proposal 2: PRS measurement with FH in RRC_INACTIVE and RRC_IDLE states can be supported subject to UE capability, and the related requirements should be defined in RAN4 if time allows. Proposal 3: As baseline, re-use CSSF definition for non-RedCap UE in clause 9.1.5.2.2 to include the impact of PRS measurement in CSSF definition for RedCap UE in clause 9.1A.5.2. Proposal 4: The Rx beam sweeping factor NRxBeam definition in Rel-17 can be reused for RedCap UE with FH. |
| R4-2319996 | Huawei, HiSilicon | Proposal 1: RAN4 not to define requirements for PRS measurements with FH without gaps. Proposal 2: RAN4 stick to the existing agreements for requirements for PRS measurements with FH in INACTIVE and IDLE. No further discussion is needed. Proposal 3: Re-use CSSF definition for non-RedCap UE in clause 9.1.5.2.2 to include the impact of PRS measurement in CSSF definition for RedCap UE in clause 9.1A.5.2. Proposal 4: The Rx beam sweeping factor (NRxBeam) definition in Rel-17 is reused for PRS measurements with FH. Proposal 5: RAN4 to define requirements at least for Case 1, FFS whether and how to define requirements for Case 2 or combined Case 1 and Case 2 depending on RAN1 conclusion on the condition and UE behaviour for Case 2. - Case 1: UE reports measurement based on multiple hops - Case 2: UE reports measurement associated to a single hop Proposal 7: The number of hops within a single MG occasion Nhop is defined as Nhop=minN-1*MK+1,Nhop,max where - N is the number of PRS repetitions within the MG occasion - M is the PRS repetition interval (given by dl-PRS-ResourceTimeGap) - K is the number of slots per hop and K=maxK',M - K'=1 if the RF switching time is ≤ 7 symbols and K'=2 otherwise - Nhop,max is the maximum number of hops derived from the configured PRS BW BWPRS Proposal 8: For Case 1, existing requirements for MG-based measurement are re-used as baseline, and the following adaptations are considered: - Lprs = Nhop * Lper_hop, where Nhop is the number of hops that UE can do in an MG occasion, and Lper_hop is the PRS duration per hop; - The requirements are applicable only to PRS resource in the sampling duration in each hop. Proposal 9: RAN4 not to further discuss enlarged soft buffer for FH given the RAN1 feature list. Proposal 10: Requirements for PRS measurements with FH are applicable subject to explicit request by the LMF to perform measurements with FH. |
| R4-2319997 | Huawei, HiSilicon | Simulation results for PRS measurement with FH |
| R4-2320370 | Ericsson | # Number of samples in measurement period requirement for latency reduced positioning Proposal 1: When UE is configured to perform positioning measurements with FH then Nsample = 2 is considered in measurement period requirement for reduced number of sample case. # Measurement period requirement with FH Proposal 2: Lavailable for PRS measurement period requirement with FH is calculated as: Lavailable= Lavailable per hop × Nhops + RFretuning time × (Nhops -1), where - Lavailable per hop is the time duration of PRS resource UE can measure in each hop and depends on the UE capability, - RFretuning time is the RF retuning time based on UE reported capability, - Nhops is calculated based on the DL PRS bandwidth UE can measure across all hops, PRS bandwidth UE can measure per hop and overlapping between hops supported by the UE. Nhops = BWmaxBWper hop- PRBoverlapping. BWTotal is the DL PRS bandwidth across all hops. BWper hop is the DL PRS BW UE can measurement in each hop. And PRBoverlapping is the number of overlapping PRBs between hops. Proposal 3: Do not define requirements for the case when overlapping between adjacent hops is not considered for PRS measurements. Proposal 4: CSSF as defined in clause 9.1A.5.2 shall apply for PRS measurement period requirements for RedCap UEs in RRC_CONNECTED state. Proposal 5: Update clause 9.1A.5.2 to be valid also for RedCap positioning. |
| R4-2320371 | Ericsson | Additional simulation results for RedCap positioning with FH |
| R4-2320372 | Ericsson | Summary of simulation results for RedCap positioning with FH |
| R4-2320855 | Nokia, Nokia Shanghai Bell | Proposal 1: For RedCap UE with FH, whether to support gapless based PRS measurement is up to RAN1. Proposal 2: RAN4 to reuse CSSF definition for non-RedCap UE in clause 9.1.5.2.2 to include the impact of PRS measurement in CSSF definition for RedCap UE in clause 9.1A.5.2. Proposal 4: RAN4 to reuse the existing definition of measurement samples for reporting measurements over the hops. Proposal 5: Rx beam sweeping factor definition in Rel-17 is reused for PRS measurements with FH. Proposal 6: RAN4 to define PRS measurement period requirements for single and multiple hop measurements, respectively. Proposal 8: The issues on soft buffer size for combining multiple hops of RedCap UE is not within the scope of RAN4 discussion. Proposal 9: RAN4 should not define any requirement for an explicit request by LMF to perform measurements with FH. |
| R4-2320856 | Nokia, Nokia Shanghai Bell | Simulation Results for RedCap Positioning with Frequency Hopping |
| R4-2320857 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to define the different PRS measurement period requirements for single and multiple hop measurements, respectively. |
| R4-2320913 | Qualcomm Incorporated | Proposal 1: The minimum PRS BW expected to be measured is given by minBWPRS,Nhops∙BWhop-BWoverlap∙Nhops-1 where - BWPRS is the configured PRS BW - BWhop is the BW per hop signaled in the UE capability - BWoverlap is the minimum hop overlap signaled in the UE capability - Nhops=Nhopsslot∙NrepPRS if MrepPRS=1 or Nhopsslot≥1, otherwise Nhops=1+NrepPRS-11MrepPRS∙Nhopsslot - NrepPRS is the number of PRS inter-slot repetitions within a single MG instance, excluding the gap retuning times - MrepPRS is the stride of PRS inter-slot repetitions - Nhopsslot is the number of frequency hops per slot as a function of number of PRS symbols, PRS comb size, and retuning time between hops. FFS how to determine Nhopsslot from these parameters and whether to limit its maximum value. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: General aspects/scenarios for RedCap positioning

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1-1: PRS measurements with FH without gaps

- Proposals

  - Option 1: CATT, Xiaomi, Nokia

    - Whether to support gapless based PRS measurements with FH is up to RAN1 and no RAN4 work until RAN1 decides to support such case.

  - Option 2: HW, OPPO

    - RAN4 not to define requirements for PRS measurements with FH without gaps.

- Recommended WF

  - Discuss the options.

Issue 2-1-2: Applicable RRC states for PRS measurements without FH

- Proposals

  - Option 1: Xiaomi

    - The requirements of RedCap positioning without FH in RRC_IDLE/RRC_INACTIVE can be firstly implemented in draftCR.

    - The following requirements of RedCap Pos in RRC_IDLE can be specified:

      - RSTD

      - PRS-RSRP and PRS-RSRPP

- Recommended WF

  - Discuss the option.

Issue 2-1-3: Applicable RRC states for PRS measurements with FH

In the approved WF in R4-2314353 at RAN4#108 the following was agreed:

- Issue 2-1-1: Applicable RRC states for Redcap positioning with FH

  - RAN4 will prioritize defining Redcap positioning requirements with FH in RRC connected and then will define Redcap positioning requirements with FH in RRC inactive state and RRC idle state.

- Proposals

  - Option 1: OPPO

    - PRS measurement with FH in RRC_INACTIVE and RRC_IDLE states can be supported subject to UE capability, and the related requirements should be defined in RAN4 if time allows.

  - Option 2: HW

    - RAN4 stick to the existing agreements for requirements for PRS measurements with FH in INACTIVE and IDLE. No further discussion is needed.

- Recommended WF

  - Considering the past agreement it is recommended not to reopen this issue and follow the agreement.

### Sub-topic 2-2: PRS measurements for RedCap without FH

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-2-1: CSSF for PRS measurement requirements in RRC connected state without FH

- Proposals

  - Option 1: HW, OPPO, E///

    - Option 1A: HW, OPPO, Nokia

      - Re-use CSSF definition for non-RedCap UE in clause 9.1.5.2.2 to include the impact of PRS measurement in CSSF definition for RedCap UE in clause 9.1A.5.2.

    - Option 1B: E///

      - CSSF as defined in clause 9.1A.5.2 shall apply for PRS measurement period requirements for RedCap UEs in RRC_CONNECTED state.

      - Update clause 9.1A.5.2 to be valid also for RedCap positioning.

- Recommended WF

  - Check if it is agreeable to update clause 9.1A.5.2?

### Sub-topic 2-3: PRS measurements for RedCap with FH

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-3-1: Side conditions for PRS measurements with FH

- Proposals

  - Option 1: CATT

    - The side conditions for measurements without FH can be reused for measurements with FH.

- Recommended WF

  - Discuss the option together with the compiled simulation results under option 2-3-2.

Issue 2-3-2: Collection of simulation results for PRS measurements with FH

- Proposals

  - Moderator suggests that in first round all the simulation results are collected and compiled e.g. in R4-2320372.

- Recommended WF

  - Discuss based on the compiled results.

Issue 2-3-3: Nsample for reduced number of samples in PRS measurement requirements with FH

- Proposals

  - Option 1: E///

    - When UE is configured to perform positioning measurements with FH then Nsample = 2 is considered in measurement period requirement for reduced number of sample case.

- Recommended WF

  - Discuss the options.

Issue 2-3-4: Measurement sample definition under FH

- Proposals

  - Option 1: CATT

    - Measurement sample under FH can be defined as a PRS measurement over multiple hops within a single measurement gap.

  - Option 2: Xiaomi

    - The “measurement samples” used to define the measurement requirements in current spec [4] shall be revisited in case of UE reporting measurements over

  - Option 3: Nokia

    - RAN4 to reuse the existing definition of measurement samples for reporting measurements over the hops.

- Recommended WF

  - Discuss the options.

Issue 2-3-5: Number of Rx beam sweeps for defining PRS measurement requirements with FH

- Proposals

  - Option 1: HW, CATT, OPPO, Nokia

    - The Rx beam sweeping factor (NRxBeam) definition in Rel-17 is reused for PRS measurements with FH.

- Recommended WF

  - Check if option 1 is agreeable?

Issue 2-3-6: Number of hops within a single MG occasion

- Proposals

  - Option 1: HW

      - The number of hops within a single MG occasion Nhop is defined as

Nhop=minN-1*MK+1,Nhop,max

where

- N is the number of PRS repetitions within the MG occasion

- M is the PRS repetition interval (given by dl-PRS-ResourceTimeGap)

- K is the number of slots per hop and K=maxK',M

- K'=1 if the RF switching time is ≤ 7 symbols and K'=2 otherwise

        - Nhop,max is the maximum number of hops derived from the configured PRS BW BWPRS

    - Option 2: QC

      - Nhops=Nhopsslot∙NrepPRS if MrepPRS=1 or Nhopsslot≥1, otherwise Nhops=1+NrepPRS-11MrepPRS∙Nhopsslot

      - NrepPRS is the number of PRS inter-slot repetitions within a single MG instance, excluding the gap retuning times

      - MrepPRS is the stride of PRS inter-slot repetitions

      - Nhopsslot is the number of frequency hops per slot as a function of number of PRS symbols, PRS comb size, and retuning time between hops. FFS how to determine Nhopsslot from these parameters and whether to limit its maximum value.

Issue 2-3-7: Type of PRS measurement requirements with FH

- Proposals

  - Option 1: Nokia

    - RAN4 to define the different PRS measurement period requirements for single and multiple hop measurements, respectively.

  - Option 2: HW

    - RAN4 to define requirements at least for Case 1, FFS whether and how to define requirements for Case 2 or combined Case 1 and Case 2 depending on RAN1 conclusion on the condition and UE behaviour for Case 2.

      - Case 1: UE reports measurement based on multiple hops

      - Case 2: UE reports measurement associated to a single hop

  - Option 3: E///

Do not define requirements for the case when overlapping between adjacent hops is not considered for PRS measurements.

Issue 2-3-8: PRS measurement period requirements with FH based on multiple hops

- Proposals

  - Option 1: E///

    - Lavailable for PRS measurement period requirement with FH is calculated as:

Lavailable= Lavailable per hop × Nhops + RFretuning time × (Nhops -1),
where

- Lavailable per hop is the time duration of PRS resource UE can measure in each hop and depends on the UE capability,

- RFretuning time is the RF retuning time based on UE reported capability,

- Nhops is calculated based on the DL PRS bandwidth UE can measure across all hops, PRS bandwidth UE can measure per hop and overlapping between hops supported by the UE.  Nhops = BWmaxBWper hop- PRBoverlapping. BWTotal is the DL PRS bandwidth across all hops. BWper hop is the DL PRS BW UE can measurement in each hop. And PRBoverlapping is the number of overlapping PRBs between hops.

  - Option 2: Xiaomi

    - RAN4 can discuss UE Rx-Tx time difference requirements for Redcap UE with frequency hoping SRS upon RAN1’s conclusion, e.g.

      - The legacy requirements on UE Rx-Tx time deference needs to be relaxed due to the additional switching time.

      - Scheduling restriction because of the collision due to SRS TX hopped

  - Option 3: HW

    - For Case 1, existing requirements for MG-based measurement are re-used as baseline, and the following adaptations are considered:

      - Lprs = Nhop * Lper_hop, where Nhop is the number of hops that UE can do in an MG occasion, and Lper_hop is the PRS duration per hop;

      - The requirements are applicable only to PRS resource in the sampling duration in each hop.

- Recommended WF

  - Discuss the options.

Issue 2-3-9: UE capability with enlarged soft buffer to support PRS Rx hopping

- Proposals

  - Option 1: Xiaomi

    - In order to support RX hoping PRS in Redcap UE, the additional UE capability with enlarged soft buffer size shall be considered.

  - Option 2: HW

    - RAN4 not to further discuss enlarged soft buffer for FH given the RAN1 feature list.

  - Option 3: Nokia

    - The issues on soft buffer size for combining multiple hops of RedCap UE is not within the scope of RAN4 discussion.

- Recommended WF

  - Discuss the options.

Issue 2-3-10: Applicability conditions for PRS measurement requirements with FH

- Proposals

  - Option 1: HW, CATT

    - Requirements for PRS measurements with FH are applicable subject to explicit request by the LMF to perform measurements with FH.

  - Option 2: Nokia

    - RAN4 should not define any requirement for an explicit request by LMF to perform measurements with FH.

- Recommended WF

  - Discuss the options.

# Topic #3: PRS/SRS BW aggregation

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2318279 | CATT | Observation: It is difficult to verify whether the aggregated PRS resources are transmitted from the same Tx RF chain in the tests. Proposal 1: RAN4 not to capture the condition of single RF chain (same antenna) in RAN4 specs which should be a prerequisite for BW aggregation. Proposal 2: Measurement requirements for one PFL combination also apply to the cases when two PFL combinations are configured. No extra RAN4 work is required. Proposal 3: Requirements for PRS bandwidth aggregation are applicable provided that the linked PRS resource sets are from the same TRP. Option 2 is also supported: - Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL. - Requirements for aggregate measurement are applicable to PRS resources in the resource sets that are indicated to be linked, provided that the alignment conditions defined by RAN1 are met. Proposal 4: The existing measurement period requirements can be reused for non-aggregated measurements, while it can be baseline for aggregated measurements with some modifications, e.g., new UE processing capabilities in FG 41-4-1/1a. Details can be discussed in draft CR. Proposal 5: Tmargin=max(Teffect,i) is added to align non-aggregated and aggregated measurements, where Teffect,i is the maximum periodicity of measurement in non-aggregated/aggregated PFLs and the definition is the same as that in the existing measurement period requirements. Proposal 6: The bandwidth aggregation requirements are applicable provided that no PRS/SRS resources dropped. Proposal 7: RAN4 not to define UE behaviour which shall depend on priority rules or UE implementation when collisions occur. Proposal 8: Interruptions at NR SRS carrier based switching defined in TS 38.133 clause 8.2.2.2.9 can be reused for the interruption due to SRS bandwidth aggregation. Detailed interruption time can be further determined based on RF session conclusion. |
| R4-2318859 | Xiaomi | PRS bandwidth aggregation Observation 1: The optimized UE implementations when the PRS collided with other signals in the aggregated PFLs are feasible and beneficial to improve all PRS resource utilization. Observation 2: At least when there is colliding on PRS with other signal, UE can fallback to the measurement with non-aggregated bandwidth aggregation PRS measurement indeed. Proposal 1: When the PRS collision with other signals on PRS bandwidth aggregation requirement, UE’s measurement can rely on the PRS of the PFL which is not collided. Observation 3-1 : For Taggregate as UE needs to measure the PRS from all aggregated PFLs, the requirement in Rel16 can be reused for it but with the replacement the number of PFL “L” with the number for PLF group number “M”. Observation 3-2: For Tlast_agg, m , regarding to the case in which the processing time of the last PRS measurement for the different PFLs within an aggregated PFL group are different, we suggest to specify the Tlast_agg, m as max(Tlast_agg, i) for each layer(“i”th layer) because Tlast is used to algin the resource to be measured in each PFL. Observation 3-3: For Tnonaggregate , the requirement in Rel16 can be completed reused. Observation 3-4: It is unnecessary to include any other Tmarign if Tlast_agg, m and Tlast_nonagg, n were counted into the overall measurement delay. Proposal 2: The total measurement period for RSTD and UE Rx-Tx requirements with bandwidth aggregation across all PFLs is defined as Txxxx,total = Tagg + Tnon-agg (1) Wherein, T agg=m=1MTagg, m (2) where M is the number of PFL groups to be aggregated, and for each PFL group: Tagg, m=kmultiTEG_agg, m*CSSFPRS_agg, m*ceilKp, PRS_agg, m*NRxBeam_agg, m*NPRS_agg, mslotNagg, m'Lavailable_PRS_agg, mNagg, m*Nsample-1*Teffect_agg, m+ Tlast_agg, m (3) Tlast_agg, m = max(Tlast_agg, i) (4) Wherein, i is the index of PFL layers within a PFL aggregated group. Tnon-agg=m=1MTnon-agg, m (5) m is the number of PFL to be measured separately. T non-agg, m=kmultiTEG,non-agg, m*CSSFPRS_non-agg, m*ceilKp, PRS_non-agg, m*NRxBeam_non-agg, m*NPRSnon-agg, mslotNnon-agg, m'LavailablePRSnon-agg, mNnon-agg, m*Nsample-1*Teffect_non-agg, m +Tlast_non-agg, m (6). SRS bandwidth aggregation Proposal 3: RAN4 can discuss the impacts due to SRS bandwidth aggregation for UE Rx-Tx time difference requirements firstly. Observation 4: For UE Rx-Tx time difference measurements with SRS aggregation, if SRS in one of aggregated PFL is dropped in a symbol, UE can restart new measurements. Proposal 4: The requirements of UE Rx-Tx time difference measurements with SRS aggregation is applicable only when there is no any dropped aggregated SRSs. Proposal 5: RAN4 to define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1 and RF session. Requirements for SRS carrier switching or antenna switching can be re-used as baseline. |
| R4-2319096 | CMCC | Proposal 1: for applicable aggregated bandwidth, no need to deprioritize measurement period and measurement performance requirements for unequal PFL bandwidth for PRS/SRS BW aggregation. It is proposed to consider both equal and unequal PFL bandwidth, and same measurement period and measurement performance requirements apply to both cases. |
| R4-2319307 | LG Electronics Inc. | Proposal 1: RAN4 to investigate as baseline for measurement performance requirements a single carrier bandwith including maximum bandwidth case. (Support - Option 1) Proposal 2: RAN4 to define interruption requirements or scheduling restriction for SRS transmission for BW aggregation on UL communication CC. |
| R4-2319484 | OPPO | Proposal 1: The scenario that aggregated SRS outside initial UL BWP should be considered when defining RRM requirements in RRC_INACTIVE state. Proposal 2: Use the conditions for PRS/SRS bandwidth aggregation agreed in RAN1 as the baseline and additional capture ‘the same RF chain (same antenna)’ in RAN4. Proposal 3: Tmargin = max(Teffect, aggregate, Teffect, non-aggregate) should be added when both Taggregate and Tnon-aggregate are larger than 0. Proposal 4: When one of aggregated PRS resources is dropped due to collision, it is up to UE implementation to perform measurement based on the remaining PRS resources and RAN4 will not specify the exact value of extended measurement period. |
| R4-2319999 | Huawei, HiSilicon | Proposal 1: RAN4 to define measurement period requirements for PRS CA in BW agnostic manner. Proposal 2: Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL. Proposal 3: Requirements for aggregate measurement are applicable to PRS resources in the resource sets that are indicated to be linked, provided that the alignment conditions defined by RAN1 are met. Proposal 4: Adopt the following addition to the measurement period requirements for PRS CA. - Taggregate is defined re-using the existing requirements as baseline, and - multiple PFLs with linked resource sets are considered as one ‘effective’ PFL - new processing capability for aggregate measurements would apply - only resources that are linked are considered in Lprs - Update the total measurement period by adding an alignment margin Txxxx,total = Taggregate + Tnon-aggregate + max(Teffect,i) Proposal 5: RAN4 not to define UE behaviour when there is no PRS resource dropping on any of the aggregated PFLs. Proposal 6: TEG is considered in measurement period requirements for PRS CA. Proposal 7: RAN4 to discuss the impact of RSRP(P) measurement on the requirements for PRS CA based on further RAN1 agreements. Proposal 9: RAN4 not to define requirements for collision between SRS CA (outside initial BWP) with other UL/DL channels in INACTIVE. Proposal 10: RAN4 not to define new applicability rule for UE Rx-Tx requirements related to SRS dropping in case of SRS CA. Proposal 11: RAN4 to discuss whether to define interruption for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1. |
| R4-2320373 | Ericsson | # Tmargin for PRS bandwidth aggregation measurement period requirement Proposal 1: Tmargin = max(Teffect,i) + time until the start of measurement on non-aggregated PFLs. In the formula i denotes index of effective PFL, corresponding to the group of PFLs aggregated for PRS measurement. # Impact of PRS collision on aggregation requirement Observation 1: UE is expected to measure PRS resource on one or more of the non-colliding PFLs. Proposal 2: Legacy measurement period requirement applies for the case when UE is configured to aggregate 2 PFLs for positioning measurement in RRC_CONNECTED state and one of the PFLs is dropped due to collision with SSB. Proposal 3: For the case when UE is configured to perform positioning measurements on 3 aggregated PFLs in RRC_CONNECTED state and one of the PFLs is dropped due to collision with SSB and non-colliding PFLs are contiguous then UE shall meet measurement period requirement for positioning measurements by aggregating 2 PFLs. Proposal 4: For the case when UE is configured to perform positioning measurements on 3 aggregated PFLs in RRC_CONNECTED state and one of the PFLs is dropped due to collision with SSB and non-colliding PFLs are non-contiguous then UE determines PFL, among the non-colliding ones, to perform positioning measurements on. In this case legacy measurement period requirement applies. # Single Tx chain definition Proposal 5: Single Tx chain is defined as “common RF transmission bandwidth and the same transmit antenna reference point” in the requirement applicability section of core requirements for bandwidth aggregation for positioning measurements. |
| R4-2320810 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to specify measurement period requirements for PRS/SRS BW aggregation in BW-agnostic way based on number of aggregated PFLs and indicated PRS resource sets for bandwidth aggregation. Proposal 3: RAN4 to deprioritize measurement period and measurement performance requirements for unequal PFL bandwidth for PRS/SRS BW aggregation. Proposal 4: RAN4 to agree the following modification of clause 9.9.1.1 to TS 38.133. Proposal 5: RAN4 to agree the addition of the following new clause 7.5.X to TS 38.133. Proposal 6: RAN4 to study activation time for PRS/SRS BW aggregation for 2 PFLs and 3 PFLs, which will impact the measurement delay requirements. Proposal 7: RAN4 to study the impact to CA data throughput due to the presence of guard period before and after aggregated SRS transmission. Proposal 8: RAN4 to modify transmission requirements in TS 38.133 for the case one of the aggregated SRS carriers has a collision with other signal/channel in at least one symbol. Proposal 9: In case of PRS resource dropping due to collision with signals on one or more PFLs, which is up to UE implementation, the UE needs to indicate to LMF the number of PFLs the aggregated PRS measurement is based on. Proposal 10: RAN4 to await further RAN1 progress on the applicability of TEG reporting for PRS/SRS BW aggregation. Proposal 11: RAN4 to await further RAN1 agreement related to PRS-RSRP and PRS-RSRPP measurement type across aggregated PFLs. Proposal 12: RAN4 to base RRM requirements for RRC_INACTIVE on the scenario that aggregated carriers with SRS for positioning are located outside the initial UL BWP. Proposal 13: RAN4 to reflect the prioritization rule of SSB or other DL channel reception over SRS for positioning transmission outside the initial BWP in RRC_INACTIVE state in terms of defining scheduling restriction in a new clause 5.6.x in TS 38.133 aligned to current scheduling restriction in clause 5.5.4. Proposal 14: RAN4 to specify a suitable additional margin for switching between aggregated PFL reception to single PFL reception, e.g. 500 us. Lower values for the additional margin may be supported according to UE capability. |
| R4-2320914 | Qualcomm Incorporated | Observation 1: The LMF may configure up to two PFL groups for measurements with PRS BW aggregation. Proposal 1: Measurement requirements with PRS BW aggregation apply provided the number of PFL groups to be aggregated is not greater than 2. Proposal 2: For PRS measurement with BW aggregation within measurement gaps, define Txxxx, aggregate as follows: Txxxx, aggregate=m=1MTxxxx, agg, m where M is the number of PFL groups to be aggregated, and for each PFL group: Txxxx, agg, m=[kmultiTEG,agg, m*]CSSFPRS, agg, m*ceilKp, PRS, agg, m*NRxBeam, agg, m*NPRS,agg, mslotNagg, m'Lavailable_PRS,agg, mNagg, m*Nsample-1*Teffect, agg, m+Tlast, agg, m - Nsample=2, 4 - Nagg, m corresponds to parameter N defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs - Nagg, m' is the maximum number of PRS resources per PFL that the UE can process in one slot, as defined in FG 41-4-1 (41-4-1a, component 5) for aggregation of 2 (3) PFLs - NPRS,agg, mslot is the maximum number of DL PRS resources configured per slot in one PFL, considering all PRS resource sets to be aggregated. - Teffect, agg, m is defined as Teffect, agg, m=Tagg, mTavailable_PRS,i*Tavailable_PRS,i - Tagg, m corresponds to parameter T defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs - Tavailable_PRS,i is the available PRS periodicity as defined in Rel-17 for any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Tavailable_PRS,i. - Lavailable_PRS, agg, m is the time duration of available PRS in PFL i during Tavailable_PRS,i, where PFL i is any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Lavailable_PRS, agg. - [kmultiTEG, agg, m is the scaling factor for measurement of same PRS resource with multiple Rx TEGs] - CSSFPRS, agg, m is the carrier-specific scaling factor for PRS measurements with BW aggregation. The definition is FFS. - Kp, PRS, agg, m is a scaling factor for a positioning frequency layer to be measured within an associated measurement gap. As baseline, RAN4 will leverage the definition of Kp, PRS, i in Rel-17 to define Kp, PRS, agg, m. - NRxBeam, agg, m is the UE Rx beam sweeping factor. NRxBeam, agg, m=1 for FR1. NRxBeam, agg is based on the Rel-17 definition for FR2, including support for reduced Rx beam sweeping factor. - Tlast, agg, m is the measurement delay for the last aggregated sample. As baseline, RAN4 will leverage the definition of Tlast, i in Rel-17 to define Tlast, agg, m. Proposal 3: In the measurement period requirement for RSTD and UE Rx-Tx with PRS BW aggregation add the following margin: Tmargin=max Teffect, i Proposal 4: When PRS-RSRP(P) is reported with timing-based positioning measurements (RSTD or UE Rx-Tx) with BW aggregation, the measurement period requirement for the timing-based measurements applies. Proposal 5: No measurement requirements will be defined for stand-alone PRS-RSRP(P) reporting with PRS BW aggregation. Proposal 6: Define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1 and RAN4 RF session. Requirements for SRS carrier switching or antenna switching can be re-used as baseline. |
| R4-2320808 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to agree that aggregated PFL measurements are performed first also in case of two PFL groups. Proposal 2: The total measurement period when two PFL groups are configured is for RSTD (and similarly for UE Rx-Tx time difference): - TRSTD,total=TRSTD, aggregate, PFL group1+Tmargin+TRSTD, aggregate, PFL group2+Tmargin+ TRSTD, non-aggregate, PFL group1+Tmargin+ TRSTD, non-aggregate, PFL group2 Proposal 3: RAN4 to consider changes to collision handling for PRS and other signals/channels to include the case of two PFL groups. Proposal 4: RAN4 to send Reply LS to RAN1 asking clarification, if the number of measurement samples can be independent of the two PFL groups. |
| R4-2319990 | Huawei, HiSilicon | Proposal 1: RAN4 to take “single Rx chain” (same Rx antenna) assumption for defining accuracy requirements for PRS/SRS CA, i.e. no imperfection related to timing, frequency or phase error would be considered when deriving the requirements. Proposal 2: RAN4 to define the following side conditions for the UE RSTD and Rx-Tx measurement period and accuracy requirements. - The UE RSTD/Rx-Tx (core) and accuracy (performance) requirements apply provided that channel over which a symbol on the antenna port on one PFL for PRS transmission is conveyed can be inferred from the channel over which the same symbol on the antenna port on an aggregated PFL is conveyed Proposal 3: RAN4 to define the following side conditions for the gNB Rx-Tx measurement accuracy requirements. The gNB Rx-Tx measurement accuracy (performance) requirements apply provided that the channel over which a symbol on the antenna port on one carrier for SRS transmission is conveyed can be inferred from the channel over which the same symbol on the antenna port on an aggregated carrier for SRS transmission is conveyed |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: General aspects/scenarios for PRS/SRS BW aggregation

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1-1: Impact of aggregated bandwidth on PRS measurement period requirements

- Proposals

  - Option 1: Nokia

    - RAN4 to deprioritize measurement period and measurement performance requirements for unequal PFL bandwidth for PRS/SRS BW aggregation.

    - RAN4 to specify measurement period requirements for PRS/SRS BW aggregation in BW-agnostic way based on number of aggregated PFLs and indicated PRS resource sets for bandwidth aggregation.

  - Option 2: HW

    - RAN4 to define measurement period requirements for PRS CA in BW agnostic manner.

  - Option 3: CMCC

    - for applicable aggregated bandwidth, no need to deprioritize measurement period and measurement performance requirements for unequal PFL bandwidth for PRS/SRS BW aggregation. It is proposed to consider both  equal and unequal PFL bandwidth, and same measurement period and measurement performance requirements apply to both cases.

- Recommended WF

  - Discuss the options.

Issue 3-1-2: Applicable condition in RRC inactive

- Proposals

  - Option 1: OPPO

    - The scenario that aggregated SRS outside initial UL BWP should be considered when defining RRM requirements in RRC INACTIVE state.

  - Option 2: Nokia

    - RAN4 to base RRM requirements for RRC_INACTIVE on the scenario that aggregated carriers with SRS for positioning are located outside the initial UL BWP.

- Recommended WF

  - Discuss the options.

### Sub-topic 3-2: PRS measurement requirements for PRS/SRS bandwidth aggregation

Sub-topic description

Open issues and candidate options before meeting:

Following as agreed in the WF in R4-2317386 in RAN4#108bis:

- Condition of single TX chain (same antenna) shall be captured at least in the core part of TS 38.133 for UE PRS measurements:

  - The UE PRS measurement requirements with PRS aggregation applies provided that all the PFLs to be aggregated by the UE are transmitted by the gNB using single Tx chain and the same transmit antenna reference point. Meaning of single Tx chain is FFS.

Issue 3.2.1: How to capture the condition of single RF chain (same antenna) in RAN4 specifications

- Proposals

  - Option 1: HW

    - RAN4 to take “single Rx chain” (same Rx antenna) assumption for defining accuracy requirements for PRS/SRS CA, i.e. no imperfection related to timing, frequency or phase error would be considered when deriving the requirements.

    - RAN4 to define the following side conditions for the UE RSTD and Rx-Tx measurement period and accuracy requirements.

      - The UE RSTD/Rx-Tx (core) and accuracy (performance) requirements apply provided that channel over which a symbol on the antenna port on one PFL for PRS transmission is conveyed can be inferred from the channel over which the same symbol on the antenna port on an aggregated PFL is conveyed

    - RAN4 to define the following side conditions for the gNB Rx-Tx measurement accuracy requirements.

      - The gNB Rx-Tx measurement accuracy (performance) requirements apply provided that the channel over which a symbol on the antenna port on one carrier for SRS transmission is conveyed can be inferred from the channel over which the same symbol on the antenna port on an aggregated carrier for SRS transmission is conveyed

  - Option 2: E///

    - Single Tx chain is defined as “common RF transmission bandwidth and the same transmit antenna reference point” in the requirement applicability section of core requirements for bandwidth aggregation for positioning measurements.

  - Option 3: Nokia

    - RAN4 to agree the following modification of clause 9.9.1.1 to TS 38.133:

When UE supports [PRS bandwidth aggregation capability] and is configured with [PRS bandwidth aggregation configuration], requirements are based on common numerology across all configured intra-band contiguous PFLs to be aggregated, whereby

-	PRS resources to be aggregated from different PFLs may have equal or different number of PRS RBs.

-	PRS resources to be aggregated from different PFLs are transmitted in the same slot and in the same symbols.

-	PRS resources to be aggregated from different PFLs are transmitted by the same TRP and associated with a common Antenna Reference Point (ARP), as specified in TS 38.104.

- 	PRS resources to be aggregated from different PFLs are received with single RF chain and the same Antenna Reference Point (ARP), as specified in TS 38.101-1 for FR1 and TS 38.101-2 for FR2.

    - RAN4 to agree the addition of the following new clause 7.5.X to TS 38.133:

7.5.X	Minimum Requirements for SRS Bandwidth Aggregation

When UE supports [SRS bandwidth aggregation capability] and is configured with [SRS bandwidth aggregation configuration], relative transmission timing difference between slot timing of all pairs of configured intra-band contiguous carriers to be aggregated are based on common numerology across all intra-band contiguous carriers, whereby

-	SRS resources to be aggregated from different carriers may have equal or different number of SRS RBs.

-	SRS resources to be aggregated from different carriers are transmitted in the same slot and in the same symbols.

-	SRS resources to be aggregated from different carriers are transmitted by single RF chain and associated with a common Antenna Reference Point (ARP), as specified in TS 38.101-1 for FR1 and TS 38.101-2 for FR2.

  - Option 4: CATT

    - RAN4 not to capture the condition of single RF chain (same antenna) in RAN4 specs which should be a prerequisite for BW aggregation.

- Recommended WF

  - Discuss the options.

  - Send LS reply to RAN1 LS (R1-2308449/R4-2315004) capturing agreement, if considered necessary.

Issue 3-2-2: Applicable conditions for meeting PRS/SRS bandwidth aggregation requirements

- Proposals

  - Option 1: OPPO

    - Use the conditions for PRS/SRS bandwidth aggregation agreed in RAN1 as the baseline and additional capture ‘the same RF chain (same antenna)’ in RAN4.

  - Option 2: CATT

    - Requirements for PRS bandwidth aggregation are applicable provided that the linked PRS resource sets are from the same TRP. Option 2 is also supported:

      - Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL.

      - Requirements for aggregate measurement are applicable to PRS resources in the resource sets that are indicated to be linked, provided that the alignment conditions defined by RAN1 are met.

    - The bandwidth aggregation requirements are applicable provided that no PRS/SRS resources dropped.

  - Option 3: HW

    - Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL.

    - Requirements for aggregate measurement are applicable to PRS resources in the resource sets that are indicated to be linked, provided that the alignment conditions defined by RAN1 are met.

  - Option 4: QC

    - Measurement requirements with PRS BW aggregation apply provided the number of PFL groups to be aggregated is not greater than 2.

- Recommended WF

  - Discuss the options.

Issue 3-2-3: Measurement period requirement for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: CATT

    - The existing measurement period requirements can be reused for non-aggregated measurements, while it can be baseline for aggregated measurements with some modifications, e.g., new UE processing capabilities in FG 41-4-1/1a. Details can be discussed in draft CR.

    - Tmargin=max(Teffect,i) is added to align non-aggregated and aggregated measurements, where Teffect,i is the maximum periodicity of measurement in non-aggregated/aggregated PFLs and the definition is the same as that in the existing measurement period requirements.

  - Option 2: Xiaomi

    - The total measurement period for RSTD and UE Rx-Tx requirements with bandwidth aggregation across all PFLs is defined as

Txxxx,total = Tagg + Tnon-agg         (1)

Wherein,

T agg=m=1MTagg, m    (2)

where M is the number of PFL groups to be aggregated, and for each PFL group:

Tagg, m=kmultiTEG_agg, m*CSSFPRS_agg, m*ceilKp, PRS_agg, m*NRxBeam_agg, m*NPRS_agg,  mslotNagg, m'Lavailable_PRS_agg, mNagg, m*Nsample-1*Teffect_agg, m+ Tlast_agg, m  (3)

Tlast_agg, m = max(Tlast_agg, i)   (4)

Wherein,  i is the index of PFL layers within a PFL aggregated group.

Tnon-agg=m=1MTnon-agg, m  (5)

m is the number of PFL to be measured separately.

T non-agg, m=kmultiTEG,non-agg, m*CSSFPRS_non-agg, m*ceilKp, PRS_non-agg, m*NRxBeam_non-agg, m*NPRSnon-agg,  mslotNnon-agg, m'LavailablePRSnon-agg, mNnon-agg, m*Nsample-1*Teffect_non-agg, m  +Tlast_non-agg, m  (6)

  - Option 2: QC

    - For PRS measurement with BW aggregation within measurement gaps, define Txxxx, aggregate as follows:

Txxxx, aggregate=m=1MTxxxx, agg, m

where M is the number of PFL groups to be aggregated, and for each PFL group:

Txxxx, agg, m=kmultiTEG,agg, m*CSSFPRS, agg, m*ceilKp, PRS, agg, m*NRxBeam, agg, m*NPRS,agg,  mslotNagg, m'Lavailable_PRS,agg, mNagg, m*Nsample-1*Teffect, agg, m+Tlast, agg, m

- Nsample=2, 4

- Nagg, m corresponds to parameter N defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- Nagg, m' is the maximum number of PRS resources per PFL that the UE can process in one slot, as defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- NPRS,agg, mslot is the maximum number of DL PRS resources configured per slot in one PFL, considering all PRS resource sets to be aggregated.

- Teffect, agg, m is defined as
Teffect, agg, m=Tagg, mTavailable_PRS,i*Tavailable_PRS,i

- Tagg, m corresponds to parameter T defined in FG 41-4-1 (41-4-1a) for aggregation of 2 (3) PFLs

- Tavailable_PRS,i is the available PRS periodicity as defined in Rel-17 for any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Tavailable_PRS,i.

- Lavailable_PRS, agg, m is the time duration of available PRS in PFL i during Tavailable_PRS,i, where PFL i is any one of the PFLs to be aggregated. Only PRS resources that satisfy the conditions for PRS BW aggregation are considered when calculating Lavailable_PRS, agg.

- [kmultiTEG, agg, m  is the scaling factor for measurement of same PRS resource with multiple Rx TEGs]

- CSSFPRS, agg, m is the carrier-specific scaling factor for PRS measurements with BW aggregation. The definition is FFS.

- Kp, PRS, agg, m is a scaling factor for a positioning frequency layer to be measured within an associated measurement gap. As baseline, RAN4 will leverage the definition of Kp, PRS, i in Rel-17 to define Kp, PRS, agg, m.

- NRxBeam, agg, m is the UE Rx beam sweeping factor. NRxBeam, agg, m=1 for FR1. NRxBeam, agg is based on the Rel-17 definition for FR2, including support for reduced Rx beam sweeping factor.

- Tlast, agg, m is the measurement delay for the last aggregated sample. As baseline, RAN4 will leverage the definition of Tlast, i  in Rel-17 to define Tlast, agg, m.

    - In the measurement period requirement for RSTD and UE Rx-Tx with PRS BW aggregation add the following margin: Tmargin=max Teffect, i

  - Option 3: HW

      - Adopt the following addition to the measurement period requirements for PRS CA.

- Taggregate is defined re-using the existing requirements as baseline, and

  - multiple PFLs with linked resource sets are considered as one ‘effective’ PFL

  - new processing capability for aggregate measurements would apply

  - only resources that are linked are considered in Lprs

- Update the total measurement period by adding an alignment margin

Txxxx,total = Taggregate + Tnon-aggregate + max(Teffect,i)

  - Option 5: E///

    - Tmargin = max(Teffect,i) + time until the start of measurement on non-aggregated PFLs. In the formula i denotes index of effective PFL, corresponding to the group of PFLs aggregated for PRS measurement.

  - Option 6: Nokia

    - RAN4 to specify measurement period requirements for PRS/SRS BW aggregation in BW-agnostic way based on number of aggregated PFLs and indicated PRS resource sets for bandwidth aggregation.

    - RAN4 to agree that aggregated PFL measurements are performed first also in case of two PFL groups.

    - The total measurement period when two PFL groups are configured is for RSTD (and similarly for UE Rx-Tx time difference):

- TRSTD,total=TRSTD, aggregate, PFL group1+Tmargin+TRSTD, aggregate, PFL group2+Tmargin+ TRSTD, non-aggregate, PFL group1+Tmargin+ TRSTD, non-aggregate, PFL group2

- RAN4 to specify a suitable additional margin for switching between aggregated PFL reception to single PFL reception, e.g. 500 us. Lower values for the additional margin may be supported according to UE capability

  - Option 7: OPPO

    - Tmargin = max(Teffect, aggregate, Teffect, non-aggregate) should be added when both Taggregate and Tnon-aggregate are larger than 0.

- Recommended WF

  - Discuss the options.

Issue 3-2-4: Impact of PRS collision with other signals on PRS bandwidth aggregation requirement

- Proposals

  - Option 1: CATT

    - RAN4 not to define UE behaviour which shall depend on priority rules or UE implementation when collisions occur.

  - Option 2: Nokia

    - In case of PRS resource dropping due to collision with signals on one or more PFLs, which is up to UE implementation, the UE needs to indicate to LMF the number of PFLs the aggregated PRS measurement is based on.

    - RAN4 to consider changes to collision handling for PRS and other signals/channels to include the case of two PFL groups.

  - Option 3: OPPO

    - When one of aggregated PRS resources is dropped due to collision, it is up to UE implementation to perform measurement based on the remaining PRS resources and RAN4 will not specify the exact value of extended measurement period.

  - Option 4: E///

    - Legacy measurement period requirement applies for the case when UE is configured to aggregate 2 PFLs for positioning measurement and one of the PFLs collide with other high priority DL signal.

    - For the case when UE is configured to perform positioning measurements on 3 aggregated PFLs in RRC_CONNECTED state and one of the PFLs is dropped due to collision with SSB and non-colliding PFLs are contiguous then UE shall meet measurement period requirement for positioning measurements by aggregating 2 PFLs.

    - For the case when UE is configured to perform positioning measurements on 3 aggregated PFLs in RRC_CONNECTED state and one of the PFLs is dropped due to collision with SSB and non-colliding PFLs are non-contiguous then UE determines PFL, among the non-colliding ones, to perform positioning measurements on. In this case legacy measurement period requirement applies.

  - Option 5: Xiaomi

    - When the PRS collision with other signals on PRS bandwidth aggregation requirement, UE’s measurement can rely on the PRS of the PFL which is not collided.

  - Option 6: HW

    - RAN4 not to define UE behaviour when there is no PRS resource dropping on any of the aggregated PFLs.

- Recommended WF

  - Discuss the options.

Issue 3-2-5: Impact of SRS collision with other signals on SRS bandwidth aggregation requirement

- Proposals

  - Option 1a: Xiaomi

    - RAN4 can discuss the impacts due to SRS bandwidth aggregation for UE Rx-Tx time difference requirements firstly.

    - The requirements of UE Rx-Tx time difference measurements with SRS aggregation is applicable only when there is no any dropped aggregated SRSs.

  - Option 1b: HW

    - RAN4 not to define new applicability rule for UE Rx-Tx requirements related to SRS dropping in case of SRS CA.

  - Option 2: Nokia

    - RAN4 to modify transmission requirements in TS 38.133 for the case one of the aggregated SRS carriers has a collision with other signal/channel in at least one symbol.

- Recommended WF

  - Discuss the option.

Issue 3-2-6: Impact of collision of other signals on SRS bandwidth aggregation requirement in RRC_INACTIVE

- Proposals

  - Option 1: HW

    - RAN4 not to define requirements for collision between SRS CA (outside initial BWP) with other UL/DL channels in INACTIVE.

  - Option 2: Nokia

    - RAN4 to reflect the prioritization rule of SSB or other DL channel reception over SRS for positioning transmission outside the initial BWP in RRC_INACTIVE state in terms of defining scheduling restriction in a new clause 5.6.x in TS 38.133 aligned to current scheduling restriction in clause 5.5.4.

- Recommended WF

  - Discuss the options.

Issue 3-2-7: TEG for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: HW

    - TEG is considered in measurement period requirements for PRS CA.

  - Option 2: Nokia

    - RAN4 to await further RAN1 progress on the applicability of TEG reporting for PRS/SRS BW aggregation.

- Recommended WF

  - Discuss the options.

Issue 3-2-8: Activation time for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: Nokia

    - RAN4 to study activation time for PRS/SRS BW aggregation for 2 PFLs and 3 PFLs, which will impact the measurement delay requirements.

- Recommended WF

  - Discuss the option.

Issue 3-2-9: Impact of guard period on CA throughput

- Proposals

  - Option 1: Nokia

    - RAN4 to study the impact to CA data throughput due to the presence of guard period before and after aggregated SRS transmission.

- Recommended WF

  - Discuss the option.

Issue 3-2-10: Interruption due to SRS transmission for BW aggregation

- Proposals

  - Option 1: CATT

    - Interruptions at NR SRS carrier based switching defined in TS 38.133 clause 8.2.2.2.9 can be reused for the interruption due to SRS bandwidth aggregation. Detailed interruption time can be further determined based on RF session conclusion.

  - Option 2: Xiaomi, QC

    - RAN4 to define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1 and RF session. Requirements for SRS carrier switching or antenna switching can be re-used as baseline.

  - Option 3: HW

    - RAN4 to discuss whether to define interruption for SRS transmission for BW aggregation on CC without PUSCH/PUCCH based on conclusions from RAN1.

  - Option 3: LG

    - RAN4 to define interruption requirements or scheduling restriction for SRS transmission for BW aggregation on UL communication CC.

- Recommended WF

  - Discuss the options.

Issue 3-2-11: PRS aggregation for PRS-RSRP and PRS-RSRPP

- Proposals

  - Option 1: HW, Nokia

    - Option 1A: HW

      - RAN4 to discuss the impact of RSRP(P) measurement on the requirements for PRS CA based on further RAN1 agreements.

    - Option 1B: Nokia

      - RAN4 to wait on further RAN1 agreement related to PRS-RSRP and PRS-RSRPP measurement type across aggregated PFLs.

  - Option 2: Qualcomm

      - When PRS-RSRP(P) is reported with timing-based positioning measurements (RSTD or UE Rx-Tx) with BW aggregation, the measurement period requirement for the timing-based measurements applies.

      - No measurement requirements will be defined for stand-along PRS-RSRP(P) reporting with PRS BW aggregation.

- Recommended WF

  - Discuss the options.

# Topic #4: RRM performance requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2319097 | CMCC | Proposal 1: For SL-PRS, it is proposed to define test cases for - SL-PRS based AoA/ZOA - SL-PRS based RTOA - SL-PRS based RSTD - SL-PRS based UE Rx-Tx time difference - SL-PRS based RSRP - SL-PRS based RSRPP Proposal 2: for LPHAP, it is proposed to define test cases for both normal (non-RedCap) and RedCap type of devices. Proposal 3: For LPHAP, it is proposed to define test cases fo following cases: - When RAN eDRX <= 10.24s - CN eDRX <= 10.24 s - CN eDRX >10.24 s - When RAN eDRX > 10.24s Proposal 4: for LPHAP, it is proposed to define test cases for eDRX with following enhancements: - Reduced number of PRS samples - parallelPRS-MeasRRC-Inactive-r17 capability - Lower Rx beam sweeping in FR2 |
| R4-2320002 | Huawei, HiSilicon | Proposal 1: RAN4 to define accuracy requirements for the following SL PRS measurements: SL RSTD, SL Rx-Tx, SL PRS-RSRP, SL PRS-RSRPP, SL AoA/ZoA. RAN4 not to define accuracy requirements for SL RTOA. Proposal 2: Accuracy requirements for SL PRS measurements are defined for following RB numbers - 15kHz SCS: 48, 96 - 30kHz SCS: 24, 48, 96 - 60kHz SCS: 24, 48 Proposal 3: Accuracy requirements for SL PRS measurements are defined based on - Es/Iot side condition of (0, -3, -3)dB or (0, 0, 0)dB - Same channel as their Uu counterparts, FFS for SL AoA/ZoA - Nsample = 1 Proposal 4: Existing accuracy requirements are applicable for PRS measurement in INACTIVE with eDRX, and PRS measurement in IDLE. Proposal 5: For RedCap with 2RX without FH, existing accuracy requirements are reused except for the applicable BW. Proposal 6: For RedCap with 1RX without FH, new accuracy requirements are defined based on the agreed Es/Iot side condition and related simulation results. Proposal 7: For RedCap with FH, new accuracy requirements are defined based same channel, Es/Iot side condition and sample number as requirements for non-FH. Proposal 8: For RedCap with FH, RAN4 to discuss the per hop BW and total measurement BW across all hops for defining accuracy requirements. Proposal 9: For PRS CA, RAN4 to define RSTD and UE Rx-Tx accuracy requirements for 2-PFL and 3-PFL, based same channel, Es/Iot side condition and sample number as requirements for single PFL. Proposal 10: For PRS CA, RAN4 to use the following per PFL BW as starting point to define accuracy requirements for RSTD and UE Rx-Tx. - 15kHz SCS: 104 (20M) - 30kHz SCS: 132 (50M), 272 (100M) - 60kHz SCS FR1: 64 (50M), 132 (100M) - 60kHz SCS FR2: 64 (50M), 132 (100M) - 120kHz SCS: 64 (100M), 128 (200M) Proposal 11: RAN4 to define accuracy requirements for DL RSCPD and relative accuracy for DL RSCP. RAN4 not to define absolute accuracy requirements for DL RSCP. Proposal 12: RAN4 not to define accuracy requirements for UL RSCP. Proposal 13: Accuracy requirements for CPP measurements are defined using same RB numbers as used in existing RSTD and UE Rx-Tx accuracy requirements. |
| R4-2320375 | Ericsson | Proposal 1: Do not define accuracy requirement specific to LPHAP. For LPHAP, rel. 17 accuracy requirements apply. Proposal 2: RAN4 to align the targeted PRS BW range for NR DL CPP to that for legacy measurement (RSTD/UE Rx-Tx). Proposal 3: RSCPD and RSCP accuracy requirements are only defined for Nsample = 1 under AWGN and Two-tap propagation conditions. Proposal 4: RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-6, -10, -10) dB. Proposal 5: UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB. Proposal 6: PRS-RSRP accuracy requirement for 1Rx RedCap UE is defined for SINR values (-3, -10, -10) dB. Proposal 7: PRS-RSRPP accuracy requirement for 1Rx RedCap UE under two path propagation condition is defined for SINR values (-3, -10, -10) dB. Proposal 8: Rel. 17 accuracy requirement for PRS-RSRP measurement is reused to define PRS-RSRP accuracy requirement for RedCap positioning with FH. Proposal 9: Prioritize accuracy requirement for 2 PFL case over 3 PFL case while defining accuracy requirement for positioning measurements based on bandwidth aggregation. |
| R4-2320913 | Qualcomm Incorporated | Proposal 1: The minimum PRS BW expected to be measured is given by minBWPRS,Nhops∙BWhop-BWoverlap∙Nhops-1 where - BWPRS is the configured PRS BW - BWhop is the BW per hop signaled in the UE capability - BWoverlap is the minimum hop overlap signaled in the UE capability - Nhops=Nhopsslot∙NrepPRS if MrepPRS=1 or Nhopsslot≥1, otherwise Nhops=1+NrepPRS-11MrepPRS∙Nhopsslot - NrepPRS is the number of PRS inter-slot repetitions within a single MG instance, excluding the gap retuning times - MrepPRS is the stride of PRS inter-slot repetitions Nhopsslot is the number of frequency hops per slot as a function of number of PRS symbols, PRS comb size, and retuning time between hops. FFS how to determine Nhopsslot from these parameters and whether to limit its maximum value. Proposal 2: - For RSTD accuracy requirements for 1Rx RedCap UE under fading propagation condition, the reference and target SINR side-conditions are SINRref = -6 dB, SINRtarget = -10 dB. - For UE Rx-Tx accuracy requirements for 1Rx RedCap UE under fading propagation condition, the SINR side-conditions are (-3, -10) dB. Proposal 3: RAN4 to define a single set of PRS-RSRP accuracy requirements for 1Rx RedCap UEs under both AWGN and fading conditions, at SINR side-conditions (-3, -10) dB. Proposal 4: RAN4 to define PRS-RSRPP accuracy requirements for 1Rx RedCap UEs under the 2-tap channel model used in Rel-17, at SINR side-conditions (-3, -10) dB. |
| R4-2318278 | CATT | Proposal 2: The brackets in the side condition for 1Rx without FH under fading propagation condition can be removed, i.e., the side conditions should be: • RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-6, -10, -10) dB. • UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB. • PRS-RSRP accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB. • PRS-RSRPP accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB. Proposal 7: The effective bandwidth for PRS measurements for RedCap with FH can be derived by the equation: BWeffective=min(BWPRS,12*15*2μ*NRBhop-NRBoverlap*Nhop+NRBoverlap), where, - BWPRS is the configured BW for PRS. - NRB_hop is the number of RBs per hop. - Nhop is number of hops. - NRB_overlap is the number of overlapping RBs between hops. |
| R4-2319999 | Huawei, HiSilicon | Proposal 8: RAN4 to approve the simulation assumption in the Annex for PRS CA. |
| R4-2320914 | Qualcomm Incorporated | Proposal 7: Simulate performance in AWGN only. Proposal 8: Simulate only RSTD and UE Rx-Tx measurements with PRS BW aggregation. Proposal 9: For PRS BW aggregation across 2 PFLs simulate the following configurations Parameter; Value. SCS, RB num, Repetition; SCS (kHz); RB num per PFL; Repetition (Note); Sample rate (Tc). 15; 104; 1; 32. 30; 48; 1; 32. 132; 1; 16. 60, FR1; 64; 1; 16. 132; 1; 8. 60, FR2; 64; 1; 16. 132; 1; 8. 120; 64; 1; 8. 128; 1; 4. PRS comb size; 4. PRS symbol size; 4. PRS periodicity; 40ms. Sample number; 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB. TOA estimation; Realistic. Path #; First path. Proposal 10: For PRS BW aggregation across 3 PFLs simulate the following configurations Parameter; Value. SCS, RB num, Repetition; SCS (kHz); RB num per PFL; Repetition (Note); Sample rate (Tc). 15; 104; 1; 16. 30; 48; 1; 16. 132; 1; 8. 60, FR1; 64; 1; 8. 132; 1; 4. 60, FR2; 64; 1; 8. 132; 1; 4. 120; 64; 1; 4. 128; 1; 2. PRS comb size; 4. PRS symbol size; 4. PRS periodicity; 40ms. Sample number; 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB. TOA estimation; Realistic. Path #; First path. Proposal 11: Channel spacing between PFLs not to exceed the nominal channel spacing for CA defined in 38.101-1, clause 5.4A.1 for FR1 and in 38.101-2, clause 5.4A.1 for FR2. FFS the exact channel spacing values for each PRS configuration. |
| R4-2320855 | Nokia, Nokia Shanghai Bell | Proposal 3: RAN4 to define performance requirements for 4 measurement samples as baseline and optionally consider the reduced number of samples Nsample = 2. - Proposal 7: RAN4 to define the BW with FH as follows: minBWPRS, NhopBWhop-Nhop-1BWoverlap, where - BWPRS is the configured PRS BW, - Nhop is the number of hops within a single MG instance, - BWhop is the BW per hop depending on UE capabilities, and BWoverlap is the BW of all overlapped RBs. |
| R4-2319307 | LG Electronics Inc. | Proposal 1: RAN4 to investigate as baseline for measurement performance requirements a single carrier bandwidth including maximum bandwidth case. (Support - Option 1) |
| R4-2319996 | Huawei, HiSilicon | Proposal 6: For Case 1, RAN4 to define the overall BW with FH BWmulti-hop=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap where - BWPRS is the minimum of the configured PRS BW, UE capability of maximum PRS BW across all hops and total BW of all hops requested by LMF - Nhop is number of hops within a single MG occasion - BWper-hop is the supported BW per hop which is UE capability - BWoverlap is the BW of the overlapping RB |
| R4-2320810 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to specify measurement period requirements for PRS/SRS BW aggregation in BW-agnostic way based on number of aggregated PFLs and indicated PRS resource sets for bandwidth aggregation. Proposal 2: RAN4 to investigate as baseline for measurement performance requirements a single carrier bandwidth of 10 MHz, 20 MHz and 50 MHz for PRS/SRS BW aggregation in FR1 and 50 MHz, 100 MHz, 200 MHz for PRS/SRS BW aggregation in FR2. Proposal 3: RAN4 to deprioritize measurement period and measurement performance requirements for unequal PFL bandwidth for PRS/SRS BW aggregation. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: RRM performance requirements for RedCap

Sub-topic description:

Open issues and candidate options before meeting:

Issue 4-1-1: PRS measurement accuracy for 2Rx RedCap UE without FH

- Proposals

  - Option 1: HW

    - For RedCap with 2RX without FH, existing accuracy requirements are reused except for the applicable BW.

- Recommended WF

  - Discuss the options.

Issue 4-1-2: PRS measurement accuracy for 1Rx RedCap UE without FH

In RAN4#108, following was agreed for 1Rx RedCap without FH in the WF in R4-2314353:

- RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-6, [-10], [-10]) dB.

- UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, [-10], [-10]) dB.

- PRS-RSRP accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, [-10], [-10]) dB.

- PRS-RSRPP accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, [-10], [-10]) dB.

- Proposals

  - Option 1: HW

    - For RedCap with 1RX without FH, new accuracy requirements are defined based on the agreed Es/Iot side condition and related simulation results.

  - Option 2: E///, QC, CATT

    - Option 2A: E///, CATT

      - RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-6, -10, -10) dB.

      - UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values (-3, -10, -10) dB.

      - PRS-RSRP accuracy requirement for 1Rx RedCap UE is defined for SINR values (-3, -10, -10) dB.

      - PRS-RSRPP accuracy requirement for 1Rx RedCap UE under two path propagation condition is defined for SINR values (-3, -10, -10) dB.

    - Option 2B: QC

      - For RSTD accuracy requirements for 1Rx RedCap UE under fading propagation condition, the reference and target SINR side-conditions are SINRref = -6 dB, SINRtarget = -10 dB.

      - For UE Rx-Tx accuracy requirements for 1Rx RedCap UE under fading propagation condition, the SINR side-conditions are (-3, -10) dB.

      - RAN4 to define a single set of PRS-RSRP accuracy requirements for 1Rx RedCap UEs under both AWGN and fading conditions, at SINR side-conditions (-3, -10) dB.

      - RAN4 to define PRS-RSRPP accuracy requirements for 1Rx RedCap UEs under the 2-tap channel model used in Rel-17, at SINR side-conditions (-3, -10) dB.

- Recommended WF

  - Discuss the options.

Issue 4-1-3: PRS measurement accuracy for RedCap UE with FH

- Proposals

  - Option 1: HW

    - For RedCap with FH, new accuracy requirements are defined based same channel, Es/Iot side condition and sample number as requirements for non-FH.

  - Option 2: E///

    - Rel. 17 accuracy requirement for PRS-RSRP measurement is reused to define PRS-RSRP accuracy requirement for RedCap positioning with FH.

  - Option 3: Nokia

    - RAN4 to define performance requirements for 4 measurement samples as baseline and optionally consider the reduced number of samples Nsample = 2.

- Recommended WF

  - Discuss the options.

Issue 4-1-4: Applicable BW for defining PRS measurement accuracy for RedCap UE with FH

- Proposals

  - Option 1: CATT

    - The effective bandwidth for PRS measurements for RedCap with FH can be derived by the equation: BWeffective=min(BWPRS,12*15*2μ*NRBhop-NRBoverlap*Nhop+NRBoverlap), where,

      - BWPRS is the configured BW for PRS.

      - NRB_hop is the number of RBs per hop.

      - Nhop is number of hops.

      - NRB_overlap is the number of overlapping RBs between hops.

  - Option 2: HW

    - For RedCap with FH, RAN4 to discuss the per hop BW and total measurement BW across all hops for defining accuracy requirements.

  - Option 3: HW, Nokia

    - For Case 1, RAN4 to define the overall BW with FH

BWmulti-hop=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

- BWPRS is the configured PRS BW and subject to UE capability

- Nhop is number of hops within a single MG occasion

- BWper-hop is the supported BW per hop which is UE capability

- BWoverlap is the BW of the overlapping RB

  - Option 4: QC

    - The minimum PRS BW expected to be measured is given by

minBWPRS,Nhops∙BWhop-BWoverlap∙Nhops-1

where

- BWPRS is the configured PRS BW

- BWhop is the BW per hop signaled in the UE capability

- BWoverlap is the minimum hop overlap signaled in the UE capability

- Nhops=Nhopsslot∙NrepPRS if MrepPRS=1 or Nhopsslot≥1, otherwise Nhops=1+NrepPRS-11MrepPRS∙Nhopsslot

- NrepPRS is the number of PRS inter-slot repetitions within a single MG instance, excluding the gap retuning times

- MrepPRS is the stride of PRS inter-slot repetitions

- Nhopsslot is the number of frequency hops per slot as a function of number of PRS symbols, PRS comb size, and retuning time between hops. FFS how to determine Nhopsslot from these parameters and whether to limit its maximum value.

- Recommended WF

  - Discuss the option.

### Sub-topic 4-2: RRM performance requirements for PRS/SRS BW aggregation

Sub-topic description:

Open issues and candidate options before meeting:

Issue 4-2-1: Applicable aggregated bandwidth for PRS measurement accuracy for PRS/SRS BW aggregation

- Proposals

  - Option 1: LG

    - RAN4 to investigate as baseline for measurement performance requirements a single carrier bandwidth including maximum bandwidth case

  - Option 2: Nokia

    - RAN4 to investigate as baseline for measurement performance requirements a single carrier bandwidth of 10 MHz, 20 MHz and 50 MHz for PRS/SRS BW aggregation in FR1 and 50 MHz, 100 MHz, 200 MHz for PRS/SRS BW aggregation in FR2.

- Recommended WF

  - Discuss the options.

Issue 4-2-2: Applicable number of PFLs for defining PRS measurement accuracy for PRS/SRS BW aggregation.

- Proposals

  - Option 1: HW

    - For PRS CA, RAN4 to define RSTD and UE Rx-Tx accuracy requirements for 2-PFL and 3-PFL, based same channel, Es/Iot side condition and sample number as requirements for single PFL.

  - Option 2: E///

    - Prioritize accuracy requirement for 2 PFL case over 3 PFL case while defining accuracy requirement for positioning measurements based on bandwidth aggregation.

- Recommended WF

  - Discuss the options.

Issue 4-2-3: Applicable conditions for defining PRS measurement accuracy for PRS/SRS BW aggregation.

- Proposals

  - Option 1: HW

    - For PRS CA, RAN4 to define RSTD and UE Rx-Tx accuracy requirements for 2-PFL and 3-PFL, based same channel, Es/Iot side condition and sample number as requirements for single PFL.

- Recommended WF

  - Discuss the option.

Issue 4-2-4: Simulations for deriving PRS measurement accuracy for PRS/SRS BW aggregation.

- Proposals

  - Option 1: QC

    - Simulate performance in AWGN only.

    - Simulate only RSTD and UE Rx-Tx measurements with PRS BW aggregation.

    - For PRS BW aggregation across 2 PFLs simulate the following configurations

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per PFL | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 32 |
|  | 30 | 48 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR1 | 64 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 60, FR2 | 64 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 120 | 64 | 1 | 8 |
|  |  | 128 | 1 | 4 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

- For PRS BW aggregation across 3 PFLs simulate the following configurations

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per PFL | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 16 |
|  | 30 | 48 | 1 | 16 |
|  |  | 132 | 1 | 8 |
|  | 60, FR1 | 64 | 1 | 8 |
|  |  | 132 | 1 | 4 |
|  | 60, FR2 | 64 | 1 | 8 |
|  |  | 132 | 1 | 4 |
|  | 120 | 64 | 1 | 4 |
|  |  | 128 | 1 | 2 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| Sample number | 2 for Es/Iot of (-3, -6, -6)dB 4 for Es/Iot of (-6, -13, -13)dB |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

- Channel spacing between PFLs not to exceed the nominal channel spacing for CA defined in 38.101-1, clause 5.4A.1 for FR1 and in 38.101-2, clause 5.4A.1 for FR2. FFS the exact channel spacing values for each PRS configuration.

- Option 2: HW

  - Approve simulation assumptions below:

Table 1. General parameters.

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 TRPs at distinct locations: <cell 1, cell 2, cell 3>, where TRP 1 is the reference TRP |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TDD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz, 60 kHz - FDD, TDD | - 40 GHz - 60 kHz, 120 kHz - TDD |
| Propagation conditions | AWGN TDL-A (30 ns delay spread, 5Hz) | AWGN TDL-C (60 ns delay spread, 300Hz) |
| Es/Iot for three TRPs (cell 1, cell 2, cell 3), [dB] | Same as Rel-17 side conditions for Nsample = 4 and Nsample = 1. |  |
| Number of samples ( Nsample) | 4 and 1 |  |
| Number of UE receive antennas | 2 |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb 2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 1 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| Number of PFLs NOTE 2 | 2 and 3 |  |
| Carrier BW | Smallest channel BW as defined in 38.101 that is larger than the PFL BW |  |
| Carrier spacing | Nominal carrier spacing for intra-band contiguous CA as defined in 38.101 |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: Each PFL belongs to a carrier, and the 2 or 3 carriers to which the PFLs belong to are intra-band contiguous. |  |  |

Table 2: PRS transmission configuration parameters for each PFL

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 64 |
|  | 30 | 132 | 1 | 32 |
|  |  | 272 | 1 | 16 |
|  | 60, FR1 | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1, 4 |  |  |  |
| PRS periodicity | 40ms, 200ms |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

Performance metric

At least the following performance characteristics are to be provided for PRS RSTD:

- RSTD error CDFs for the 2 neighbour cells

- 5%-ile and 95%-ile of the RSTD errors

At least the following performance characteristics are to be provided for UE Rx-Tx:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge).

- UE Rx-Tx error = abs (estimated TUE-RX – ideal TUE-RX) (based on perfect channel and UE location knowledge)

  - Option 3: LG

- RAN4 to investigate as baseline for measurement performance requirements a single carrier bandwidth including maximum bandwidth case. (Support - Option 1)

- Recommended WF

  - Discuss the options.

### Sub-topic 4-3: RRM performance requirements for CPP measurement

Sub-topic description:

Open issues and candidate options before meeting:

Issue 4-3-1: Type of measurement accuracy for DL CPP measurement

- Proposals

  - Option 1: HW

    - RAN4 to define accuracy requirements for DL RSCPD and relative accuracy for DL RSCP. RAN4 not to define absolute accuracy requirements for DL RSCP.

- Recommended WF

  - Discuss the option.

Issue 4-3-2: Applicable number of RBs for defining DL CPP measurement accuracy requirement.

- Proposals

  - Option 1: HW, E///

    - Accuracy requirements for CPP measurements are defined using same RB numbers as used in existing RSTD and UE Rx-Tx accuracy requirements.

- Recommended WF

  - Discuss the option.

Issue 4-3-3: Nsample for defining DL CPP measurement accuracy requirement.

- Proposals

  - Option 1: E///

    - RSCPD and RSCP accuracy requirements are only defined for Nsample = 1 under AWGN and Two-tap propagation conditions.

- Recommended WF

  - Discuss the option.

Issue 4-3-4: UL CPP measurement accuracy requirement.

- Proposals

  - Option 1: HW

    - RAN4 not to define accuracy requirements for UL RSCP.

- Recommended WF

  - Discuss the option.

### Sub-topic 4-4: RRM performance for SL positioning measurements

Sub-topic description:

Open issues and candidate options before meeting:

Issue 4-4-1: SL PRS measurements for defining accuracy requirement.

- Proposals

  - Option 1: HW

    - RAN4 to define accuracy requirements for the following SL PRS measurements: SL RSTD, SL Rx-Tx, SL PRS-RSRP, SL PRS-RSRPP, SL AoA/ZoA. RAN4 not to define accuracy requirements for SL RTOA.

- Recommended WF

  - Discuss the option.

Issue 4-4-2: Applicable number of RBs for defining SL PRS measurement accuracy requirement.

- Proposals

  - Option 1: HW

    - Accuracy requirements for SL PRS measurements are defined for following RB numbers

      - 15kHz SCS: 48, 96

      - 30kHz SCS: 24, 48, 96

      - 60kHz SCS: 24, 48

- Recommended WF

  - Discuss the option.

Issue 4-4-3: Applicable parameters for defining SL PRS measurement accuracy requirement.

- Proposals

  - Option 1: HW

    - Accuracy requirements for SL PRS measurements are defined based on

      - Es/Iot side condition of (0, -3, -3)dB or (0, 0, 0)dB

      - Same channel as their Uu counterparts, FFS for SL AoA/ZoA

      - Nsample = 1

- Recommended WF

  - Discuss the option.

Issue 4-4-4: SL PRS measurement test cases

- Proposals

  - Option 1: CMCC

    - For SL-PRS, it is proposed to define test cases for

      - SL-PRS based AoA/ZOA

      - SL-PRS based RTOA

      - SL-PRS based RSTD

      - SL-PRS based UE Rx-Tx time difference

      - SL-PRS based RSRP

      - SL-PRS based RSRPP

- Recommended WF

  - Discuss the option.

### Sub-topic 4-5: RRM performance for LPHAP

Sub-topic description:

Open issues and candidate options before meeting:

Issue 4-5-1: Applicable PRS measurement accuracy

- Proposals

  - Option 1: HW, E///

    - Existing accuracy requirements are applicable for PRS measurement in INACTIVE with eDRX, and PRS measurement in IDLE.

- Recommended WF

  - Discuss the option.

Issue 4-5-2: Test cases for non-RedCap and RedCap UEs

- Proposals

  - Option 1: CMCC

    - for LPHAP, it is proposed to define test cases for both normal (non-RedCap) and RedCap type of devices.

- Recommended WF

  - Discuss the option.

Issue 4-5-3: eDRX cycle configurations in test cases

- Proposals

  - Option 1: CMCC

    - For LPHAP, it is proposed to define test cases fo following cases:

      - When RAN eDRX <= 10.24s

        - CN eDRX <= 10.24 s

        - CN eDRX >10.24 s

      - When RAN eDRX > 10.24s

- Recommended WF

  - Discuss the option.

Issue 4-5-4: Enhancements/aspects to verify in test cases

- Proposals

  - Option 1: CMCC

    - for LPHAP, it is proposed to define test cases for eDRX with following enhancements:

      - Reduced number of PRS samples

      - parallelPRS-MeasRRC-Inactive-r17 capability

      - Lower Rx beam sweeping in FR2

  - Recommended WF

    - Discuss the option.