3GPP TSG-RAN WG4 Meeting #109										                  R4-2318178

Chicago, US, November 13 – 17, 2023

Agenda item:			8.23.5

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [109][222] NR_MC_enh

Document for:	Information

Introduction

This email thread discusses the RRM core and performance requirements of WI on Multi-carrier enhancements.

Topic #1:	Test cases for DL interruption for Tx switching across 3/4 bands

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2319376 | Huawei, HiSilicon | According to latest RF endorsed CR [R4-2317608], the discription on the DL interruption determined by the next larger value is changed (yellow highlight below). if the UE does not indicate [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18] and if UE additionally reports the capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] as optional UE behaviour, UE is not required to transmit on any of the three bands during the switching period indicated by UE capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] located on band X, as shown in Figure 6.3C.3.5-4.. To allign with RF discription, the modification on RRM part is provided. |

Open issues summary

Sub-topic 1: Correction on interruption

Issue 1-1: The modification on “next larger” DL interruption is provided to in line with RF

Background

According to latest RF endorsed CR [R4-2317608], the discription of the DL interruption determined by the next larger value is changed (yellow highlight below).

if the UE does not indicate [“1”] in the capability [uplinkTxSwitchingMaintainedUL-Trans-r18] and if UE additionally reports the capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] as optional UE behaviour, UE is not required to transmit on any of the three bands during the switching period indicated by UE capability [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] located on band X, as shown in Figure 6.3C.3.5-4.

- Proposals

  - Option 1(Huawei):

If one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs. If [UplinkTxSwitchingAdditionalPeriodDualUL-r18] is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by [UplinkTxSwitchingAdditionalPeriodDualUL-r18]. If [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled] is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by [TBD ULTxSAdditionalPeriod-on-unaffected-band-invovled].

- Recommended WF

Is option 1 agreeable? Directly discuss the CR R4-2319376.

Topic #2:	Test cases for DL interruption for Tx switching across 3/4 bands

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2318765 | Nokia, Nokia Shanghai Bell | - Option 1 provide better test coverage in comparison of Option 2. - Test cases listed in Option 1 should be selected to test performance requirements for DL interruption time in case of UL Tx switching across 3/4 bands. Different switching scenarios are tested in case of 3 and 4 bands. If UE support four uplink bands, tests should be conducted for both three and four bands. Introduce the test cases for the fallback band combination scenario taking into account the new signaling pending on RAN2 discussion. |
| R4-2318766 | Nokia, Nokia Shanghai Bell | Draft CR for test cases |
| R4-2318940 | Qualcomm, Inc. | Proposal 1: Consider the following tests: For single TAG (time offset is 0us): Switching pattern\CA config; A; B; C; D. AB -> CD; TDD; TDD; TDD; TDD. AB->C; FDD; FDD; TDD. For two TAGS (time offset is 9us): Switching pattern\CA config; A; B; C; D. AB -> CD; FDD; FDD; TDD; TDD. AB->C; TDD; TDD; TDD. Proposal 2: The tests should be applicable to UE w/ and w/o this capability, and the interruption requirement is based on the capability and the interruption time specified in 38.133. Proposal 3: Use legacy switching between 1 or 2 bands tests as a starting point. |
| R4-2319297 | ZTE Corporation | Proposal 1: The following test cases are to be defined to verify the interruption due to UE dynamic switching across 3/4 band uplink carriers: For single TAG - DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA - DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA For Two TAGs - DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA - DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA |
| R4-2319377 | Huawei, HiSilicon | Proposal 1: The following test cases are to be defined to verify the interruption due to UE dynamic switching across 3/4 band uplink carriers: For single TAG - DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA - DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA For Two TAG - DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA - DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA Proposal 2: Tx switching scenario for FDD-TDD uplink CA cases for TX switching across 3 bands: - FR1 FDD PCell (Cell 1), FR1 FDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C. - TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2. Case 1; 1 Tx on FDD band A and 1 Tx on FDD band B. Case 2; 2 Tx on TDD band C. - Where TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL Proposal 3: Tx switching scenario for TDD-TDD uplink CA cases for TX switching across 3 bands - FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C. - TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2. Case 1; 1 Tx on TDD band A and 1 Tx on TDD band B. Case 2; 2 Tx on TDD band C. Where - TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL on band A and Band B; - TDD UL/DL pattern is 1D1S2U, S = 10DL: 2GP: 2UL on band C. Proposal 4: Tx switching scenario for uplink CA cases for TX switching across 4 bands For FDD+TDD CA, dynamic Tx switches are switched between case 1 and case 2: Case 1; 1 Tx on FDD band A and 1 Tx on FDD band B. Case 2; 1 Tx on TDD band C and 1 Tx on TDD band D. where TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL For TDD+TDD CA, dynamic Tx switches are switched between case 1 and case 2: Case 1; 1 Tx on TDD band A and 1 Tx on TDD band B. Case 2; 1 Tx on TDD band C and 1 Tx on TDD band D. where - TDD UL/DL pattern is 3D1S4U, S = 10DL: 2GP: 2UL on band A and band B; - TDD UL/DL pattern is 1D1S2U, S = 10DL: 2GP: 2UL on band C and band D. Proposal 5: To verify the symbol level DL interruption for TX switching across 3 or 4 bands, the below legacy principle is considered as a starting point: Triggering an aperiodic CSI-RS L1-RSRP reporting with CSI-RS resources on the OFDM symbol right before the interruption and check UE’s aperiodic L1-RSRP report with corresponding measurement accuracy. Proposal 6: For 1-TAG, time offset is set as 0us; For dual TAG, time offset is set as 9us. Proposal 7: For FDD-TDD uplink CA cases for TX switching across 3 or 4 bands for single TAG, the following symbols are to be verified: - Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability of Switching Period on NR FDD carrier - Symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability of Switching Period on NR TDD carrier Proposal 8: For TDD-TDD uplink CA cases for TX switching across 3 or 4 bands for single TAG, the following symbols are to be verified: - symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability of Switching Period on NR TDD carrier - symbol #4 or symbol #5 or symbol #8 on the 2nd special slot of every 8 slots depending on UE capability of Switching Period NR TDD carrier Proposal 9: For FDD-TDD uplink CA cases for TX switching across 3 or 4 bands for Dual TAG, the following symbols are to be verified: - Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability of Switching Period on NR FDD carrier - Symbol #3 or symbol #5 or symbol #8 on the special slot depending on UE capability of Switching Period on NR TDD carrier Proposal 10: For TDD-TDD uplink CA cases for TX switching across 3 or 4 bands for Dual TAG, the following symbols are to be verified: - Symbol #3 or symbol #5 or symbol #8 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability of Switching Period on NR TDD carrier - Symbol #3 or symbol #5 or symbol #8 on the special slot depending on UE capability of Switching Period on NR TDD carrier |

Open issues summary

Sub-topic 2-1: Test list and test principle

Issue 2-1-1: Test case list for UL Tx switching across 3/4 bands

- Proposals

  - Option 1 (Nokia, ZTE):

For single TAG

- DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA

- DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA

For Two TAG

- DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA

- DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA

  - Option 2 (Qualcomm)

For single TAG (time offset is 0us):

| Switching pattern\CA config | A | B | C | D |
| --- | --- | --- | --- | --- |
| AB -> CD | TDD | TDD | TDD | TDD |
| AB->C | FDD | FDD | TDD |  |

For two TAGS (time offset is 9us):

| Switching pattern\CA config | A | B | C | D |
| --- | --- | --- | --- | --- |
| AB -> CD | FDD | FDD | TDD | TDD |
| AB->C | TDD | TDD | TDD |  |

  - Option 3 (Huawei):

For single TAG

- DL interruptions at Tx switching across three uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA

For Two TAG

- DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA

- DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA

- Recommended WF

Further discussion

Issue 2-1-2: For UE supporting four uplink switching, whether both three bands and four bands are to be verified?

- Proposals

  - Option 1 (Nokia): If UE support four uplink bands, tests should be conducted for both three and four bands.

- Recommended WF

This issue depends on issue 2-1-1.

Issue 2-1-3: Test of fallback band combination

Background

An LS on length of switching period for fallback band combinations is sent to RAN2, and wait for RAN2 feedback.

Issue: Length of switching period for the fallback band combinations From RAN4 UE implementation perspective, when UE support the two Tx switching band combinations of band A+B+C+D and band A+B+C+E, it is possible that UE has different switching periods for the same band pair, for example: - For band A+B+C+D, A+B with period 35us, A+C with period 140us - For band A+B+C+E, A+B with period 140us, A+C with period 35us In this case, RAN4 asks RAN2 the following question: - When the network configures band A+B+C, how to determine the switching period for band pair A+B and A+C from RAN2 signalling perspective? RAN4 is still discussing the applied switch period for the case of A+B+C from RAN4 perspective, and RAN4 will keep RAN2 updated if any new progress.

- Proposals

  - Option 1 (Nokia): Introduce the test cases for the fallback band combination scenario taking into account the new signaling pending on RAN2 discussion.

- Recommended WF

No discussion at this meeting, wait for more RAN2 input.

Issue 2-1-4: Whether and how to verify the capability of “UplinkTxSwitchingAdditionalPeriodDualUL-r18”

Moderator note: “uplinkTxSwitchingPeriod1T1Tto2T” is updated to “UplinkTxSwitchingAdditionalPeriodDualUL-r18”.

- Proposals

  - Option 1 (Qualcomm): The tests should be applicable to UE w/ and w/o UplinkTxSwitchingAdditionalPeriodDualUL-r18, and the interruption requirement is based on the capability and the interruption time specified in 38.133.

- Recommended WF

DL interruption length is determined by the uplink Tx switching period which depends on UE capability. UplinkTxSwitchingAdditionalPeriodDualUL-r18 is a degenerate capability if UE does not support basic switchingPeriod and it is also within the same set of values as {35 us, 140 us, 210 us}. Therefore there is no need to further discuss this issue.

Tentative agreement:

DL interruption length is determined by the uplink Tx switching period which depends on UE capability (e.g., switchingPeriodFor2T-r18, switchingPeriodFor1T-r18, UplinkTxSwitchingAdditionalPeriodDualUL-r18).

Issue 2-1-5: Principle of verifying symbol level DL interruption for TX switching across 3 or 4 bands

- Proposals

  - Option 1(Nokia, Qualcomm, Huawei): The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point, that is,

Triggering an aperiodic CSI-RS L1-RSRP reporting with CSI-RS resources on the OFDM symbol right before the interruption and check UE’s aperiodic L1-RSRP report with corresponding measurement accuracy.

- Recommended WF

Is option 1 agreeable?

Sub-topic 2-2: Tx switching Scenarios

Issue 2-2-1: Tx switching scenario for FDD-TDD uplink CA cases for TX switching across 3 bands

- Proposals

  - Option 1(Nokia, Huawei):

- FR1 FDD PCell (Cell 1), FR1 FDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C.

- TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2.

| Case 1 | 1 Tx on FDD band A and 1 Tx on FDD band B |
| --- | --- |
| Case 2 | 2 Tx on TDD band C |

- Recommended WF

Is option 1 agreeable?

Issue 2-2-2: Tx switching scenario for TDD-TDD uplink CA cases for TX switching across 3 bands

- Proposals

  - Option 1(Nokia, Huawei):

- FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1 with 1TX is on band A, cell 2 with 1TX is on band B, and cell 3 with 2TX is on band C.

- TX switching is from band A to band C, and from band B to band C. Dynamic Tx switches between case1 and case 2.

| Case 1 | 1 Tx on TDD band A and 1 Tx on TDD band B |
| --- | --- |
| Case 2 | 2 Tx on TDD band C |

- Recommended WF

Is option 1 agreeable?

Issue 2-2-3: Tx switching scenario for uplink CA cases for TX switching across 4 bands

- Proposals

  - Option 1(Huawei):

- TX switching scenario for TX switching across 4 bands can be similar as 3 bands, where Dynamic Tx switches are switched between case 1 and case 2

- For FDD+TDD CA

| Case 1 | 1 Tx on FDD band A and 1 Tx on FDD band B |
| --- | --- |
| Case 2 | 1 Tx on TDD band C and 1 Tx on TDD band D |

- For TDD+TDD CA

| Case 1 | 1 Tx on TDD band A and 1 Tx on TDD band B |
| --- | --- |
| Case 2 | 1 Tx on TDD band C and 1 Tx on TDD band D |

- Recommended WF

Is option 1 agreeable?

Sub-topic 2-3: Test configurations

Issue 2-3-1: TDD UL/DL pattern for FDD-TDD uplink CA cases for TX switching across 3 or 4 bands

- Proposals

  - Option 1(Nokia): The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point, that is

- TDD UL/DL pattern is 3D1S4U, S = 11DL: 1GP: 2UL

- Recommended WF

Is option 1 agreeable?

Issue 2-3-2: Different TDD UL/DL patterns for TDD-TDD uplink CA cases for TX switching across 3 or 4 bands

- Proposals

  - Option 1(Nokia): The legacy RRM tests for Tx switching on one or two UL bands can be considered as a starting point, that is

- TDD UL/DL pattern is 3D1S4U, S = 11DL: 1GP: 2UL;

- TDD UL/DL pattern is 1D1S2U, S = 11DL: 1GP: 2UL

- Recommended WF

Is option 1 agreeable?

Issue 2-3-3: MRTD setup for single TAG and dual TAG

- Proposals

  - Option 1(Nokia, Qualcomm, Huawei):

- For 1-TAG, time offset is set as 0us;

- For 2-TAG, time offset is set as 9us.

- Recommended WF

Is option 1 agreeable?

Issue 2-3-4: Which symbol to be verified for FDD-TDD uplink CA cases for TX switching across 3 or 4 bands for single TAG

- Proposals

  - Option 1(Nokia, Huawei):

- Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR FDD carrier

- Symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

- Recommended WF

Is option 1 agreeable?

Issue 2-3-5: Which symbol to be verified for TDD-TDD uplink CA cases for TX switching across 3 or 4 bands for single TAG

- Proposals

  - Option 1(Nokia, Huawei):

- symbol #4 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD PCell

- symbol #4 or symbol #5 or symbol #8 on the 2nd special slot of every 8 slots depending on UE capability uplinkTxSwitchingPeriod NR TDD SCell

- Recommended WF

Is option 1 agreeable?

Issue 2-3-6: Which symbol to be verified for FDD-TDD uplink CA cases for TX switching across 3 or 4 bands for Dual TAG

- Proposals

  - Option 1(Nokia, Huawei):

Symbol #8 or symbol #9 or symbol #10 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR FDD carrier

Symbol #3 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

- Recommended WF

Is option 1 agreeable?

Issue 2-3-7: Which symbol to be verified for TDD-TDD uplink CA cases for TX switching across 3 or 4 bands for Dual TAG

- Proposals

  - Option 1(Nokia, Huawei):

Symbol #3 or symbol #5 or symbol #8 in the slot overlapping with the special slot of the NR TDD carrier depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

Symbol #3 or symbol #5 or symbol #8 on the special slot depending on UE capability uplinkTxSwitchingPeriod on NR TDD carrier

- Recommended WF

Is option 1 agreeable?