3GPP TSG-RAN WG4 Meeting #109	R4-2318234

Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Title:	Release independence of DualTAG with TX switching

Source:	Qualcomm Incorporated

Agenda item:	8.23.2.2

Release:	Rel-18

WI: 	NR_MC_enh-Core

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

Dual TAG requirements have been adopted to the specifications with CR [49] and [50]. This paper discusses the adoption of the feature to the release independent.

# 2. 	Discussion

The TX switching feature has been constrained to the single TAG with sentence saying so in the TS 38.101-1 in earlier releases. Also WI’s were confined to single TAG case.

One of the advertised use cases for TX switching with two bands has been FDD-TDD band combinations and 3 and 4 band cases would be adding one TDD band more and possibly an other FDD band. Typical spectrum holding is mid band (1.7 GHz) FDD combined with high (2.6 GHz) or ultra high (3.5 GHz) band TDD. Mid bands typically are deployed with 15 kHz SCS and TDD bands at higher frequencies are deployed with 30 kHz SCS.

With single TAG, there is only one timing alignment used and one reference used and therefore the UE transmits at the same time on the two bands when switching option dualUL is used. When switchedUL is used, the UL timing is adopted from the reference band. Switching to the band that is not used as a reference will cause UE to start transmitting mid symbol on the switching from band. If there are more than one UE transmitting on the same slot on different RBs, the timing of the symbols at the gNB receiver maybe misaligned with each other.

Observation 1: Using two TAGs enables network to ensure better alignment of the transmissions between different UE when TX Switching is deployed

In RAN2 capability structures, UE capability for multiple TAGs is already built in from Rel-16 since TX Switching adopts the regular CA capability structure. Nested structure is as follows from 38.331:

BandCombination-UplinkTxSwitch

BandCombination

CA-ParametersNR

supportedNumberTAG {n2, n3, n4}

In RAN1 specifications are agnostic to the number of TAGs.

Number of supported TAGs is UE capability and it is up to network to configure the UE with its preferred number of TAG and their reference DL cells. UE can continue to support only one TAG and network continue to configure only one TAG. With this, there can not be any backwards compatibility issues if it is added as release independent from earlier release.

Observation 2: In conclusion, there are no technical reasons not to support > 1 TAG with TX switching from Rel-16.

# Conclusion

We observer the following

Observation 1: Using two TAGs enables network to ensure better alignment of the transmissions between different UE when TX Switching is deployed

Observation 2: In conclusion, there are no open issues from specifications point of view to support > 1 TAG with TX switching from Rel-16.

# Reference

[1]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97e, Electronic meeting, September 12 - 16, 2022

[2] R4-2211508 (R1-2205502), “LS on UL Tx switching across 3 or 4 bands in Rel-18”, RAN1, Ran4#104, Aug 2022

[3]	R4-2204605, “Introduction of TX switching for non-collocated UL CA”, Ericsson, 3GPP TSG-RAN WG4 Meeting#102-e, Electronic meeting, 21 February – 3 March 2022

[5]	R1-2204724, “On multi-carrier UL Tx switching scheme”, MediaTek Inc.

[6]	RP-221880, “Discussion on target scenarios for Rel-18 UL Tx switching in NR Multi-carrier enhancements WI”, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

[7]	R1-2205052, “Discussion on Rel-18 UL Tx switching”, Qualcomm, 3GPP TSG RAN WG1 #109-emeeting, e-Meeting, May 9th – May 20th, 2022

[8]	R4-2214044, “UL TX switching requirements for two TAGs”, RAN4#104e, Online, Aug 2022

[9]	R4-2214043, “UL TX switching requirements for multiple TAGs”, RAN4#104e, Online, Aug 2022

[10]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[11]	RP-222672, “Moderator's summary for discussion [97e-19-R18-Multicarrier]”, Moderator (NTT DOCOMO, INC.), 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[12]	R4-2214463, “WF on UL Tx switching across 3/4 bands with single TAG”, China Telecom, RAN4#104e, Online, Aug 2022

[13]	R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, China Telecom, RAN4#104e, Online, Aug 2022

[14]	R4-2215163, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104e, Online, Aug 2022

[15]	R4-2215306, “LS on switching option capability for UL 2Tx-2Tx switching”, RAN2, RAN4#104Bis-e, Online, Oct 2022

[16]	R4-2217740, “WF on UL Tx switching across 3/4 bands with single TAG, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[17]	R4-2217741, “LS on Rel-18 UL Tx switching”, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[18]	R4-2217742, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104Bis-e, Online, Oct 2022

[19]	R4-2220546, “WF on UE Tx switching requirement for single TAG”, China Telecom, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[20]	R4-2220547, “WF on UE Tx swithing requirement for multiple TAG”, Ericsson, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[21]	R4-2220548, “LS on Rel-18 Tx switching”, China Telecom, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[22]	R4-2220118, “Topic summary for [105][138] NR_MC_enh_UERF”, Moderator(China Telecom), 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, November 14th - November 18th, 2022

[23]	R4-2219876, “UE behaviour in case of non-equal switching time”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[24]	R4-2219877, “UE requirements during overlaps due to NZ-MTTD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[25]	R4-2215569, “UL Outage time considerations for 2TAG”, Qualcomm, 3GPP TSG-RAN4 Meeting #104Bis-e, Oct 2022

[26] R4-2303693, “WF on Multi-carrier enhancements for NR”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[27]	R4-2303507, “LS on Rel-18 Multi-carrier enhancement for NR”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[28]	R4-2303719, “CR for 38.101-1: Time mask for switching across three or four uplink bands”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[29]	R4-2215569, “UL Outage time considerations for 2TAG”, 3GPP TSG-RAN4 Meeting #104Bis-e, Oct 2022, Online

[30]	R4-2300822, “UE switching time in more complicated scenarios”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[31]	R4-2304127, “Switching period ambiguity”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-Bis-e, Online,, 17th April – 26th April 2023

[32]	R4-2302751, “UE ON-OFF time masks for non-equal switching time cases”, Qualcomm Incorporated,  3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[33]	R4-2305789, “3 and 4 band TX switching open issues and requirements, Qualcomm Incorporated,  3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[34]	R4-2305808, “draft CR on TX Switching when UE is configured for 3 or 4 bands UL”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-bis-e, Online,, 17th April – 26th April 2023

[35]	R4-2306621, “WF on Rel-18 Tx switching across 3/4 bands”, China Telecom, 3GPP TSG-RAN WG4 Meeting #106-bis-e, Electronic Meeting, 17 April - 26 April, 2023

[36]	R4-2304162, “Simultaneous switching”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-bis-e, Online,, 17th April – 26th April 2023

[37]	R4-2310270, “CR for 38.101-1: Time mask for switching across three or four uplink bands”, China Telecom,3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, 22 - 26 May, 2023

[38]	R4-2310503, “CR for 38.101-1 to clarify the time mask for switching with multiple TAGs”, Huawei, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[39]	R4-2310495, “LS on multi-carrier enhancement”, vivo, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[40]	R4-2310496, “WF on ambiguity issues for Tx switching accorss four bands”, Mediatek, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[41]	R4-2312746, “Remaining issues on Rel-18 Multi-carrier enhancements”, Huawei, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21 – August 25, 2023

[42]	RP-232658, “Summary for discussion [RAN1-R18-ULTxSwitching]”, NTT DOCOMO, 3GPP TSG-RAN Meeting #101, Bangalore, India, September 11-15, 2023

[43]	RP-232688, “Revised WID: NR CA band combinations with two SUL cells in Rel-18”, CMCC, China Telecom, 3GPP TSG-RAN Meeting #101, Bangalore, India, September 11-15, 2023

[44]	R4-2314933, “LS on Rel-18 Tx switching enhancement”, Huawei, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21 – August 25, 2023

[45]	R4-2317609, “LS on Rel-18 UL Tx switching for parallel switching on four bands”, RAN4(Mediatek), 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[46]	R4-2317610, “LS on unaffected band case for uplink Tx switching”, RAN4(vivo), 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[47]	R4-2317774, “draftLS on Rel-18 Tx switching enhancement” , RAN4(Huawei), 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[48]	R4-2317608, “Draft CR for 38.101-1: Time mask for switching across three or four uplink bands” China Telecom, 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[49]	R4-2314674, “Correction of applicability of time mask for Tx swtiching with dual TAG”,  Ericsson, Sony, 3GPP TSG-RAN WG4 Meeting#108, Toulouse, France, 21 – 25 August 2023

[50]	R4-2310503, “CR for 38.101-1 to clarify the time mask for switching with multiple TAGs”, Huawei, HiSilicon, Xiaomi, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023