3GPP TSG-RAN4 Meeting #109	R4-2318248

Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Title:	UE capability clarifications

Source:	Qualcomm Incorporated

Agenda item:	8.23.2.1

Release:	Rel-18

WI: 	NR_MC_enh-Core

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

RAN4 has defined number of new capabilities for rel-18 TX switching. In some cases the discussion has been independent focusing on a problem at hand. The resulting capabilities might benefit from comprehensive view and consolidation in some cases. In this paper we discuss how some capabilities could be merged to clarify the behaviour.

# 2. 	Discussion

## 2.1 	Capabilities related to switching period in four band switching case {1T,1T,0,0} to {0,0,1T,1T}

For reference, we consider where the case when UE switches from band pair [A,B] to [C,D].

The baseline behaviour was agreed and communicated to RAN2 with LS [17] where it was stated that none of the bands are expected to the capable for transmissions during a switching on an band pair and then later with LS [21] where the baseline the switching period is max{Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}.

The clarification to this ambiguity and to allow UE to be faster or slower switching period, an additional capability uplinkTxSwitching1T1Tto1T1 was defined to explicitly define the switching period from list {35u, 140us, 210us} for this case was agreed in communicated to RAN2 with LS [39].

Lastly, an additional intermediate enhancement for this case was agreed and communicated to RAN2 with LS [45] where by declaring this capability UE informs network that it can follow the order of granted transmissions and informs network which is the preferred order of the grants and if UE is scheduled in its preferred order, then the switching period is min{max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

LS [45] also stated that the two above can not be declared simultaneously.

RAN2 draft CR this is called UplinkTxSwitchingAdditionalPeriodDualUL

These could be merged to be one feature group since they are used to declare switching period for the same switching case.

## 2.2 	Transmission on a third band during a switching on a band pair e.g. {1T,1T,0} to {1T,0,1T)

The baseline behaviour was agreed and communicated to RAN2 with LS [13] (and also mentioned no further agreement were made in LS [17]) where it was stated that none of the bands are expected to the capable for transmissions during a switching on a band pair.

In LS [21] a capability was agreed and communicated to RAN2 that UE can declare being capable of transmission on a band that is not a part of switching, i.e. switch-to or switch-from band.

In LS [39], the [on-unaffected-band-involved] further capability was agreed and communicated to RAN2. With this capability, UE can declare that if there is an ongoing transmission on a band C that is not scheduled for switching e.g. between A and B, UE will need longer switching time and none of the three bands can transmit during that switching period.  This was later refined in LS [46] so that the capability is an exact value rather than the “next larger from the list”.

This FG could look like the following:

UplinkTxSwitchingAdditionalPeriodDualUL (was Unaffected band in RAN4)

Per band per band pair per band combination where the band is not a member of the band pair.  Absense of this capability means that the UE is not expected to be capable for transmissions on the band during the switching period between the declared band pair. Options are:

- UE is able to transmit on the band

- Switching period when UE is not able to transmit on the band or the band pair{35, 140, 210} usec

# Conclusion

We discussed some ideas for simplification of the capabilities for the TX switching objective of MC_enh WI.

# Reference

[1]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97e, Electronic meeting, September 12 - 16, 2022

[2] R4-2211508 (R1-2205502), “LS on UL Tx switching across 3 or 4 bands in Rel-18”, RAN1, Ran4#104, Aug 2022

[3]	R4-2204605, “Introduction of TX switching for non-collocated UL CA”, Ericsson, 3GPP TSG-RAN WG4 Meeting#102-e, Electronic meeting, 21 February – 3 March 2022

[5]	R1-2204724, “On multi-carrier UL Tx switching scheme”, MediaTek Inc.

[6]	RP-221880, “Discussion on target scenarios for Rel-18 UL Tx switching in NR Multi-carrier enhancements WI”, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

[7]	R1-2205052, “Discussion on Rel-18 UL Tx switching”, Qualcomm, 3GPP TSG RAN WG1 #109-emeeting, e-Meeting, May 9th – May 20th, 2022

[8]	R4-2214044, “UL TX switching requirements for two TAGs”, RAN4#104e, Online, Aug 2022

[9]	R4-2214043, “UL TX switching requirements for multiple TAGs”, RAN4#104e, Online, Aug 2022

[10]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[11]	RP-222672, “Moderator's summary for discussion [97e-19-R18-Multicarrier]”, Moderator (NTT DOCOMO, INC.), 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[12]	R4-2214463, “WF on UL Tx switching across 3/4 bands with single TAG”, China Telecom, RAN4#104e, Online, Aug 2022

[13]	R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, China Telecom, RAN4#104e, Online, Aug 2022

[14]	R4-2215163, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104e, Online, Aug 2022

[15]	R4-2215306, “LS on switching option capability for UL 2Tx-2Tx switching”, RAN2, RAN4#104Bis-e, Online, Oct 2022

[16]	R4-2217740, “WF on UL Tx switching across 3/4 bands with single TAG, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[17]	R4-2217741, “LS on Rel-18 UL Tx switching”, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[18]	R4-2217742, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104Bis-e, Online, Oct 2022

[19]	R4-2220546, “WF on UE Tx switching requirement for single TAG”, China Telecom, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[20]	R4-2220547, “WF on UE Tx swithing requirement for multiple TAG”, Ericsson, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[21]	R4-2220548, “LS on Rel-18 Tx switching”, China Telecom, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[22]	R4-2220118, “Topic summary for [105][138] NR_MC_enh_UERF”, Moderator(China Telecom), 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, November 14th - November 18th, 2022

[23]	R4-2219876, “UE behaviour in case of non-equal switching time”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[24]	R4-2219877, “UE requirements during overlaps due to NZ-MTTD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[25]	R4-2215569, “UL Outage time considerations for 2TAG”, Qualcomm, 3GPP TSG-RAN4 Meeting #104Bis-e, Oct 2022

[26] R4-2303693, “WF on Multi-carrier enhancements for NR”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[27]	R4-2303507, “LS on Rel-18 Multi-carrier enhancement for NR”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[28]	R4-2303719, “CR for 38.101-1: Time mask for switching across three or four uplink bands”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[29]	R4-2215569, “UL Outage time considerations for 2TAG”, 3GPP TSG-RAN4 Meeting #104Bis-e, Oct 2022, Online

[30]	R4-2300822, “UE switching time in more complicated scenarios”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[31]	R4-2304127, “Switching period ambiguity”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-Bis-e, Online,, 17th April – 26th April 2023

[32]	R4-2302751, “UE ON-OFF time masks for non-equal switching time cases”, Qualcomm Incorporated,  3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[33]	R4-2305789, “3 and 4 band TX switching open issues and requirements, Qualcomm Incorporated,  3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[34]	R4-2305808, “draft CR on TX Switching when UE is configured for 3 or 4 bands UL”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-bis-e, Online,, 17th April – 26th April 2023

[35]	R4-2306621, “WF on Rel-18 Tx switching across 3/4 bands”, China Telecom, 3GPP TSG-RAN WG4 Meeting #106-bis-e, Electronic Meeting, 17 April - 26 April, 2023

[36]	R4-2304162, “Simultaneous switching”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-bis-e, Online,, 17th April – 26th April 2023

[37]	R4-2310270, “CR for 38.101-1: Time mask for switching across three or four uplink bands”, China Telecom,3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, 22 - 26 May, 2023

[38]	R4-2310503, “CR for 38.101-1 to clarify the time mask for switching with multiple TAGs”, Huawei, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[39]	R4-2310495, “LS on multi-carrier enhancement”, vivo, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[40]	R4-2310496, “WF on ambiguity issues for Tx switching accorss four bands”, Mediatek, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[41]	R4-2312746, “Remaining issues on Rel-18 Multi-carrier enhancements”, Huawei, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21 – August 25, 2023

[42]	RP-232658, “Summary for discussion [RAN1-R18-ULTxSwitching]”, NTT DOCOMO, 3GPP TSG-RAN Meeting #101, Bangalore, India, September 11-15, 2023

[43]	RP-232688, “Revised WID: NR CA band combinations with two SUL cells in Rel-18”, CMCC, China Telecom, 3GPP TSG-RAN Meeting #101, Bangalore, India, September 11-15, 2023

[44]	R4-2314933, “LS on Rel-18 Tx switching enhancement”, Huawei, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21 – August 25, 2023

[45]	R4-2317609, “LS on Rel-18 UL Tx switching for parallel switching on four bands”, RAN4(Mediatek), 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[46]	R4-2317610, “LS on unaffected band case for uplink Tx switching”, RAN4(vivo), 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[47]	R4-2317774, “draftLS on Rel-18 Tx switching enhancement” , RAN4(Huawei), 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[48]	R4-2317608, “Draft CR for 38.101-1: Time mask for switching across three or four uplink bands” China Telecom, 3GPP TSG-RAN WG4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023

[49]	R4-2314674, “Correction of applicability of time mask for Tx swtiching with dual TAG”,  Ericsson, Sony, 3GPP TSG-RAN WG4 Meeting#108, Toulouse, France, 21 – 25 August 2023

[50]	R4-2310503, “CR for 38.101-1 to clarify the time mask for switching with multiple TAGs”, Huawei, HiSilicon, Xiaomi, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023