3GPP TSG-RAN WG4 Meeting #109                                  R4-2318253

Chicago, US, November 13 – 17, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	RSD of PC2 n13 and n14

Agenda Item:	7.22.2	High power UE for FR1 for FDD single band(s) with PC2	[HPUE_NR_FR1_FDD_R18]

Document for:	Approval

Introduction

Reference Sensitivity Degradation (RSD) of n13 and n14 for PC2 was discussed at the RAN4#108-bis meeting, however, no agreement was reached. More contributions were encouraged [1]. In this paper, we proposed RSD for PC2 n13 and n14.

Discussion

Review WF

<Way forward>: Take the following values in table below for n13 and n14 as starting point, companies are encouraged to provide additional RSD proposals at RAN4#109 and to further check the values for FDD PC2 by 1Tx architecture:

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | Skyworks(R4-2300652) | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 0.9 | 1.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 0.8 | 0.9 |  |  |  |  |  |  |  |  |
| n14 | Skyworks (R4-2311152) | 0.4 | 0.8 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.3 | 0.6 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 0.9 | 1.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 0.5 | 0.8 |  |  |  |  |  |  |  |  |

<Way forward>: Take the following values in table below for n13 and n14 as starting point, companies are encouraged to provide additional RSD proposals at RAN4#109 and to further check the values for FDD PC2 by 2Tx architecture:

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | Skyworks(R4-2300652) | 1.7 | 1.7 |  |  |  |  |  |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 2.0 | 2.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 1.4 | 1.4 |  |  |  |  |  |  |  |  |
| n14 | Skyworks (R4-2311152) | 0.8 | 1.3 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 2.0 | 2.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 1.2 | 1.4 |  |  |  |  |  |  |  |  |

RSD for PC2 n13 and n14

FE architecture

The two kinds of FE architecture shown in the Figure 1 are used to analyse RSD. For 2Tx/2Rx architecture, the mutual influence between primary Tx and diversity Tx was considered.

Figure 1. FE architecture

Calculation conditions

Calculation conditions shown in the Table 1 are used to RSD estimation.

Table 1. Calculation conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 23 dBm | For 1Tx PC3, at ANT1 |
|  | 26 dBm | For 1Tx PC2, at ANT1 |
|  | 23 dBm | For 2Tx PC2, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1⇔ANT2 |
| Duplexer rejection | 55 | Tx to Rx, at Tx band |
|  | 55 | Tx to Rx, at Rx band |
|  | 45 | Tx to ANT, at Rx band |
| BPF | 45 | ANT to Rx, at Tx band |
| MRC | ANT1&ANT2 | Correlated |

Results

The calculation results are shown in Table 2.

Table 2. RSD for PC2 n13 and n14

|  | Parameter | Unit | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz | 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | 1Tx PC2 RSD | dB | 0.7 | 0.8 | - | - | - | - | - | - | - |
|  | 2Rx PC2 RSD | dB | 0.7 | 1.0 | - | - | - | - | - | - | - |
| n14 | 1Tx PC2 RSD | dB | 0.7 | 0.8 | - | - | - | - | - | - | - |
|  | 2Rx PC2 RSD | dB | 0.7 | 1.0 | - | - | - | - | - | - | - |

Proposal:

The RSD of PC2 n13 and n14 are proposed highlighted in Table 2.

Conclusion

In this paper, we proposed the RSD of PC2 n13 and n14.

Proposal:

The RSD of PC2 n13 and n14 are proposed highlighted in Table 2.

Table 2. RSD for PC2 n13 and n14

|  | Parameter | Unit | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz | 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | 1Tx PC2 RSD | dB | 0.7 | 0.8 | - | - | - | - | - | - | - |
|  | 2Rx PC2 RSD | dB | 0.7 | 1.0 | - | - | - | - | - | - | - |
| n14 | 1Tx PC2 RSD | dB | 0.7 | 0.8 | - | - | - | - | - | - | - |
|  | 2Rx PC2 RSD | dB | 0.7 | 1.0 | - | - | - | - | - | - | - |

References

[1] R4-2317586 “WF on HPUE for FDD bands”, China Unicom, 3GPP TSG-RAN WG4 Meeting #108-bis, October 09 – 13, 2023