# 3GPP TSG-RAN WG4 Meeting #109                                                             R4-2318265

# Chicago, US, 13th – 17th November 2023

# Title:	TP for TR37.718-11-21 Support of DC_8A_n1A-n79A

# Source:	Softbank

# Agenda Item:	7.6.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 37.718-11-21[5] combinations as requested in RAN4#108.

- EN-DC 8_n1-n79 (1 band LTE (1 DL / 1 UL)) and 2 NR bands (2 DL / 1 UL))

- Reference

[1] 3GPP TR 21.905 Vocabulary for 3GPP Specifications (Release 17) V17.1.0

[5] 3GPP TS37.718-11-21 Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) (Release 18) V0.8.0

[6] 3GPP TS37.718-21-11 Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band (Release 18) V0.8.0

[7] 3GPP TS38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18) V18.3.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

- 6.x		DC_8_n1-n79

- 6.x.1	 Operating bands for DC

Table 6.x.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n1A-n79A1 | 8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n791 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 1: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. |  |  |  |  |  |  |  |  |

- 6.x.2	 configuration for DC

Table 6.32.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n1A-n79A | DC_8A_n79A |

- 6.x.3	 Co-existence studies

For DC_8_n1-n79, the harmonic and IMD issues are reused from DC_1-8_n79 referring to 5.1.16.3 in TR37.716-21-11[6].

When Uplink EN-DC configuration is DC_8A_n79A, IMD4 of (3*B8-n79) will fall into Rx band of n1.

- 6.x.4	 ∆TIB and ∆RIB values

For DC_8_n1-n79, the ΔTIB and ΔRIB values are reused from CA_n1-n8-n79 referring to TS38.101-1[7] Table 6.2A.4.2.4-1, Table 7.3A.3.2.3-1 and are given tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n1-n79 | 0.3 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n1-n79 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

- 6.x.5	MSD

For DC_8_n1-n79, the MSD value is reused from DC_1-8_n79 referring to 5.1.16.3 in TR37.716-21-11[6].

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n1A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |

-- Unaffected parts omitted –

-- End of TP --