# 3GPP TSG-RAN WG4 Meeting # 109				                                            R4-2318329

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0127 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on release independent for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LTE_V2X_PC5_combos_R18-Core |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is the resubmission of endorsed draft CR R4-2315140 in RAN4#108bis. - Release independent is considered as the objective for the baseket WI for V2X con-current operation combinations. The RF requirements for R18 introduced band combinations were specified in R4-2218443 and R4-2218444, and the release independent requirements need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the release independent requirements for concurrent operation of Uu and V2X bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Release independent requirements for concurrent operation of Uu and V2X bands are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8, B.4.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR would introduce new Rel-18 specification of TS 38.307. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.8	Additional V2X configurations for NR frequency range 1

Requirements for a Rel-18 UE for additional NR SL configurations involving V2X con-current operation compared to TS 38.101-1 of Rel-18 [4] are introduced via this clause.

Table 5.8-1: NR V2X con-current operation within in FR1

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| Operating bands for V2X communication with con-current operation | Rel-16 | Table B.4.12-1, Table B.4.12-2 | NR V2X new bands, intra-band/inter-band con-current operation, SL MIMO/TxD are release independent according to requirements in Table Table B.4.12-1, Table B.4.12-2 |

<End of Change 1>

<Start of Change 2>

B.4.12	Common UE RF requirements for NR V2X

The requirements and test cases listed in Table B.4.12-1 are specified in REL-18 version of TS 38.101-1 [2].

Table B.4.12-1: Common UE RF requirements for release independent operating bands and con-current operation for NR V2X

| Clause | Description |
| --- | --- |
| 5.2E | Operating band for V2X |
| 5.2E.1 | V2X operating bands |
| 5.2E.2 | V2X operating bands for con-current operation |
| 5.3E | Channel bandwidth for V2X |
| 5.4E | Channel arrangement for V2X |
| 6.2E | Transmitter power for V2X |
| 6.3E | Output power dynamics for V2X |
| 6.4E | Transmit signal quality for V2X |
| 6.5E | Output RF spectrum emissions for V2X |
| 7.3E | Reference sensitivity for V2X |
| 7.4E | Maximum input level for V2X |
| 7.5E | Adjacent channel selectivity for V2X |
| 7.6E | Blocking characteristics for V2X |
| 7.7E | Spurious response for V2X |
| 7.8E | Intermodulation characteristics for V2X |

The requirements and test cases listed in Table B.4.12-2 are specified in REL-18 version of TS 38.101-3 [2].

Table B.4.12-2: Common UE RF requirements for release independent intra-band and inter-band con-current operation for NR V2X

| Clause | Description |
| --- | --- |
| 5.2E | Operating bands for V2X |
| 5.3E | UE Channel bandwidth for V2X |
| 5.5E | Configuration for V2X operation |
| 6.2E | Transmitter power for V2X in FR1 |
| 6.3E | Output power dynamics for V2X |
| 6.4E | Transmit signal quality for V2X operation in FR1 |
| 6.5E | Output RF spectrum emissions for V2X operation in FR1 |
| 7.3E | Reference sensitivity for V2X operation in FR1 |
| 7.4E | Maximum input level for V2X operation in FR1 |
| 7.5E | Adjacent channel selectivity for V2X operation in FR1 |
| 7.6E | Blocking characteristics for V2X in FR1 |
| 7.7E | Spurious response for V2X in FR1 |
| 7.8E | Intermodulation characteristics for V2X operation in FR1 |

<End of Change 2>