# 3GPP TSG-RAN WG4 Meeting # 109				                                             R4-2318337

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR #27: on SL Rx-Tx time difference and SL RSRPP measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - SL PRS based measurements are introduced in Rel-18 positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce measurement requirements for SL PRS based UE Rx-Tx time difference and RSRPP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement requirements for SL PRS based UE Rx-Tx time difference and RSRPP are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 12A.4, 12A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12A.4	SL Rx-Tx time difference measurements

12A.4.1	Introduction

The requirements in clause 12A.4 shall apply provided the UE has received [NR-SL-RxTx-RequestLocationInformation] from LMF or another UE via SLPP requesting the UE to measure and report SL Rx-Tx time difference measurements defined in TS 38.215 [4].

12A.4.2	Requirements Applicability

The requirements in clause 12A.4 apply for periodic and triggered SL Rx-Tx time difference measurements, provided:

-	SL Rx-Tx time difference related side conditions given in clause [TBD] are met for a corresponding Band.

12A.4.3	Measurement Capability

SL Rx-Tx time difference measurement capability is as indicated by the UE in [NR-SL-RxTx-ProvideCapabilities] according to TS 38.355 [TBD].

12A.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause [TBD].

The SL Rx-Tx time difference measurements performed and reported according to this section shall meet the SL Rx-Tx time difference measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.

12A.4.5	Measurement Period Requirements

When the physical layer receives [NR-SL-RxTx-ProvideAssistanceData] message from [NR-SL-RxTx-RequestLocationInformation] message from LMF or another UE via SLPP, the UE shall be able to measure multiple (up to the UE capability specified in Clause [TBD]) SL Rx-Tx time difference measurements, defined in TS 38.215 [4], during TSL-RxTx defined as:
TSL-RxTx=s=1S-1Teffect,S+Tlast

Where,

S = scaling factor * Nsample. Scaling factor is [FFS]. Nsample = FFS.

Teffect,S=ts+1-ts, where ts and ts+1 are the start of the s-th and (s+1)-th slot where UE needs to measure SL-PRS.

Tlast=TSL_processing, where TSL_processingis the minimum processing time after the end of a slot carrying the active SL-PRS resource(s) as indicated in [Common SL PRS Processing Capability in a SL BWP].

The requirements in clause 12A.4 do not apply if the SL-PRS configuration given by [NR-SL-PRS-AssistanceData] exceeds any of the UE measurement capabilities given by [NR-SL-PRS-ResourcesCapability] in [NR-SL-RxTx-ProvideCapabilities], and it is up to UE implementation which SL-PRS resources are measured, subject to UE measurement capabilities.

When synchronization reference source change occurs at the UE which is performing the measurement, UE shall restart the SL Rx-Tx time difference measurement after change and the previous measurement samples are dropped. The measurement period requirements in this section apply, and the starting point of the measurement period is after the change.

Editor’s note: FFS whether to limit the number of restarting.

12A.5	SL PRS-RSRPP measurements

12A.5.1	Introduction

The requirements in clause 12A.5 shall apply provided the UE has received [NR-SL-RSRPP-RequestLocationInformation] from LMF or another UE via SLPP requesting the UE to measure and report SL PRS-RSRPP measurements defined in TS 38.215 [4].

12A.5.2	Requirements Applicability

The requirements in clause 12A.5 apply for periodic and triggered SL PRS-RSRPP measurements, provided:

-	SL PRS-RSRPP related side conditions given in clause [TBD] are met for a corresponding Band.

12A.5.3	Measurement Capability

SL PRS-RSRPP measurement capability is as indicated by the UE in [NR-SL-RSRPP-ProvideCapabilities] according to TS 38.355 [TBD].

12A.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

The SL PRS-RSRPP measurements performed and reported according to this section shall meet the SL PRS-RSRPP measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.

12A.5.5	Measurement Period Requirements

When the physical layer receives [NR-SL-RSRPP-ProvideAssistanceData] message from [NR-SL-RSRPP-RequestLocationInformation] message from LMF or another UE via SLPP, the measurement period requirements for SL PRS-RSRP defined in 12A.3.5 is re-used for PRS-RSRPP measurement.

<End of Change 1>