# 3GPP TSG-RAN WG4 Meeting # 109				 R4-2318344

# Chicago, US, November 13 – 17, 2023

Title: 	Discussion on Rel-18 feature list for NR

Source: 	CATT

Agenda item:	10

Document for:	Discussion

- Introduction

This contribution includes the RAN4 UE feature list for Rel-18 NR.

- Discussion

The feature list for MG enhancement, NTN enhancement and NES are included and summarized as below.

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Combination of Pre-MG and concurrent MG | Support of combined configuration of Pre-MG and concurrent MG |  | Yes | 19-2, 19-3-1/19-3-2 | UE cannot be configured with concurrent MG and Pre-MG simultaneously | Per UE | No | No |  |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Combination of NCSG and concurrent MG | Support of combined configuration of NCSG and concurrent MG |  | Yes | 19-1a/19-1b, 19-2 | UE cannot be configured with concurrent MG and NCSG simultaneously | Per UE | No | No |  |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Interruption for UE supporting needforgap | Report whether interruptions are needed when UE report “no-gap” in needforgap |  | Yes | 24-6 | Interruption is not clear when UE report “no-gap” in needforgap | Per UE | No | No |  | Already sent to RAN2 | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Interruption for inter-RAT NR measurement | Report whether interruptions and gap are needed for inter-RAT NR measurement |  | Yes |  | Gap and interruptioninformation is not clear for inter-RAT NR measurement | Per UE | No | No |  | Already sent to RAN2 | Optional with capability signalling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh | 40-1 | Supported MG patterns for FR2-NTN | Support of FR2 MG patterns for FR2-NTN |  | Yes |  | UE cannot support FR2 MG for NTN | Per UE | No | No |  |  | Optional with capability signalling |
| 40. NR_NTN_enh | 40-2 | Beam switching time beam switching delay from one satellite to another for type 2 UE | The value of beam switching time beam switching delay from one satellite to another for type 2 UE |  | Yes |  | NW cannot know the beam switching time beam switching delay from one satellite to another for type 2 UE for NTN | Per UE | No | No |  |  | Optional with capability signalling |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. # Netw_Energy_NR | 42-1 | Support of SSB-less SCell operation | Support of SSB-less SCell operation |  | Yes |  | UE cannot support SSB-less SCell operation | Per BC | No | No |  |  | Optional with capability signalling |