3GPP TSG-RAN WG4 Meeting #109	R4-2318351

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulations for UE Demodulation for non-colocated FR1 intra-band NR-CA

Agenda item:	8.11.4

Document for:	Discussion

Introduction

In RAN4#108b a lot of progress was made on UE demodulation requirements for non-colocated FR1 intra-band NR-CA. The WF [1] captures nearly all needed simulation parameters/configuration, with only the MCS indices remaining open:

Way forward: Rank/MCS selection - Interested companies are encouraged to bring the PDSCH simulation results with impairments in RAN4#109 to decide the MCS index pair and corresponding SNR test point pair to define PDSCH NR-CA demodulation requirements. Type 2 UE NR-CA demodulation requirements. Received power difference; 25 dB. Received time difference; 33 µs. Channel model; AWGN (TS38.101-4 B.1.1). Maximum number of HARQ transmission; 1. Signal/Noise setting; Set both Es and Noc to set SNR test points. Throughput measurement procedure; Measure both carriers. Antenna configuration; 2x2 for both carriers. MCS table and MCS index; FFS. Test metric; 70% of the maximum throughput. Rank; MCS table; MCS index. Carrier with lower power; Rank 1; MCS table 1; Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37). Carrier with higher power; Rank 2; MCS table 2; Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86). Agreement: AWGN channel model - Confirm that the AWGN channel model in the WF last meeting means the static channel model defined in B.1.1 in TS 38.101-4. Agreement: CA combination - Select single bandwidth of 40 MHz for the requirement.

In this contribution we will deliver Nokia’s simulation results to be used in the companion discussion contribution [2] to enable final MCS pair decisions.

Please note that the TE TxEVM assumptions offline discussion is also treated in the companion discussion contribution [2].

Simulations

Setup

All other non-listed configurations are as Rel-15 CA requirements:

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| CBW | 40MHz |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| MCS table and MCS index | FFS |
| Test metric | 70% of the maximum throughput |

Table 1: Simulation setup

Summary

Among the MCS pairs given as options in the WF we have identified the following two pairs as the most acceptable:

Table 1: Two feasible MCS pairs (without TxEVM)

| Carrier with lower power Rank1 | Carrier with higher power Rank1 |
| --- | --- |
| Table 1 – MCS 4 | Table 2 – MCS 23 |
| -3.6 dB | 22.1 dB |
| Table 1 – MCS 4 | Table 2 – MCS 24 |
| -3.6 dB | 23.1 dB |

Table 3: Two feasible MCS pairs (with 3/6% TxEVM)

| Carrier with lower power Rank1 | Carrier with higher power Rank1 |
| --- | --- |
| Table 1 – MCS 4 | Table 2 – MCS 23 |
| -3.5 dB | 23.0 dB |
| Table 2=1 – MCS 4 | Table 2 – MCS 24 |
| -3.5 dB | 24.1 dB |

Full results

The full curves for TxEVM less case are as follows:

Figure 1: Relative TPUT curves.
Table 1 MCS (4) vs. Table 2 MCS (24).

Figure 1: Relative TPUT curves.
Table 1 MCS (4) vs. Table 2 MCS (23).

Conclusion

Within this contribution we presented the simulation results for demodulation requirements for non-colocated FR1 intra-band NR-CA. 
No observations or proposals have been made.

We discuss the results in our companion discussion document [2].

- References

- R4-2316917, WF on NonCol_intraB_ENDC_NR_CA_Demod, Ericsson, RAN4#108b.

- R4-2318350, Discussion on UE Demodulation for non-colocated FR1 intra-band NR-CA, Nokia, Nokia Shanghai Bell, RAN4#109.