3GPP TSG-RAN WG4 Meeting # 109	R4-2318392

Chicago, US, 13 – 17 November, 2023

Agenda Item:	8.6.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on UE RF requirements for FR2-1 UL 256QAM

Document for:	Approval

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is to specify the UE RF requirements. This topic was discussed at TSG RAN4#108 and the WF was agreed [2]. This contribution provides proposals on UE RF requirements for FR2-1 UL 256QAM according to the agreed WF and the related discussion [3].

# 2.	Discussion

2.1.	MRP requirements

The following tentative agreements were listed in the WF [2].

Issue 1-1-1: MRP requirements for PC1 of 29GHz Tentative Agreement: - The single CC MPR for PC1 in 256QAM operation shall be 3 dB more than that of 64QAM. Issue 1-1-2: MRP requirements for PC2 of 29GHz Tentative Agreement: - The single CC MPR for PC2 in 256QAM operation shall be 3 dB more than that of 64QAM. Issue 1-1-3: MRP requirements for PC5 of 29GHz Tentative Agreement: - The single CC MPR for PC5 in 256QAM operation shall be 3 dB more than that of 64QAM. Issue 1-1-4: MRP requirements for 39GHz Tentative Agreement: - The single CC MPR for PC1/2/5 in 256QAM operation shall be 4 dB more than that of 64QAM for 39 GHz band. How to capture this in the spec is FFS. Issue 2-1-2: How to capture MPR values for PC2/5 Tentative Agreement: - Introduce a new table into related clause of PC2 6.2.2.2 for 256QAM: Modulation; MPRWT, BWchannel. Inner RB allocations, Region 1; Edge RB allocations. DFT-s-OFDM; 256 QAM; TBD; TBD. CP-OFDM; 256 QAM; TBD; TBD.

The MPR simulation results provided by companies up to TSG RAN4#108bis, which are summarized in the tables in the topic summary [3] have shown that the MPR for UL 256 QAM would not exceed that of 64QAM by more than 3 dB using the average of the MPR simulation results from different companies. Therefore, we propose to agree on the above tentative agreements on MPR requirements in the WF.

On how to capture the MPR requirements for 39GHz in the specification, we propose to adopt the approach in the endorsed CR [4], i.e., specify △MPR of 1dB for 39GHz in a new table.

On how to capture MPR values for PC2/5, we propose to agree on the tentative agreement to introduce a new table into related clause of PC2 6.2.2.2 for 256QAM which can then be referred to for 256QAM with PC5.

2.2	EVM

The following tentative agreements were listed in the WF [2].

Issue 1-2-2: how to capture no PTRS in EVM test Tentative Agreement: - No further discussion. Issue 2-1-1: How to specify the EVM with limit MCS for UL256QAM Tentative Agreement: - No need limit in RAN4 Spec.

We see no need to limit PTRS or MCS choice in the gNB scheduler in the RAN4 specification, therefore, we propose to agree on the tentative agreements on EVM in the WF.

2.3	MPR requirements for intra-band CA

The following open issues were listed in the WF [2].

Issue 1-1-4: MPR requirements for intra-band CA - Proposals - Option 1: Intra-band CA MPRs for both, contig. and NC, and forPC1/2/5 in 256QAM operation are increased from their respective 64QAM values by 3 dB. - Option2: Others.

Since we have agreed to apply 3 dB and 4 dB delta for the other MPR requirements, it is reasonable to apply the same delta to intra-band CA MPR, i.e., we propose: Intra-band CA MPRs for both, contig. and NC, and for PC1/2/5 in 256QAM operation are increased from their respective 64QAM values by 3 dB for 29 GHz and 4 dB for 39 GHz.

# 3.	Conclusion

This contribution has provided proposals on UE RF requirements for FR2-1 UL 256QAM according to the agreed WF and the related discussion, they are summarized as follows:

Proposal 1: The single CC MPR for PC1 in 256QAM operation shall be 3 dB more than that of 64QAM.

Proposal 2: The single CC MPR for PC2 in 256QAM operation shall be 3 dB more than that of 64QAM.

Proposal 3: The single CC MPR for PC5 in 256QAM operation shall be 3 dB more than that of 64QAM.

Proposal 4: The single CC MPR for PC1/2/5 in 256QAM operation shall be 4 dB more than that of 64QAM for 39 GHz band.

Proposal 5: Specify △MPR of 1dB for 39GHz in a new table in the specification.

Proposal 6: Introduce a new table into related clause of PC2 6.2.2.2 for 256QAM which can then be referred to for 256QAM with PC5.

Proposal 7: No further discussion on how to capture no PTRS in EVM test.

Proposal 8: No need to specify the EVM with limit MCS for UL256QAM in RAN4 specification.

Proposal 9: Intra-band CA MPRs for both, contig. and NC, and for PC1/2/5 in 256QAM operation are increased from their respective 64QAM values by 3 dB for 29 GHz and 4 dB for 39 GHz.

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-2317596, “WF on FR2-1 UL 256QAM requirements”, Xiaomi.

[3]	R4-2317250, “Topic summary for [108-bis][126] FR2_enh_req_Ph3_part2”, Xiaomi.

[4]	R4-2317595, “Draft CR for Rel-18 38.101-2 to introduce FR2-1 UL 256 QAM RF requirements”, Xiaomi, Nokia, vivo, LG Electronics, ZTE, Qualcomm, Sony, MediaTek, Huawei, Apple, Ericsson.