3GPP TSG RAN WG4 Meeting #109		R4-2318411

Chicago, USA, November 13 – 17th 2023

Agenda item:	8.4.1.1

Source:	Apple

Title:	On specification options for enhanced channel raster

WI/SI:	NR_channel_raster_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e., the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33MHz. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1]. The SI was concluded at RAN#99 meeting with the general conclusions that two methods – overlapping channels from network perspective and the next larger channel – can be used to support irregular channels without requiring changes at the UE side. In addition to that, companies concluded that the existing NR channel raster design is not flexible and does not allow configuring certain NR channel combinations. In response to that RAN#99 agreed a new WI with the only main objective to enable a more flexible raster design [2].

During the previous RAN WG4 several solutions were discussed, for which at the end RAN WG4 concluded that a new 10kHz raster will be introduced as an explicit channel raster for both UE and gNB. In this discussion paper we consider several options on how the enhanced channel raster can be captured in the existing specifications.

# 2	Specification aspects of enhanced channel raster

## 2.1	UE side

Referring to TS 38.101-1, there is a table summarising applicable channel raster points (see Appendix A), where for every band the table specifies the starting and the ending point, and the step size. Thus, if the intention is to enable a more granular channel raster of 10kHz, then the easiest way is that the corresponding band step size is changed to 2 and raster is changed 10kHz. Table 2.1-1 below shows an example when for instance band n1 remains unchanged, but bands n5, n12, n26, n28, n29, n85 (as captured in TR 38.844) will have enhanced channel raster of 10kHz.

Table 2.1-1: Exemplary modifications to the channel raster table to enable 10kHz raster.

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n5 | 10 | 164800 – <2> – 169800 | 173800 – <2> – 178800 |
| n12 | 10 | 139800 – <2> – 143200 | 145800 – <2> – 149200 |
| n26 | 10 | 162800 – <2> – 169800 | 171800 – <2> – 178800 |
| n28 | 10 | 140600 – <2> – 149600 | 151600 – <2> – 160600 |
| n85 | 10 | 139600 – <2> – 143200 | 145600 – <2> – 149200 |

Observation 1a:	The simplest approach for introduction of the enhanced channel raster is to modify raster and step size for the corresponding bands.

Even though example in Table 2.1-1 is one of the simplest approaches requiring minimal changes to the specifications, it has several hidden issues. The main problem is that from the viewpoint of the enhanced channel raster feature, there will be several classes of bands:

a)	Legacy bands that have 100kHz raster and which do not need enhanced channel raster because there are no operator requests to enable a more granular raster.

b)	Legacy bands that have 100kHz raster, for which an operator requests to enable a more granular raster of 10kHz. As already discussed by RAN WG4, 10kHz will be obviously optional for earlier releases, i.e. 100kHz will be the only mandatory raster at least for earlier releases.

c)	Legacy bands that have 100kHz raster, for which an operator requests to enable a more granular raster of 10kHz that becomes mandatory starting from a certain release X. In other words, 100kHz is the only mandatory raster before release X, and only starting from release X also the 10kHz raster becomes the mandatory one.

d)	A completely new band, which can be agreed to have the enhanced raster of 10kHz from the moment when it is introduced to the specifications.

As can be seen, a simple approach fails to differentiate between different cases presented above. For instance, it will not be clear whether a certain band has 10kHz raster as a purely optional feature; and if it is mandatory, there is no way to indicate starting from which release it is mandatory.

Observation 1b:	The simplest approach (with just modifying raster and step size) fails to capture properly whether 10kHz raster is optional or mandatory for the given band; and if it is mandatory, then starting from which release.

To avoid any ambiguity, it is possible to consider a slightly revised version of the channel raster table as exemplified below in Table 2.1-2:

-	A new column is introduced to indicate explicitly the applicable step size, which depending on the band will be only 100kHz or both 100 and 10kHz. For a completely new band requiring enhanced channel raster the step size can be assumed to be only 10kHz.

-	To avoid duplicating data, columns with uplink and downlink range of N_ref can refer to the corresponding step size values.

-	If needed, it is possible to add additional qualifiers to indicate whether the given raster value is mandatory or optional for the band in the given version of the specification.

Table 2.1-2: Exemplary modifications to the channel raster table to enable 10kHz raster.

| NR operating band | NREF Step size S (kHz) | ΔFRaster (S x 5kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- | --- |
| n1 | 20 | 100 | 384000 – <S> – 396000 | 422000 – <S> – 434000 |
| n5 | 20, 2 | 100, 10 | 164800 – <S> – 169800 | 173800 – <S> – 178800 |
| n12 | 20, 2 | 100, 10 | 139800 – <S> – 143200 | 145800 – <S> – 149200 |
| n26 | 20, 2 | 100, 10 | 162800 – <S> – 169800 | 171800 – <S> – 178800 |
| n28 | 20, 2 | 100, 10 | 140600 – <S> – 149600 | 151600 – <S> – 160600 |
| n85 | 20, 2 | 100, 10 | 139600 – <S> – 143200 | 145600 – <S> – 149200 |
| nX | 2 | 10 | XXXX – <S> – YYYY | XXXX – <S> – YYYY |

Proposal 1:	Introduce changes to the raster table in TS 38.101-1 to indicate unambiguously which channel raster (100kHz and/or 10kHz) is applicable to a band, and whether it is mandatory or optional.

## 2.2	BS side

TS 38.104 also contains the table summarising channel raster points and step size for all bands supported by the base station specifications. Following the same line of reasoning as presented in section 2.1, the channel raster table in TS 38.104 can be extended so that it specifies unambiguously which band supports the 10kHz raster.  The only major difference comparing to the UE side is that it is up to the network implementation whether to deploy 100kHz or 10kHz raster and thus there is no need to capture whether the 10kHz raster is optional or mandatory for the given band.

Nevertheless, TS 38.104 should account whether the band and the UE supports the 10kHz raster. In other words, the network should not configure the 10kHz raster if the band does not support it; or the legacy band supports the 10kHz raster, but it is not supported by the UE.

Proposal 2a:	Introduce changes to the raster table in TS 38.104 to indicate unambiguously which channel raster (100kHz and/or 10kHz) is applicable to a band.

Proposal 2b:	Clarify in TS 38.104 that the network should not configure the 10kHz raster if it is not supported by the band or by the UE.

## 2.3	Other considerations

As a final aspect, it was agreed during the RAN4#108bis meeting that addition of the enhanced channel raster to the legacy bands will be based on the operator requests, which in turn may consider making it mandatory starting from a certain release. Since it is the question of enabling a new feature for legacy bands and even making it mandatory, there must be a clear and transparent process of making these requests so that it can be discussed properly by RAN WG4.

Proposal 3:	Introduction of the (mandatory) enhanced channel raster for the legacy band should follow the established process agreed by RAN WG4.

# 3	Conclusions

In this discussion paper we have presented our further considerations on the specification related aspects of the enhanced channel raster. As summarized in our paper, the easiest way to enable a more flexible raster is to specify a more granular step size in the table that specifies applicable raster points to the existing bands. However, this approach fails to capture properly whether the enhanced raster is optional or mandatory; and if it is mandatory, then starting from which release. We also suggest making similar clarifications to TS 38.104 so that it is clear for the network side whether 10kHz is configurable for the given band and a UE accounting for its capabilities.

Proposal 1:	Introduce changes to the raster table in TS 38.101-1 to indicate unambiguously which channel raster (100kHz and/or 10kHz) is applicable to a band, and whether it is mandatory or optional.

Proposal 2a:	Introduce changes to the raster table in TS 38.104 to indicate unambiguously which channel raster (100kHz and/or 10kHz) is applicable to a band.

Proposal 2b:	Clarify in TS 38.104 that the network should not configure the 10kHz raster if it is not supported by the band or by the UE.

Proposal 3:	Introduction of the (mandatory) enhanced channel raster for the legacy band should follow the established process agreed by RAN WG4.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson

RP-230813, "New WID on Channel raster enhancement", Ericsson

Appendix A: NR bands channel raster points

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n85 | 100 | 139600 – <20> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| n89 | 100 | 164800 – <20> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20> – 480000 | N/A |
| n98 | 100 | 376000 – <20> – 384000 | N/A |
| n99 | 100 | 325300 – <20> – 332100 | N/A |
| n100 | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. |  |  |  |