3GPP TSG RAN WG4 Meeting #109		R4-2318412

Chicago, USA, November 13 – 17th, 2023

Agenda item:	8.4.1.2

Source:	Apple, Ligado Networks, Inmarsat, Viasat, Globalstar, Thales, Hughes/Echostar

Title:	Enhanced channel raster for NTN bands

WI/SI:	NR_channel_raster_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e., the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33MHz. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1]. The SI was concluded at RAN#99 meeting with the general conclusions that two methods – overlapping channels from network perspective and the next larger channel – can be used to support irregular channels without requiring changes at the UE side. In addition to that, companies concluded that the existing channel raster design is not flexible and does not allow configuring certain NR channel combinations. In response to that RAN#99 agree a new WI with the only main objective to enable a more flexible raster design [2].

While most of the previous discussions for the irregular channels and flexible channel raster were conducted in the context of the TN bands, the same problem exists also for the NTN bands. In fact, this issue was already raised earlier during the SI on efficient usage of irregular spectrum blocks. During RAN4#106bis the follow contribution was submitted to RAN WG4 [3], based on which it was concluded that TN channel raster enhancements will be also applicable to NTN [4]. During the RAN4#108bis it was concluded to proceed with 10kHz as the enhanced channel raster for both TN and NTN bands. Furthermore, it was also agreed that based on the operator request the enhanced channel raster can be considered as mandatory feature including earlier releases.

# 2	Channel raster enhancements for the NTN bands

One of the main rationales for having channel raster enhancements also for the NTN bands was already presented in [3], for which RAN WG4 has already concluded that "any changes should also be applicable to NTN bands". Most of the FR1 NTN bands are "irregular" in a sense that their size is not aligned on the 5MHz boundary. As can be seen from the Table 2-1 below, only the satellite S-band, n256, is aligned on the 5MHz boundary. On the contrary to it, neither L-band (n255) nor L-/S-band (n254) are aligned on 5MHz. Furthermore, the so-called "extended" L-band size is also irregular occupying the 7MHz spectrum block. In other words, even if the L-band and the extended L-band are treated as one band, its total size is still irregular taking the 41MHz spectrum block.

Table 2-1: Summary of the NTN bands

| Satellite band | 3GPP band | Band size (MHz) |  |
| --- | --- | --- | --- |
| L-band | n255 | 34 | 41 |
| Extended L-band | [n253] | 7 |  |
| L-/S-band | n254 | 16.5 |  |
| S-band | n256 | 30 |  |

As concluded in TR 38.849, there are several methods that can be used to support irregular channels, one of which is overlapping channels from the network perspective. This approach is based on configuring two overlapping cells such that they form a contiguous set of RBs allowing the network to schedule data over all RBs. However, if the network has to configure both overlapping channels on the 100kHz raster time ensuring that the cells are RB aligned, then the least common multiplier between these requirements is 900kHz, i.e. 5RBs. For certain irregular channels it results in sub-optimal utilisation of available spectrum resources. In the Table 2-2 below we present the summary of potential gains that can be achieved for the irregular NTN spectrum blocks if a more flexible channel raster is enabled also for the NTN bands. For the sake of reference, we present how many RBs can be scheduled when both overlapping cells are on the 100kHz raster and when at least one of the cells can be on non-100kHz raster. As can be seen from the table, for almost all the irregular channels we can gain at least one extra RB; and for 16.5MHz and 34MHz channel we can gain even more than one RB.

Table 2-2: Summary of available RBs for overlapping channels in the irregular spectrum blocks.

| Irregular channel (MHz) | Next smaller channel (MHz) | Minimum guard band (kHz) | 100kHz raster |  |  | non-100kHz raster |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N_RB | Guard band (kHz) | Utilisation (%) | N_RB | Guard band (kHz) | Utilisation (%) |
| 7 | 5 | 242.5 | 35 | 342.5 | 90 | 36 | 252.5 | 92.6 |
| 16.5 | 15 | 382.5 | 84 | 682.5 | 91.6 | 87 | 412.5 | 94.9 |
| 34 | 30 | 592.5 | 180 | 792.5 | 95.3 | 182 | 612.5 | 96.4 |
| 41 | 40 | 552.5 | 221 | 602.5 | 97 | 221 | 602.5 | 97 |

Thus, enhanced channel raster of 10kHz will enable more flexible NTN deployments allowing even better spectrum utilisation comparing to the legacy 100kHz channel raster. There are two Rel-17 NTN bands – n255 and n256 – and one Rel-18 NTN band n254. To avoid a situation with the legacy UEs supporting only 100kHz raster, which would implicitly require the NTN operators to provide the default configuration aligned with the 100kHz raster, we suggest making the 10kHz raster as a mandatory feature for all NTN bands starting from Rel-17.

Proposal 1:	Enable the 10kHz raster as a mandatory feature for the Rel-17 NTN bands n255 and n256.

Proposal 2:	Enable the 10kHz raster as a mandatory feature for the Rel-18 NTN band n254.

# 3	Conclusions

In this discussion paper we elaborated further on the benefits of applying enhanced channel raster of 10kHz for the NTN FR1 bands. Following the agreements from the RAN4#108bis meeting, this discussion paper presents the operator request to enable the enhanced channel raster of 10kHz as a mandatory feature for the Rel-17 and Rel-18 NTN bands n254, n255 and n256.

Proposal 1:	Enable the 10kHz raster as a mandatory feature for the Rel-17 NTN bands n255 and n256.

Proposal 2:	Enable the 10kHz raster as a mandatory feature for the Rel-18 NTN band n254.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson

RP-230813, "New WID on Channel raster enhancement", Ericsson

R4-2304335, "Flexible channel raster for the NTN bands", Apple, Globalstar, Ligado Networks

R4-2306598, "WF on channel raster enhancement", Ericsson

RP-231180, "Revised WID on Channel raster enhancement", Apple, Globalstar, Ligado Networks, Inmarsat, Thales