3GPP TSG-RAN WG4 Meeting # 109		R4-2318417

Chicago, China, November 13 – 17, 2023

Agenda item:         7.1.1.1

Source:                  Apple

Title:                      MSD analysis for CA_n5B-n12,  CA_n5B-n14, and CA_n5B-n29 with CA_n5B UL

WI/SI:                    NR_CADC_R18_2BDL_xBUL

Release:                 Rel-18

Document for:      Discussion

# 1	Introduction

In the previous RAN4 meeting (#108-Bis, Xiamen, China), discussions were held on specifying the MSD values for the three CA combinations CA_n5B-n12, CA_n5B-14, and CA_n5B-n29 with CA_n5B UL. The test points were defined in the WF [1] and are captured in the table below:

# <Topic 1-2 CA_n5B_n12, CA_n5B_n14, and CA_n5B_n29 test points for evaluation > <Way forward>: The following test points are used for evaluation in next meeting: Band / Channel bandwidth / NRB / Duplex mode; Source of IMD. NR CA band combination; NR band; UL Fc (MHz); UL/DL BW (MHz); UL CLRB; DL Fc (MHz); MSD (dB); Duplex mode. CA_n5-n12; n5; 829; 10; 10 (RBSTART=0); 874; N/A; FDD; N/AX. 838.9; 10; 10 (RBSTART=42); 883.9; N/A; N/AY. n12; N/A; 5; N/A; 743.5; TBD; FDD; IMD9. CA_n5-n12; n5; 829; 10; 10 (RBSTART=0); 874; N/A; FDD; N/AX. 838.9; 10; 10 (RBSTART=36); 883.9; N/A; N/AY. n12; N/A; 5; N/A; 743.5; TBD; FDD; IMD11. CA_n5-n14; n5; 829; 10; 10 (RBSTART=0); 874; N/A; FDD; N/AX. 838.9; 10; 10 (RBSTART=42); 883.9; N/A; N/AY. n14; N/A; 5; N/A; 765.5; TBD; FDD; IMD7. CA_n5-n14; n5; 829; 10; 10 (RBSTART=0); 874; N/A; FDD; N/AX. 838.9; 10; 10 (RBSTART=28); 883.9; N/A; N/AY. n14; N/A; 5; N/A; 765.5; TBD; FDD; IMD9. CA_n5-n29; n5; 829; 10; 10 (RBSTART=0); 874; N/A; FDD; N/AX. 838.9; 10; 10 (RBSTART=42); 883.9; N/A; N/AY. n29; N/A; 5; N/A; 725.5; TBD; FDD; IMD11. CA_n5-n29; n5; 829; 10; 10 (RBSTART=0); 874; N/A; FDD; N/AX. 838.9; 10; 10 (RBSTART=37); 883.9; N/A; N/AY. n29; N/A; 5; N/A; 725.5; TBD; FDD; IMD13. NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified. NOTE: Only one MSD testpoint for each NR CA combination is specified based on the evaluation

Table 1-1: Test points for CA_n5B-12, CA_n5B-n14, and CA_n5B-29 from RAN4 # 108-Bis meeting

In this contribution paper, we will use the test points highlighted in the table above to derive the MSD requirements due to the IMDs generated by the intra band CA_n5B UL.

# Discussion

In the RAN plenary meeting #100 in Taipei, Taiwan, new LBLB inter-band BCS were introduced with CA_n5B as DL and UL configurations. The requested BCS can be seen in Table 2-1 below.

| CA combination | UL CA configuration | NR band | Channel BW (MHz) | Bandwidth combination set ( BCS ) |
| --- | --- | --- | --- | --- |
| CA_n5B-n12A | CA_n5B CA_n5A-n12A | n5 | CA_n5B_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n5B-n14A | CA_n5B CA_n5A-n14A | n5 | CA_n5B_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n5B-n29A | CA_n5B | n5 | 2 | 0 |
|  |  | n29 | 5, 10 |  |

Table 2-1: Requested BCS

The two UL CCs within the C_n5B UL configurations could generate IMDs that may affect the receiver of the victim bands. This therefore requires generating new MSD’s due to those IMDs, which is the goal of this of this discussion paper.

The IMD orders of interest are based on CA_n5B BCS0 request which is 20MHz maximum aggregated bandwidth:

- CA_n5B-n12A: n12 is subject to IMD9 and IMD11 of the two CA_n5B CCs RB allocations

- CA_n5B-n14A: n14 is subject to IMD7 and IMD9 of the two CA_n5B CCs RB allocation

- CA_n5B-n29A: n29 is subject to IMD11 and IMD13 of the two CA_n5B CCs RB allocation.

  - MSD Analysis

For the MSD analysis, we will rely on the 2-antenna architecture with triplexers which is the best compromise for size and performance, critical for UE implementations as was concluded during the study phase of the 700/800/900 WID [3] [4] [5] [6]. The three RF front-end diagrams for the 3 combos are captured in the figure below.

Figure 2.1-1: Front-end RF architecture for CA_n5B-n12A, CA_n5B-14A, and CA_n5B-n29A

The analysis will be based on the following worst-case assumptions for the RF front-end performance parameters:

- TX to RX isolation: 40dB

- Tx-Antenna Rejection: 35dB

- Antenna coupling: 10dB

- RxBW: 5MHz

- Output Power at antenna (PC3): 23dBm

- Post-antenna front-end loss: 4dB

Given the assumptions above, the MSD analysis using the test points and the RB allocation highlighted in Table 1-1 are summarized in the table below:

| Parameter | Unit | CA_n5B-n29 UL CLRB: 829MHz:10 (START 0); 838.9MHz:10 (START 42) |  | CA_n5B-n29 UL CLRB: 829MHz:10 (START 0); 838.9MHz:10 (START 37) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main | Main | Main | Div. |
| CBW | MHz | 5 |  | 5 |  |
| Simulated Tx noise @ PA Output | dBm/CBW | -46 | -56 | -32.0 | -42 |
| FE Loss | dB | 4.5 | 4.5 | 4.5 | 4.5 |
| Tx-Rx Iso / Tx-Ant rejection | dB | 40 | 37 | 40 | 37 |
| Tx noise @ Ant. | dBm | -86 | -93 | -72 | -79 |
| 3GPP 5MHz CBW REFSENS | dBm/CBW | -97.0 |  | -97.0 |  |
| SNR | dB | -1.0 | -1.0 | -1.0 | -1.0 |
| Thermal Noise | dBm | -96 | -96 | -96 | -96 |
| Composite Noise | dBm | -85.6 | -91.2 | -71.9 | -78.9 |
| Composite noise after MRC | dBm | -90.2 |  | -77.7 |  |
| MSD | dB | 6.7 |  | 19.3 |  |

Figure 2.1-1: MSD analysis for CA_n5B-n29A

| Parameter | Unit | CA_n5B-n12 UL CLRB: 829MHz:10 (START 0); 838.9MHz:10 (START 42) |  | CA_n5B-n12 UL CLRB: 829MHz:10 (START 0); 838.9MHz:10 (START 36) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main | Main | Main | Div. |
| CBW | MHz | 5 |  | 5 |  |
| Simulated Tx noise @ PA Output | dBm/CBW | -44 | -54 | -27.0 | -37 |
| FE Loss | dB | 4.5 | 4.5 | 4.5 | 4.5 |
| Tx-Rx Iso / Tx-Ant rejection | dB | 40 | 37 | 40 | 37 |
| Tx noise @ Ant. | dBm | -84 | -91 | -67 | -74 |
| 3GPP 5MHz CBW REFSENS | dBm/CBW | -97.0 |  | -97.0 |  |
| SNR | dB | -1.0 | -1.0 | -1.0 | -1.0 |
| Thermal Noise | dBm | -96 | -96 | -96 | -96 |
| Composite Noise | dBm | -83.7 | -89.8 | -67 | -74 |
| Composite noise after MRC | dBm | -88.7 |  | -72.8 |  |
| MSD | dB | 8.3 |  | 24.2 |  |

Figure 2.1-1: MSD analysis for CA_n5B-n12A

| Parameter | Unit | CA_n5B-n14 UL CLRB: 829MHz:10 (START 0); 838.9MHz:10 (START 42) |  | CA_n5B-n14 UL CLRB: 829MHz:10 (START 0); 838.9MHz:10 (START 28) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main | Main | Main | Div. |
| CBW | MHz | 5 |  | 5 |  |
| Simulated Tx noise @ PA Output | dBm/CBW | -41 | -51 | -21.0 | -31 |
| FE Loss | dB | 4.5 | 4.5 | 4.5 | 4.5 |
| Tx-Rx Iso / Tx-Ant rejection | dB | 40 | 37 | 40 | 37 |
| Tx noise @ Ant. | dBm | -81 | -88 | -61 | -68 |
| 3GPP 5MHz CBW REFSENS | dBm/CBW | -97.0 |  | -97.0 |  |
| SNR | dB | -1.0 | -1.0 | -1.0 | -1.0 |
| Thermal Noise | dBm | -96 | -96 | -96 | -96 |
| Composite Noise | dBm | -80.8 | -87.3 | -61 | -68 |
| Composite noise after MRC | dBm | -86.1 |  | -66.8 |  |
| MSD | dB | 10.8 |  | 30.2 |  |

Figure 2.1-1: MSD analysis for CA_n5B-n14A

# 3	Conclusion

In this contribution, we shared our analysis results of the MSD due to IMDs of CA_n5B UL for the LBLB combinations CA_n5B-n12A, CA_n5B-n14A and CA_n5B-n29A and make the following proposal.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | 8.3 | FDD | IMD9 |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 | N/A |  | N/AY |
|  | n12 | N/A | 5 | N/A | 743.5 | 24.2 | FDD | IMD11 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | 10.8 | FDD | IMD7 |
| CA_n5-n14 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 | N/A |  | N/AY |
|  | n14 | N/A | 5 | N/A | 765.5 | 30.2 | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=42) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | 6.7 | FDD | IMD11 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A | FDD | N/AX |
|  |  | 838.9 | 10 | 10 (RBSTART=37) | 883.9 | N/A |  | N/AY |
|  | n29 | N/A | 5 | N/A | 725.5 | 19.3 | FDD | IMD13 |
| NOTE X: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified NOTE Y: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified |  |  |  |  |  |  |  |  |

Table 3-1: Proposal for MSD based on 10RB+10RB UL allocation

# 4	References

R4-2317656, WF on cross-band MSD test point for FDD bands with intra-band contiguous CA

R4-2306470 WF on RF architecture and critical issues for CA_n28-n105, Apple

R4-2306469 WF on RF architecture and critical issues for CA_n5-n105, Xiaomi

R4-2306471 WF on RF architecture and critical issues for CA_n26-n28, Huawei

R4-2307546 Big CR for TR 38.872 to introduce outcomes of CA_n5-n105 CA_n28-n105 CA_n26-n28 in sub-1GHz SI, Huawei, Hisilicon