3GPP TSG-RAN WG4 Meeting #109		R4-2318418

Chicago, USA, November 13 – 17, 2023

Agenda item:       7.1.1.1

Source:                 Apple

Title:                     MSD analysis for CA_n5B

WI/SI:                   NR_CA_R18_intra-Core

Release:                Rel-18

Document for:     Discussion

# 1	Introduction

In the previous RAN4 meeting (#108-Bis, Xiamen, China), proposals were made on the MSD value for the CA combo CA_n5B, BCS1 (25MHz of total aggregated channel bandwidth). After discussions, an agreement was made to take the average of the MSD values and put it in brackets to allow additional inputs at RAN4 #109 as captured in the WF summary below [1]:

| Proposal | CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCS0 Ref | CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| Qualcomm | CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 43.4 | 20.6 | FDD |
| Murata | CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [42.5] | [18.2] | FDD |
| Skyworks | CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 46.4 | 26.2 | FDD |
| Average | CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [44.1] | [21.7] | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1 for both 5MHz + 20MHz and 10MHz + 15 MHz. Only 5MHz + 20MHz test point is defined. |  |  |  |  |  |  |  |  |

In this contribution paper, we will use the test points highlighted in the table above  to derive the MSD requirements for the due CA_n5B due  to the IMDs generated by the two intra-band CCs within the CA_n5B UL with a total aggregated CBW of 25MHz (5MHz +20MHz).

# Discussion

The requested bandwidth classes (BCS) for the CA_n5B is captured in table captured in Table 2-1 below:

| NR CA configuration | Uplink NR CA configurations | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n5B | CA_n5B | 5, 10, 15 | 5, 10, 15 | 20 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 | 25 | 1 |

Table 2-1: CA_n5B BCS

For BCS1 (5MHz+20MHz case),  SCC is victim of IMD3 overlap and PCC is victim of IMD5.

  - MSD Analysis

Noise levels measured in each CC Rx CBW (dBm/measurement bandwidth) and corresponding MSD levels are plotted in Figure 1-left and Figure 1-right respectively. MSD is evaluated using the following parameters:

- Power class 3 operation, Tx power level: 23dBm,

- Tx-Rx duplexer isolation: 50dB,

- Post-PA insertion losses: 4dB

- Primary antenna to diversity antenna isolation: 10dB

- SNR: -1 dB

Using the parameters above, the MSD are computed as numbers shown in the Table 2.1-1 below.

| Parameter | Unit | PCELL |  | SCELL |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main | Div. | Main | Div. |
| Rx CBW | MHz | 5 |  | 20 |  |
| Simulated Tx noise @ PA Output | dBm/CBW | -1 | -11 | -8 | -18 |
| Tx-Rx isolation | dB | 50 | 45 | 50 | 45 |
| LNA to Antenna loss | dB | 4 | 2 | 4 | 2 |
| Tx noise | dBm/CBW | -47 | -54 | -54 | -61 |
| 3GPP 5MHz CBW REFSENS | dBm/CBW | -98.0 |  | -86.8 |  |
| SNR | dB | -1.0 | -1.0 | -1.0 | -1.0 |
| Thermal Noise | dBm | -97 | -97 | -90.9 | -90.9 |
| Composite Noise | dBm | -47 | -54 | -54 | -61 |
| Composite noise after MRC | dBm | -54.8 |  | -61.9 |  |
| MSD | dB | 45 |  | 26 |  |

Table 2.1-1: CA_n5B IMD-based MSD analysis for PCELL and SCELL

# 3        Conclusions

In this contribution, we presented the results of our MSD analysis for the initial CA_n5B with 5MHz + 20MHz Tx BW combination and propose the following:

Proposal: Adopt CA_n5B MSD Levels proposed in Table 3-1 shown below.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 45 | 26 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

Table 3-1: CA_n5B MSD Levels for 5MHz +20MHz

# 4        References

[1] R4-2317654, WF on n5B BCS1, 3GPP TSG-RAN WG4 Meeting # 108-Bis, Xiamen, China