3GPP TSG-RAN WG4 Meeting #109		R4-2318419

Chicago, USA, November 13 – 17, 2023

Agenda item:       7.30.2.2

Source:                 Apple

Title:                     MSD analysis for CA_n5_n28_n105

WI/SI:                   NR_700800900_combo_enh-Core

Release:                Rel-18

Document for:     Discussion

# 1	Introduction

In the previous RAN4 meeting (#108-Bis, Xiamen, China), proposals were made on the MSD value due to IMD interferences for the CA combo CA_n5_n28_n105 and the WF was to use the test point shown in the Table below to derive the MSD values for the following meeting [1].

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | [22.9] | FDD | IMD3 |

Table 1-1: Test point for CA_n5-n28-n105 MSD Analysis

In this contribution paper, we will use the test point highlighted  above  to derive the MSD requirements for the due CA_n5_n28_n105 due  to the IMD interferences.

# Discussion

  - RF Front-end Architectures

The aggregated spectrum allocated for the CA_n5-n28-n105 is shown below in Figure 2.1-1.

Figure 2.1-1: Aggregated spectrum allocation for the CA_n5-n28-n105 combination

During the study phase for this band combination, several RF front-end architectures were considered [2]:

- Two-antenna architectures with the use of complex multiplexers ( especially pentaplexers and hexaplexers). Although only two low band antennas are kept due to the compact size of smartphone, UE has to face the difficulties of wide bandwidth low-band antenna and complex multiplexers.

- Three-antenna architectures using a quadplexer (n5UL/n5DL/n105DL/n105UL) with moderate complexity compared to the multiplexers used for the two-antenna architecture cases.

- Four-antenna architectures with simpler RF filtering implementation. However, the use of four antennas creates implementation challenges for the UE.

The three-antenna architecture seems to provide the best compromise for RF front-end design complexity and UE implementation. Therefore, we will use the architecture shown in Figure 2.1-2 below.

-	1) n5UL+n5DL+n105UL+n5DL quadplexer is routed through the  main antenna#1 (Main TRX1)

-	2) n28A UL + n28A DL duplexer and  n28B UL + n28B DL duplexer are routed through the main antenna#2 (Main TRX2)

-   The diversity antenna is used for  n5DL n28DL+n105UL.

Figure 2.2-1: Three-antenna based architecture the CA_n5-n28-n105 combination

  - Channel Bandwidths

Table 2.2-1 displays the CBWs for CA_n5A-n28A-n105A.

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n28A-n105A | CA_n5A-n105A CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 2.2-1: CA_n5A-n28A-n105A BCS0

  - MSD Analysis

As was concluded in [2], there is no intermodulation (IMD) product which falls into the Rx frequency range of the third DL band n28 when CA_n5-n105 is the UL configuration. However, 3rd order IMD products may fall into the Rx frequency range of the third DL band n105 when CA_n5-n28 is the UL configuration.

For the MSD analysis, we will use the following worst-case assumptions for the RF front-end performance parameters:

We will use the commonly used linearity assumptions shown in Table 2.3-1 [2] and the RF front-end rejection performance highlighted in Table 2.3-2 below: As can be seen from Table 2.3-1, the PA and the LNA are the most dominant contributors for the linearity. PC3 is assumed.

| Component | IP3 (dBm) |
| --- | --- |
| LNA | -6 |
| PA Forward IP3 | 30 |
| PA reverse IP3 | 28 |
| Diplexer | 80 |
| Multiplexer | 75 |
| Antenna switch | 70 |

Table 2.3-1: Linearity (IP3) assumptions

| Parameter | Value (dB) |
| --- | --- |
| Antenna isolation | 10 |
| FE losses | 4 |
| PCB isolation | 55 |
| n5 Tx and n28 Tx attenuation in n105 Rx band | 33 |
| n105 Rx filter attenuation in ns Tx band | 43 |

Table 2.3-2: Front-end rejection assumptions

Given the assumptions above, the analysis for the MSD due to IMD3  is summarized in the table below:

| Parameter | Unit | Main Ant. | Div. Ant. |
| --- | --- | --- | --- |
| n5 PA IMD3 | dBm/5MHz | -93.5 |  |
| n28 PA IMD3 | dBm/5MHz | -72.6 |  |
| n105 LNA IMD3 | dBm/5MHz | -64.5 | -74.5 |
| Composite IMD3 | dBm/5MHz | -63.9 | -74.5 |
| 3GPP 5MHz CBW REFSENS | dBm | -97.2 | -97.2 |
| SNR | dB | -1 | -1 |
| Thermal noise | dBm | -96.2 | -96.2 |
| Composite noise | dBm | -63.9 | -74.5 |
| After MRC | dBm | -72.8 |  |
| MSD value | dB | 24.3 |  |

Table 2.1.1-2: CA_n5-n28-n105 MSD analysis with CA_n5A-n28A UL

# 3        Conclusions

In this contribution, we presented the results of our MSD analysis for CA_n5-n28-n105 with CA_n5A-n28A UL and proposed the following:

Proposal: Adopt CA_n5-n28-n105 MSD value due to IMD3 for n105 with CA_n5A-n28A configured for UL as shown in the Table below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 24.3 | FDD | IMD3 |

Table 3-1: Proposed MSD value for r CA_n5-n28-n105 with CA_n5A-n28A UL

# 4        References

[1] R4-2317589, WF on NR_700800900_combo_enh, 3GPP TSG-RAN WG4 Meeting # 108-Bis, Xiamen, China

[2] R4-2314659, CR for TR 38.872 to introduce some potential RF impacts for CA_n5-n28-n105, 3GPP TSG-RAN WG4 Meeting # 108, Toulouse, France