3GPP TSG RAN WG4 Meeting #109		R4-2318432

Chicago, USA, November 13 – 17th 2023

Agenda item:	8.15.2.1

Source:	Apple, Vodafone, T-Mobile USA

Title:	On UL MIMO test methodology

WI/SI:	NR_FR1_TRP_TRS_enh-Core

Release:	Rel-18

Document for:	Decision

# Introduction

In the previous RAN4 meeting it was agreed on [1] that the test methodology based on sweep TPMI Index Options 1 and 2 postprocessing should be further evaluated

Option 1  consists on the averaging of TRPs measured with different TPMI Indexes.

Option 2 is based on the selection of maximum EiRP from all available TPMI Indexes per spherical coordinate, and calculate the TRP based on such maximum EiRP selection. Option 2 had been extensively documented on [2-6].

While Option 2 is based on realistic network driven TPMI Index optimization, Option 1 has no basis on realistic UE operation.

# 2	Graphical representation of sweep TPMI Index

For clarity and proper understanding of Option 2 fundamental proposal, the Figure 1a – 1d brings a graphical representation of the optimal maximum EiRP selection process based on multiple TPMI Index related measurements.

Such graphical representation is valid for both coherent TPMI index 2-5 (0, 90, 180 or  270 sets), and non-coherent scenarios TPMI Indexes 0, 1and 2.

Figure 1a, TPMI Index 0, EiRP exemplification                     Figure 1b, TPMI Index 1, EiRP exemplification

Figure 1c, TPMI Index 2, EiRP exemplification                Figure 1d, Sweep (Dynamic) TPMI Index combined Maximum EiRP

# 3	Results and discussions

The results on Table 1 were collected on real device, these are Free Space and BHHR TRP measurement results taken on a device capable of Power Class 1.5:

- TPMI Index 0, is related to a single antenna (1) transmitting at 26 dBm

- TPMI Index 1, is related to a single antenna (2) transmitting at 26 dBm

- TPMI Index 2, is related to both antennas (1 and 2) transmitting simultaneously at 26 dBm

Table 1, summary of Option 1 and Option 2 post-processing results

Observation 1:	TRP calculated based on the average of TPMI Indexes 0, 1 and 2 produces TRP which is lower than Fix TPMI Index 2, or in many cases lower than an individual dominant antenna..

Observation 2:	The averaging of EiRP produces even more pessimistic TRP in cases where there's a large unbalance between individual antennas TRP, e.g.  EU1 n41 BHHR (high channel) where the average TRP is 1.41 dB lower than TPMI Index 2 TRP, while TRP based on the max EiRP is 0.25 higher.

.

Observation 3:	Averaging TRP (or EiRP) among TPMI Indexes 0, 1 and 2 produces over-pessimistic and unrealistic TRP results. Such proposal can’t be supported by any technical argument, such condition has no base on real field/network behavior.

Observation 4:	The measurement  results presented in this contribution are conservative, considering that were gathered in a single device, and this device antenna system was not built/designed to showcase the sweep (Dynamic) TPMI Index methodology. Larger variation between fix and sweep (Dynamic) TPMI Indexes should be possible in large devices sampling

# 5	Observations and Proposals

Observation 1:	TRP calculated based on the average of TPMI Indexes 0, 1 and 2 produces TRP which is lower than Precoding 2 or in many cases lower than an individual dominant antenna.

Observation 2:	The averaging of EiRP produces even more pessimistic TRP in cases where there's a large unbalance between individual antennas TRP, e.g.  EU1 n41 BHHR (high channel) where the average TRP is 1.41 dB lower than TPMI Index 2 TRP, while TRP based on the max EiRP is 0.25 higher.

.

Observation 3:	Averaging TRP (or EiRP) among TPMI Indexes 0, 1 and 2 produces over-pessimistic and unrealistic TRP results. Such proposal can’t be supported by any technical argument, such condition has no base on real field/network behavior.

Observation 4:	The measurement  results presented in this contribution are conservative, considering that were gathered in a single device, and this device antenna system was not built/designed to showcase the sweep (Dynamic) TPMI Index methodology. Larger variation between fix and sweep (Dynamic) TPMI Indexes should be possible in large devices sampling.

Proposal 1:	RAN4 shall not consider Option 1 (average TRP) when post-processing the sweep (Dynamic) TPMI Index.

Proposal 2:	RAN4 shall consider Option 2 (TRP calculation considering Maximum EiRP) as the only sweep (Dynamic) TPMI Index post-processing methodology for performance requirement definitions.

# 6	References

[1]		R4-2317004, “WF on NR_FR1_TRP_TRS_enh”, vivo, 3GPP RAN4 #108bis

[2]		R4-2315382, “On multi-Tx radiated output power methodologies”, Apple, 3GPP RAN4 #108bis

[3]		R4-2311227, “UL MIMO radiated output power metric and test methodology,” Apple, 3GPP RAN4 #108, August 2023

- [4]	R4-2309055, “UL MIMO radiated output power metric and test methodology,” Apple, 3GPP RAN4 #107, May 2023

- [5]	R4-2304343, “On TRP TRS test methodology,” Apple, 3GPP RAN4 #106bis, April 2023

- [6]		R4-2300349, “Test methodology for radiated power of UL MIMO capable devices,” Apple, 3GPP RAN4 #106, February 2023