3GPP RAN WG4 Meeting #109		R4-2318434

Chicago, USA, November 13th – 17th, 2023

Agenda item:	7.30.2.5

Source:	Apple

Title:	MSD analysis for UL CA_n26(2A)

WI/SI:	NR_700800900_combo_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

CA_n26(2A) with UL configuration same as in DL is the latest addition to the “enhancement on 700/800/900 band combinations” work item which was approved in RAN #101 meeting [1]. The combination is expected to be subject to odd-order UL intermodulation (IMD3, IMD5, IMD7, ...) self-interference to both DL carriers where MSD from IMD3 and IMD5 direct hit could potentially be relatively high for single-PA implementation, as was observed from our preliminary analysis in last RAN4 meeting [2]. To harmonize the MSD analysis among companies, a specific test configuration based on dual-PA implementation was proposed and agreed in last RAN4 meeting which was captured in the approved WF on NR_700800900_combo_enh [3]. In this contribution, we provide our further MSD analysis according to the agreed test configuration for both single-PA and dual-PA implementations based on numerical simulations and link analysis.

# Discussion

Figure 2-1 depicts the UL/DL carrier configuration agreed in last RAN4 meeting for MSD analysis where the carrier spacing has been chosen as half of the duplex spacing such that IMD3 and IMD5 will have a direct hit to DL PCC and SCC or vice versa.

Figure 2-1 CA_n26(2A) test configuration with IMD3 and IMD5 direct hit to DL carriers

The Tx interference noise power to DL PCC and SCC frequency ranges at the PA output was evaluated based on numerical simulations where the simulation assumptions are summarized in Table 2-1.

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx and PA model | Same as used for MPR/A-MPR simulations | CIM3, CIM5, IRR, etc. |
| Total Tx power | 23 dBm | At antenna port |
| FE insertion loss | 4 dB |  |
| Channel BW | PCC UL/DL: 5/5; SCC UL/DL: 5/5 | MHz |
| UL configuration | PCC: 12RB RBStart =0; SCC: 12RB RBStart = 3 |  |

Table 2-1 Simulation assumptions for CA_n26(2A) Tx interference noise power to DL PCC and SCC

Figure 2-2 shows the simulated spectral profile at n26 PA output (single-PA) where the ranges highlighted in light green are the DL carrier allocations. The IMD3 and IMD5 direct hit to DL carriers can be observed via the second and the third noise humps next to the right UL carrier.

Figure 2-2 Simulated spectral profile at n26 PA output (single-PA) for UL CA_n26(2A)

The simulated Tx noise power in DL PCC and SCC ranges was then incorporated in link analysis for MSD calculations where further assumed parameters are summarized in Table 2-2.

| Parameter | Value | Comment |
| --- | --- | --- |
| FE insertion loss | 4 dB |  |
| Duplexer isolation | 50 dB | Tx to Rx |
|  | 42 dB | Tx to ANT |
| Antenna isolation | 10 dB | Main to diversity |
| MRC |  | Uncorrelated |

Table 2-2 Assumed parameters for CA_n26(2A) MSD link analysis

In the MSD link analysis, apart from the Tx noise generated from PA nonlinearity, other RF impairments such as passive components IMD and LNA IMD were also considered which however have much less noise contribution than PA nonlinearity. In the case of dual-PA implementation with one Tx for each UL carrier, the MSD would be dominated by the PA reversed IMD. The PA reverse IP3 was assumed 2dB worse than forward IP3 and reverse IP5 was assumed 1dB better than forward IP5 which was based on commercially available PA bench characterizations.

Observation 1: Among all the RF impairments contributing to MSD, Tx noise generated from PA nonlinearity dominates.

Observation 2: In the case of dual-PA implementation with one Tx for each UL carrier, the MSD would be dominated by the PA reversed IMD.

Table 2-3 summarizes the calculated MSD results for the CA configuration as shown in Figure 2-1.

| UE Architecture | PCC MSD (IMD3) | SCC MSD (IMD5) |
| --- | --- | --- |
| Single-PA | 48.9 dB | 32.3 dB |
| Dual-PA | 35.2 dB | 1.0 dB |

Table 2-3 Calculated MSD results for CA_n26(2A) configuration as shown in Figure 2-1

It can be seen that the MSD caused by IMD3 is quite severe which may render the combination not being so useful. On the other hand, though the dual-PA architecture can help improve MSD due to IMD5 substantially, the IMD3 MSD is still relatively high which may not justify the increased cost and RF front-end complexity for dual-PA implementation.

Observation 3: For UL/DL CA_n26(2A), the MSD caused by IMD3 is quite severe which may render the combination not being so useful.

Observation 4: Though the dual-PA architecture can help improve MSD due to IMD5 substantially, the IMD3 MSD is still relatively high which may not justify the increased cost and RF front-end complexity for dual-PA implementation.

Based on the above analysis, we propose RAN4 to reconsider whether there is sufficient technical justification to support UL CA_n26(2A) as its DL carriers are susceptible to relatively high MSD.

Proposal 1: RAN4 to reconsider whether there is sufficient technical justification to support UL CA_n26(2A) as its DL carriers are susceptible to relatively high MSD.

Proposal 2: If RAN4 would proceed to specify UL/DL CA_n26(2A), take the MSD values in Table 2-3 into consideration.

# 3	Conclusion

In this contribution, we provide our further MSD analysis according to the agreed test configuration for both single-PA and dual-PA implementations based on numerical simulations and link analysis.

Observation 1: Among all the RF impairments contributing to MSD, Tx noise generated from PA nonlinearity dominates.

Observation 2: In the case of dual-PA implementation with one Tx for each UL carrier, the MSD would be dominated by the PA reversed IMD.

Observation 3: For UL/DL CA_n26(2A), the MSD caused by IMD3 is quite severe which may render the combination not being so useful.

Observation 4: Though the dual-PA architecture can help improve MSD due to IMD5 substantially, the IMD3 MSD is still relatively high which may not justify the increased cost and RF front-end complexity for dual-PA implementation.

Proposal 1: RAN4 to reconsider whether there is sufficient technical justification to support UL CA_n26(2A) as its DL carriers are susceptible to relatively high MSD.

Proposal 2: If RAN4 would proceed to specify UL/DL CA_n26(2A), take the MSD values in the table below into consideration.

| CA Combination | UE Architecture | PCC MSD (IMD3) | SCC MSD (IMD5) |
| --- | --- | --- | --- |
| UL/DL CA_n26(2A) | Single-PA | 48.9 dB | 32.3 dB |
|  | Dual-PA | 35.2 dB | 1.0 dB |

# 4	References

RP-232680 “Revised WID on enhancement for 700/800/900MHz band combinations”, CATT, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023

R4-2315389 “MSD analysis for UL CA_n26(2A)”, Apple, 3GPP RAN WG4 Meeting #108bis, Xiamen, China, October 9th – 13th, 2023

R4-2317589 “WF on NR_700800900_combo_enh”, CATT, 3GPP RAN WG4 Meeting #108bis, Xiamen, China, October 9th – 13th, 2023