3GPP TSG RAN WG4 #109 meeting 	 	R4-2318444

Xiamen, China, 09th – 13th October, 2023

Agenda Item:	8.3.1.4

Source:	Meta Ireland

Title:	TP on TR38.881 v0.7.0 to update contents based on agreements

Document for:	Approval

- Introduction

In RAN4 #108bis meeting, RAN4 agreed the main open issues such as MSD type and order and signalling overhead issues. Based on agreed WFs [1][2][3], we propose to update the FFS and TBD in some sub-clauses as follow:

Text Proposal

<<<<<<<<<<< Start of changes in section 7.1.2 >>>>>>>>>>

7.1.2	Candidate solution 2: MSD reporting for frequency regions affected by self-interference

#### 7.1.2.1	Basic MSD information unit

Based on the survey of the existing MSD specifications, the essential MSD information for a band combination includes: the cause of the MSD (such as harmonic interference, intermodulation, cross-band isolation and etc), the aggressor band(s), the victim band and the amount of MSD in dB.

The first three pieces of information are correlated. After reducing the redundancy, the basic MSD information unit can be escribed as a 3-tuple of <MSD value, MSD source, Victim band >.

The details of the basic MSD information unit are shown in Table 7.1.2.1-1 below. Note that MSD=0dB indicates no degradation of the receiver sensitivity.

Table 7.1.2.1-1: Basic MSD Information Unit for a band combination

| MSD 3-tuple | <MSD value, MSD source, Victim band> |
| --- | --- |
| MSD Value | The index within the set of {MSD=0dB, MSD≤Th1 dB, MSD≤Th2 dB, …, MSD≤Thi dB, …, MSD≤ThM-1 dB }, where M is the total number of thresholds |
| MSD Source | The index within the set of {Harmonic, Harmonic mixing, cross-band ISO, IMDn (n=2, …, K)}, where K is the highest IMD order up to 5 in Rel-18. [Handling of order for Harmonic and Harmonic mixing is FFS]. |
| Victim Band | Band number or the band index within the DL band combination |

More explicitly, as per the latest 3GPP specifications, the MSD sources can be summarised in Table 7.1.2.1-2 below. However, not all the details are to be reported in order to reduce the signalling overhead.

Table 7.1.2.1-2: MSD sources

| Interference source | 3GPP notations |
| --- | --- |
| UL harmonic | UL2/DL1, UL3/DL1, UL4/DL1, UL5/DL1 (Only direct-hit is counted.) |
| Harmonic mixing | UL1/DL2, UL1/DL3, UL1/DL4, UL1/DL5, UL2/DL3, UL3/DL2, UL4/DL3 |
| Intermodulation | IMD2, IMD3, IMD4, IMD5, IMD7, IMD9 |
| Cross-band Isolation | ACLR1, ACLR2, >ACLR2 |

In the following, some examples are given to demonstrate the basic MSD information to be reported by a UE. For different types of MSD sources, the MSD information units to be reported are shown in Table 7.1.2.1-3.

Table 7.1.2.1-3: Examples of MSD information unit.

| MSD in TS 38.101-1 | MSD information unit |
| --- | --- |
|  | For CA_n1-n3: <MSD≤25, IMD3, n1> For CA_n1-n78: <MSD≤10, IMD4, n1> For CA_n3-n78: <MSD≤30, IMD2, n3> <MSD≤10, IMD4, n3> <MSD≤5, IMD7, n3> |
|  | For CA_n1-n3-n78: <MSD≤30, IMD2, n78> <MSD≤15, IMD4, n78> <MSD≤30, IMD2, n3> |
|  | For CA_n3-n78: <MSD≤25, Harmonic, n78> |
|  | For CA_n1-n3: <MSD≤20, Cross-band ISO, n3> |

Note that for the purpose of MSD improvement, a UE should perform better than the minimum requirement in order to claim the low-MSD capability. For example, if the minimum requirement falls into the interval of MSD≤Thi dB, the UE should be able to support at least one-level lower (e.g., MSD≤Thi-1 dB) in order to report the low-MSD capability. If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case.

<<<<<<<<<<< Unchanged sections are omitted >>>>>>>>>>

###### 7.1.2.2.3.2	Effect of introducing network preferred minimum Lower MSD class as one of the filtering conditions

It has already been agreed that the following MSD thresholds would be defined as following in Table 7.1.2.2.3.2-1 :

Table 7.1.2.2.3.2-1: Actual lower MSD capability signalling index

| Index | Maximum allowed actual MSD (i.e. Thresholds) | Lower MSD Capability classes | Note |
| --- | --- | --- | --- |
| 0 | 0dB | Ⅰ | No degradation |
| 1 | 3 dB | Ⅱ | Actual MSD ≤ 3dB |
| 2 | 6 dB | Ⅲ | Actual MSD ≤ 6dB |
| 3 | 9 dB | IV | Actual MSD ≤ 9dB |
| 4 | 12 dB | Ⅴ | Actual MSD ≤ 12dB |
| 5 | 15 dB | Ⅵ | Actual MSD ≤ 15dB |
| 6 | 18 dB | Ⅶ | Actual MSD ≤ 18dB |
| 7 | 22dB | Ⅷ | Actual MSD ≤ 22dB |

However, there are still views that this granularity is much finer than the network really needs, depending on how to use this capability by the network. For example, the network may simply use one MSD value as a threshold to set different scheduling method for a particular band / BC, and some minor refinements such as a few dB in relative higher region, e.g 18dB -> 15 or 12 dB, may not that meaningful for the network. In this case, even the UE may be qualified for “Lower MSD” in a certain scenario including band/MSD type etc, the network may simply treat this UE the same to a UE which only satisfy minimum MSD requirement, and the signalling of Lower MSD for this particular scenario, might actually not be needed by the network.

In another word, depending on how the network would utilize Lower MSD capability class, the lower MSD capable UE for a certain scenario may not be differentiated by the network with UE only satisfy minimum MSD requirements, particularly if the lower MSD capability class is relatively large.

Based on this condition, a network preferred minimum Lower MSD capability class can be considered as another filter parameter for reducing capability reporting. For example, if network would only consider a maximum allowed 6dB MSD for harmonic interference for n3-n78 harmonics interference for n78, it can consider using the lower MSD capability class as a filtering parameter, and set up the maximum allowed report class as III (index 2), which corresponds to a maximum allowed actual MSD of 6dB. In this case, the UE would report the lower MSD class for this scenario if it can satisfy class I or II or III. If the UE support lower MSD but can only achieve lower MSD class larger than III, the UE would not report the capability since anyway the network would treat it the same to UE only support minimum requirements.

In this way, the signalling overhead is saved based on network needs. Though the UE may lose the opportunity to report in the condition, those reporting would anyway be meaningless since network would no use it and explicitly indicted. Furthermore, the whole scheme is well controlled by the network.

### 7.1.3	Candidate solution 3: MSD reporting for frequency regions affected by self-interference

#### 7.1.3.1	Basic MSD information unit

Based on the survey of the existing MSD specifications, there are types of MSD mechanism includes: uplink(aggressor) harmonic, receiver harmonic mixing, cross-band isolation, intermodulation and etc. The aggressor power class also have direct impact on the victim band and the amount of MSD in dB.

The improved lower MSD capability signalling shall include its mechanism, aggressor order, aggressor power class and improved MSD values per victim band. The lower MSD capability signalling can be specified as per victim band per BC as a 2-tuple of < MSD mechanism/Aggressor power class and its order, MSD value >. This can save 1-tuple signaling overhead on adaptive network signalling approach comparing to 3-tuple signalling element.

Some examples are given below to demonstrate the improved MSD reporting information for different types of MSD mechanisms. Table 7.1.3.1-1 referred from [3] for ease of understanding. For one mechanism same aggressor power class, order and victim order, only the worst MSD value after improvement is reported to reduce unnecessary signalling overhead. All the MSD in existing specs are within the scope of lower MSD capability reporting. Further, if new MSD type is identified and specified by RAN4 in the future, the new MSD type can also be considered for indicating lower MSD capability.

UE would not be allowed to report the lower MSD capability if there’s no improvement.

Table 7.1.3.1-1: Information element examples of improved MSD reporting per BC for non-threshold based approach

| MSD in TS 38.101-1 | MSD information unit |
| --- | --- |
|  | For CA_n1-n3: Victim n1: <PC3IMD3, 17> For CA_n1-n78: Victim n1: <PC3IMD4, 4> For CA_n3-n78: Victim n3: <PC3IMD2, 19> Victim n3: <PC3IMD4, 4> Victim n3: <PC3IMD7, 0> |
|  | For CA_n1-n3-n78: Victim n78: <PC3IMD2, 19> Victim n78: <PC3IMD4, 6> Victim n3: <PC3IMD2, 19> |
|  | For CA_n3-n78: Victim n78: <PC3UL2DL1, 17> |
|  | For CA_n1-n3: Victim n3: <PC3XB, 11> |

An alternative expression on the information element of low-MSD reporting can also be a few bits index that pointed to some actual MSD values like the examples below,

Table 7.1.3.1-2: Information element examples of improved MSD reporting per BC for threshold-based approach

| MSD in TS 38.101-1 | MSD information unit |
| --- | --- |
|  | For CA_n1-n3: Victim n1: <PC3IMD3, index 7> For CA_n1-n78: Victim n1: <PC3IMD4, index 2> For CA_n3-n78: Victim n3: <PC3IMD2, index 7> Victim n3: <PC3IMD4, index 3> Victim n3: <PC3IMD7, index 2> |
|  | For CA_n1-n3-n78: Victim n78: <PC3IMD2, index 7> Victim n78: <PC3IMD4, index 2> Victim n3: <PC3IMD2, index 7> |
|  | For CA_n3-n78: Victim n78: <PC3UL2DL1, index 6> |
|  | For CA_n1-n3: Victim n3: <PC3XB, index 4> |

where the reported index can be pointed to an agreed MSD values in Table 7.1.2.2.3.2-1.

Reference

[1] R4-2303695, “WF on study for lower MSD,” Huawei

[2] R4-2314923, “WF on lower MSD,” Huawei

[3] R4-2317601, “WF on lower MSD for inter-band CA/EN-DC/DC combinations,” Huawei

[4] RP-230753, “Revised WID on Further RF requirements enhancement for NR and EN-DC in FR1,” Huawei

[5] TR38.881: Study on lower MSD for inter-band CA/EN-DC/DC combinations

[6] TS38.101-1

[7] TS38.101-3