# 3GPP TSG-RAN WG4 Meeting # 109	R4-2318461

# Chicago, US, November 13 – 17, 2023

Agenda item:			8.26.6.3

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for NR NTN mobility enhancement

Document for:	Discussion

# Introduction

In this contribution, our views on the mobility enhancement impact on RRM requirement are provided.

# IDLE: cell reselection enhancements

### TN-to-NTN Cell reselection

### Issue 4-1: TN to NTN cell reselection

Agreement:

- FFS: whether/how to define TN to NTN cell reselection.

As discussed in RAN4#108bis [1]:

In our understanding, skipping NTN measurements based on TN coverage information has not yet been covered by RAN2, and it seems no strong motivation to specify the requirement if the UE may or may not perform measurements. Considering the scenario for TN-to-NTN is much infrequent comparing to NTN-to-TN, TN-to-NTN cell re-selection requirement can be deprioritized or not pursued.

Proposal 1: Deprioritized or not pursue TN-to-NTN reselection requirements.

### NTN-to-TN Cell reselection

### Issue 4-2: NTN to TN cell reselection

Agreement:

- UE is allowed to skip TN neighbour cells measurement in an area where there is no coverage of the frequency based on the provided TN cell coverage information and UE GNSS position information. FFS whether and how to implement it RAN4 CR.

- FFS on how to enhance NTN-to-TN cell reselection in case of mismatch between practical TN cell coverage and TN cell coverage information provided by serving cell.

Regarding TN coverage provision, whether to define UE behavior when the UE is out of TN coverage, as discussed in WF [1]:

In our understanding, the TN network coverage is provided with frequency association. Thus, for a given frequency, the NTN UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. And we think a simple clarification note is sufficient.

Proposal 2: (Option A) RAN4 to not update the existing UE requirements other than adding the clarification that UE is not required to perform TN cell measurement in an area where there is no TN network coverage on a given frequency layer based on the provide TN cell coverage information and UE GNSS position information.

# Summary

The observations and proposals in this contribution are summarized:

Proposal 1: Deprioritized or not pursue TN-to-NTN reselection requirements.

Proposal 2: (Option A) RAN4 to not update the existing UE requirements other than adding the clarification that UE is not required to perform TN cell measurement in an area where there is no TN network coverage on a given frequency layer based on the provide TN cell coverage information and UE GNSS position information.

# Reference

- R4-2317374, WF on RRM WF and CR work split for NR_NTN_enh, RAN4#108bis, Oct. 2023.