3GPP TSG-RAN4 Meeting #109	R4-23xxxxx

# Chicago, USA, November 13 – 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-18 feature list

Agenda Item:	10

Document for:	Discussion

Introduction

In this contribution we would like to provide our view on Rel-18 UE feature list.

Discussion

Update to feature list is proposed for following features.

# 27. Further RF requirements enhancement for NR and EN-DC in FR1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27.NR_ENDC_RF_FR1_enh2 | 27-1 | Support TxD for 4Tx | Support per FS TxD capability for 4Tx from Rel-18 |  | Yes | N/A | No capability is available to indicate Tx diversity for 4Tx | Per FS | N/A | Applicable to FR1 only | N/A | see LS in R4-2317617 | Optional with capability signalling |
| 27.NR_ENDC_RF_FR1_enh2 | 27-2 | LowerMSD for inter-band NR CA and EN-DC | Capability to indicate better MSD performance than the specified minimum requirements. The essential information of this capability includes: - victim band and aggressor band(s) of the band combination - MSD type - Lower-MSD capability class - power class |  | Yes |  | The UE shall comply with the minimum requirements for MSD. | Per BC Note: This does not prevent the capability is reported in a “per victim band” way as described in RAN2 LS R2-2311586. | No | Applicable to FR1 only | Support mixture of FDD/TDD | Detailed information can refer to the LS to RAN2 in R4-2312247, R4-2310276, R4-2306594 and R4-2315238. | Optional with capability signalling |
| 27.NR_ENDC_RF_FR1_enh2 | 27-3 | [Static Antenna switching SRS Insertion Loss Imbalance Reporting] | Capability to indicate the absolute value of reported actual IL imbalance of each SRS resource other than the first one within the SRS resource set configured for antenna switching usage. - The reported values shall not exceed the ΔTRxSRS requirements that defined in TS 38.101-1 for each AS-SRS capability accordingly. If the UE support this capability, it should also support 27-5. | 2-55 or 23-8-3 | Yes |  |  | Per BC | N/A | N/A | N/A | Static reporting means that the UE will not change the indicated actual IL imbalance once it has reported that for configured SRS resource set with antenna switching usage. | Optional with capability signalling |
| 27.NR_ENDC_RF_FR1_enh2 | 27-4 | [Dynamic Antenna switching SRS Insertion Loss Imbalance Reporting] | Capability to indicate support of dynamic report of the absolute value of actual IL imbalance of each SRS resource other than the first one within the SRS resource set configured for antenna switching usage. If the UE support this capability, it should also support 27-5. | 2-55 or 23-8-3 | Yes |  |  | Per BC | N/A | N/A | N/A | The reported values shall not exceed the ΔTRxSRS requirements that defined in TS 38.101-1 for each AS-SRS capability accordingly. Dynamic reporting means that the UE may trigger a reporting once it finds out the actual IL imbalance would be different from previous reporting. | Optional with capability signalling |
| 27.NR_ENDC_RF_FR1_enh2 | 27-5 | [Tx power threshold] | Capability to indicate the threshold. When the transmission power (as defined in TS 38.213 clause 7.3) is larger than the threshold, the UE shall maintain the reported values by following [Static Antenna switching SRS Insertion Loss Imbalance Reporting] or [Dynamic Antenna switching SRS Insertion Loss Imbalance Reporting] corresponding to each SRS resource other than the first one for each configured SRS resource set. The candidate value of the threshold can be one from the set {P0, P0-1, P0-2,…, P0-8}dBm, where P0 = min(PEMAX,c, PPowerClass) | (2-55 or 23-8-3) and (27-3 or 27-4) | Yes |  |  | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |

# 28. NR Channel raster enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28.NR_channel_raster_enh | 28-1 | 100 KHz channel raster | Capability to indicate support of the enhanced channel raster, i.e. 10 KHz channel raster |  | yes |  | 100 KHz channel raster shall be applied | Per Band | N/A | Applicable to FR1 only | N/A | The capability should be indicated per band and be applicable for all TN and NTN FR1 bands below 3GHz with a 100 kHz channel raster. (R4-2317773) | Optional with capability signalling |

# 29. NR RF requirements enhancement for FR2, Phase 3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29.NR_RF_FR2_req_Ph3 | 29-1 | Beam correspondence in RRC_IDLE and RRC_INACTIVE | Support of beam correspondence requirement specified in TS 38.101-2 section 6.6.4 for RRC_IDLE and RRC_INACTIVE. The requirement applies to Power Class 3 in this release. | N/A | N/A | N/A | The UE’s beam correspondence performance in RRC_IDLE and RRC_INACTIVE is unspecified. | Per UE | N/A | FR2 only | N/A |  | Mandatory without capability signalling |

# 32. NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Combination of pre-configured MG and concurrent MG | UE can be configured with concurrent MGs with one or more of the gaps being pre-configured MGs | 19-2, and 19-3-1 or 19-3-2 | Yes |  | UE cannot be configured with combination of pre-configured MG and concurrent MG | Per-UE | N/A | N/A |  |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Combination of pre-configured MG and concurrent MG with dynamic collision | UE can be configured with concurrent MGs with one or more of the gaps being pre-configured MGs, and the pre-configured MG is configured with higher priority and is colliding with another MG. | 32-1 | Yes |  | UE cannot be configured with combination of pre-configured MG and concurrent MG with dynamic collision | Per-UE | N/A | N/A |  |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Combination of NCSG and concurrent MG | UE can be configured with concurrent MGs with one or more of the gaps being NCSG | 19-2 and 19-1-2 | Yes |  | UE cannot be configured with combination of NCSG and concurrent MG | Per-UE | N/A | N/A |  |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-X | Short effective measurement window duration for inter-RAT E-UTRA measurement | UE can be configured with short effective measurement window duration of 2ms for inter-RAT E-UTRA measurement, where the measurement is performed outside MG and causes scheduling restriction | 19-1 or [new capability for Case b-1] | Yes |  | UE cannot be configured with short effective measurement window duration for inter-RAT E-UTRA measurement. | Per-UE | N/A | N/A |  |  | Optional with capability signalling |

# 33. Support of intra-band non-collocated EN-DC/NR-CA deployment

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33.NonCol_intraB_ENDC_NR_CA | 33-1 | Support of intra-band non-collocated NR-CA deployment | The UE can support TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA in 7.10A in 38.101-1. |  | Yes | N/A | The UE will not be able to support an intra-band non-collocated NR-CA deployment | per BC | N/A | FR1 Only |  |  | Optional with capability signalling |
| 33.NonCol_intraB_ENDC_NR_CA | 33-2 | Rel-18 requirement type indication for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 | For Rel-18, introduce a new UE capability which indicates the support of the new BS signaling per UE. This UE capability is only applicable to the UE indicating “interBandMRDC-WithOverlapDL-Bands-r16”. | 2-19 | Yes | N/A | The UE will support “interBandMRDC-WithOverlapDL-Bands-r16” only which means in Rel-18 the network can’t enforce modification on the UE types | per BC | N/A | FR1 Only |  |  | Optional with capability signalling |

# 34. Enhanced NR support for high speed train scenario in frequency range 2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-1 | Support of FR2 HST multi-RX reception. | Support of simultaneous DL reception with two different QCL TypeD RSs on single component carrier for FR2 HST. | 16-2c 16-2a or 16-2b-1 | N/A | N/A | The UE could not receive downlink signals with two different QCl TypeD RSs | Per Band | N/A | FR2 only | N/A |  | Optional with capability signalling |

# 37.	Expanded and improved NR positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37.NR_pos_enh2 | 37-1 | DL PRS or UL SRS frequency hopping for RedCap UEs | Indicate the switching time for UL SRS or DL PRS hopping for RedCap UEs. |  | Yes | N/A | UE does not support DL PRS or UL SRS frequency hopping for RedCap UEs | Per Band | N/A | N/A | N/A | Candidate value set: FR1: {70us, 140us, 210us}. FR2: {35us, 70us, 125us} Detailed information can refer to the LS to RAN1 in R4-2306659 and R4-2310305 | Optional with capability signalling |
| 37.NR_pos_enh2 | 37-2 | DL PRS or UL SRS frequency hopping before the first hop and after the last hop for RedCap UEs | Indicate the switching time for UL SRS or DL PRS transmission between the initial/active BWP to first hop and switching time between last hop to the initial/active BWP for RedCap UEs. | 37-1 | Yes | N/A | The switching time repoted in 37-1 would apply for DL PRS or UL SRS frequency hopping before the first hop and after the last hop for RedCap UEs | Per Band | N/A | N/A | N/A | Candidate value set: {100us, 140us, 200us, 300us, 500us } Detailed information can refer to the LS to RAN1 in R4-2314732. | Optional with capability signalling |

# 38. Multi-carrier enhancements for NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38.NR_MC_enh | 38-1 | Dynamic Tx switching between 2 bands in 3-band or 4-band combination 1Tx-1Tx switching | Indicate the supported switching period for dynamic UL Tx switching between two bands both with one transmit antenna connectors in 3-band or 4-band band combination with inter-band UL CA or SUL |  | Yes | N/A | UE does not support Tx switching among 3 or 4 bands 1Tx-1Tx switching for inter-band UL CA and SUL band combinations | UE signals supported switching period per band pair per band combination | N/A | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2303507 and R4-2310271. | Optional with capability signalling |
| 38.NR_MC_enh | 38-2 | Dynamic Tx switching between 2 bands in 3-band or 4-band combination 1Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between one band capable of one transmit antenna connector and the other band capable of two transmit antenna connectors in 3-band or 4-band band combination with inter-band UL CA or SUL |  | Yes | N/A | UE does not support Tx switching among 3 or 4 bands 1Tx-2Tx switching for inter-band UL CA and SUL band combinations | UE signals supported switching period per band pair per band combination | N/A | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2214464. | Optional with capability signalling |
| 38.NR_MC_enh | 38-3 | Dynamic Tx switching between 2 bands in 3-band or 4-band combination 2Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between two bands both with two transmit antenna connectors in 3-band or 4-band band combination with inter-band UL CA or SUL. |  | Yes | N/A | UE does not support Tx switching among 3 or 4 bands 2Tx-2Tx switching for inter-band UL CA and SUL band combinations | UE signals supported switching period per band pair per band combination | N/A | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2214464. | Optional with capability signalling |
| 38.NR_MC_enh | 38-4 | Continuation of uplink transmission on the band with number of Tx chain unchanged | When dynamic UL Tx switching happens between two bands both with one transmit antenna connectors, and there is another band in the band combination with number of Tx chain unchanged, this capability indicates UE support of continuation of uplink transmission on the band with number of Tx chain unchanged during the switching period of the Tx switching between other two bands If this capability is not supported, the switching period of the two bands applies to the band with number of Tx chain unchanged. | 38-1 | Yes | N/A | The switching period of the two bands applies to the band with number of Tx chain unchanged. | Per band per band pair per band combination | N/A | Applicable only to FR1 | Support mixture of FDD/TDD | Detailed information can refer to the LS to RAN2 in R4-2303507 | Optional with capability signalling |
| 38.NR_MC_enh | 38-5 | Dynamic Tx switching of two band pairs among 3 bands 1Tx-1Tx switching | Indicate the switching period for dynamic UL Tx switching of two band pair in three bands all with one transmit antenna connector. | 38-1 | Yes | N/A | The switching period applied to the two band pairs in the three bands is shorter. | UE signals supported switching period per band(with the number of Tx chains unchanged) per band pair per band combination | No need | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2317610 | Optional with capability signalling |
| 38.NR_MC_enh | 38-6 | Dynamic Tx switching with additional period of two band pairs among 3 bands 1Tx-2Tx switching | Indicate the additional switching period for dynamic UL Tx switching of two band pair in three bands including one band with two transmit antenna connectors. | 38-2 | Yes | N/A | The switching period applied to the two band pairs in the three bands is shorter. | UE signals supported switching period per band combination | N/A | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2310495 | Optional with capability signalling |
| 38.NR_MC_enh | 38-7 | Dynamic Tx switching of two Tx chains with additional period among 4 bands 1Tx-1Tx switching | Indicate the additional switching period for two Tx chains switching among 4 bands. | 38-1 | Yes | N/A | The switching period applied to the two band pairs in the four bands is shorter | UE signals supported switching period per band combination | N/A | Applicable only to FR1 | Support mixture of FDD/TDD | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2310495 | Optional with capability signalling |
| 38.NR_MC_enh | 38-8 | Dynamic Tx switching between two uplink carriers 1Tx-2Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between one carrier capable of one transmit antenna connector and other carrier capable of two transmit antenna connectors in inter-band UL CA or SUL | 7-1 | Yes | N/A | UE does not support Tx switching between two uplink carriers 1Tx-2Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 38.NR_MC_enh | 38-9 | Dynamic Tx switching between 2CC 2Tx-2Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between two uplink carriers with two transmit antenna connectors in inter-band UL CA or SUL | 16-1 | Yes | N/A | UE does not support Tx switching between 2CC 2Tx-2Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 38.NR_MC_enh | 38-10 | Dynamic Tx switching between 3CC 1Tx-2Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between one band (with one carrier) capable of one transmit antenna connector and the other band (with two carriers) capable of two transmit antenna connectors in inter-band UL CA or SUL | 16-2 | Yes | N/A | UE does not support Tx switching between 3CC 1Tx-2Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 38.NR_MC_enh | 38-11 | Dynamic Tx switching between 3CC 2Tx-2Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between one band (with one carrier) capable of two transmit antenna connectors and one band (with two carriers) capable of two transmit antenna connectors in inter-band UL CA or SUL | 16-3 | Yes | N/A | UE does not support Tx switching between 3CC 2Tx-2Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 38.NR_MC_enh | 38-12 | Dynamic Tx switching between 2 bands in 3-band or 4-band combination 1Tx-1Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between two bands both with one transmit antenna connector in 3-band or 4-band band combination with inter-band UL CA or SUL | 38-1 | Yes | N/A | UE does not support Tx switching between 2 bands in 3-band or 4-band combination 1Tx-1Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 38.NR_MC_enh | 38-13 | Dynamic Tx switching between 2 bands in 3-band or 4-band combination 1Tx-2Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between one band capable of one transmit antenna connector and the other band capable of two transmit antenna connectors in 3-band or 4-band band combination with inter-band UL CA or SUL | 38-2 | Yes | N/A | UE does not support Tx switching between 2 bands in 3-band or 4-band combination 1Tx-2Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 38.NR_MC_enh | 38-14 | Dynamic Tx switching between 2 bands in 3-band or 4-band combination 2Tx-2Tx switching with dual-TAG | Support UE omitting uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference for dynamic UL Tx switching between two bands both with two transmit antenna connectors in 3-band or 4-band band combination with inter-band UL CA or SUL. | 38-3 | Yes | N/A | UE does not support Tx switching between 2 bands in 3-band or 4-band combination 2Tx-2Tx switching with dual-TAG |  | N/A | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signalling |

# 40.	NR NTN enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40.NR_NTN_enh | 40-1 | FSS in NTN | - Support fixed station service Support access to an MSS cell or an FSS cell |  | Yes | N/A | The network doesn’t know the UE type and cannot decide whether to handover this UE to an FSS cell. | [Per UE/Per band] | N/A | N/A | N/A |  | Optional with capability signalling |
| 40.NR_NTN_enh | 40-2 | MSS in NTN | - Support mobile station service - Support access to an MSS cell Support receiving access control indication in system information |  | Yes | N/A | The network doesn’t know the UE type and cannot control whether to allow an MSS UE to access and cannot decide whether to handover this UE to an FSS cell | [Per UE/Per band] | N/A | N/A | N/A |  | Optional with capability signalling |
| 40.NR_NTN_enh | 40-3 | Rx beam steering | Support of Rx beam steering capability - Type 1: Fully electronically-steered beam UEs - Type 2: Fully mechanically-steered beam UEs A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 must indicate support of this capability with one and only one type. The capability is not applicable for UE other than VSAT. |  | Yes | N/A | Rx beam steering is not supported. | Per-band | FDD only | N/A | N/A |  | Optional with capability signaling |

# 41. Further NR coverage enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41.NR_cov_enh2 | 41-1 | [Static full power transmission mode capabilities indication for HPUE under max duty cycle exceedance and return from exceedance scenarios] | For HPUE, In addition to the full power transmission mode capability indication (e.g., one from ul-FullPwrMode-r16, ul-FullPwrMode2-r16 or ul-FullPwrMode2-TPMIGroup-r16) for the advertised power class, indicate all capabilities for each reference power class (refer to TS 38.101-1 the definition of ΔPPowerClass) as well. Specifically, for a PC1.5 capable HPUE, it is allowed to indicate full power transmission mode capability for PC1.5, PC2 and PC3 respectively. For a PC2 capable HPUE, if is allowed to indicate full power transmission mode capability for PC2 and PC3, respectively. |  | Yes |  | The legacy ul-FullPwrMode canapbility per FS shall be applied | Per band per band combination | N/A | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 41.NR_cov_enh2 | 41-2 | [Dynamic full power transmission mode capabilities indication for HPUE under max duty cycle exceedance and return from exceedance scenarios] | For HPUE, indicate the capability (e.g., one from ul-FullPwrMode-r16, ul-FullPwrMode2-r16 or ul-FullPwrMode2-TPMIGroup-r16) once max duty cycle exceedance or return from exceedance happens, which means such UE may change its capability for the advertised/reference power class (refer to TS 38.101-1 the definition of ΔPPowerClass). |  | Yes |  | The legacy ul-FullPwrMode canapbility per FS shall be applied | Per band per band combination | N/A | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 41.NR_cov_enh2 | 41-3 | [ΔPPowerClass reporting for HPUE] | To indicate the support of ΔPPowerClass reporting, which is triggered by max duty cycle exceedance or return from exceedance so that there is change for the value of ΔPPowerClass. |  | Yes |  |  | Per band per band combination | N/A | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |
| 41.NR_cov_enh2 | 41-4 | [Power boosting] | For PC3 capable UE, indicate the support of [1] dB power boosting for QPSK DFT-s-OFDM waveform and inner RB allocations. |  | Yes |  |  | Per band | N/A | FR1 only |  |  | Optional with capability signalling |

# 42.	Network energy saving for NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42.Netw_Energy_NR | 42-1 | SCell without SS/PBCH block for inter-band CA | Support of SCell without SS/PBCH block for inter-band CA |  | Yes | NA | UE cannot support SCell without SS/PBCH block for inter-band CA | Per BC | NA | FR1 only | NA | UE support this feature shall be able to use SS/PBCH block from other Cells from another band within the band combination for time/frequency synchronization of SCell without SS/PBCH block. UE shall meet the SCell activation requirements based on periodic CSI-RS for tracking defined in 38.133 8.3.2 | Optional with capability signaling |
| 42.Netw_Energy_NR | 42-2 | Aperiodic CSI-RS for tracking for fast SCell activation for SCell without SS/PBCH block for inter-band CA | Support of aperiodic CSI-RS for tracking for fast SCell activation for SCell without SS/PBCH block for inter-band CA | 42-1 | Yes | NA | UE cannot support fast SCell activation based on aperiodic CSI-RS for tracking for SCell without SS/PBCH block for inter-band CA | Per band | NA | FR1 only | NA | UE support this feature shall support fast SCell activation based on aperiodic CSI-RS for tracking for SCell without SS/PBCH block for inter-band CA. UE shall meet the SCell activation requirements based on Aperiodic CSI-RS for tracking defined in 38.133 8.3.2 | Optional with capability signaling |

# 44. Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44.4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC | 44-1 | Support per band per band combination TxD for 2Tx | Support per FS TxD capability for 2Tx from Rel-18 |  | Yes | N/A | The existing txDiversity-r16 cannot indicate the diversity capability differently per band and per band combination for a same NR band | Per FS | N/A | FR1 |  | see LS in R4-2317762 When TxD is supported for one of the bands in the CA/DC band combination. a new per band per band combination UE capability would be required to indicate support of TxD in the band configured with 2Tx, in Rel18. | Optional with capability signalling |

# 45. NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 | UE power class for sidelink CA | Support power class 3 per band combination for intra-band contiguous sidelink CA |  | Yes | Yes | UE does not support power class 3 for intra-band sidelink CA band combination | Per BC | NA | FR1 only | NA | Detailed information can refer to the LS to RAN2 in R4-2317751 | Optional [with capability signaling] |