3GPP TSG-RAN WG4 Meeting # 109 	R4-2318472

Chicago, US, November 13 – 17, 2023

Agenda item:	8.19.2.2.1

Source:	Murata Manufacturing Co. Ltd

Title:	TP to TR 38.858: Feasibility of FR1 WA BS aspects

Document for:	Approval

Introduction

This contribution provides text proposal to TR 38.858 on the feasibility of FR1 wide area BS. The relevant document is R4-2318471.

TP to TR 38.858

<Start of TP>

#### 9.2.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

<Unchanged sections omitted>

##### 9.2.1.2.7	Murata

Analogue Filtering

In order to improve self-interference, implementing analogue filter in receiver front end to suppress DL Tx signal was investigated. Murata analysed the possibility of this filter and impact of its temperature dependence.

The target performance of filter was set as follows based on the analysis in RAN4. And Q-factor was assumed as 2,000.

Table 9.2.1.2.7-1: Analysis condition of blocking filter

| Item | Specification | Note |
| --- | --- | --- |
| Carrier frequency | 3.5GHz |  |
| SBFD configuration | DUD(40-20-40MHz) |  |
| Guardband | 5PRB | 1.8MHz |
| Attenuation | > 25dB |  |

Figure 9.2.1.2.7-1: Simulation result of blocking filter(guardband=5PRB)

In order to achieve > 25dB attenuation at stopband, the insertion loss of band edge is 10.9dB, in-band deviation is 8.7dB, it is clearly too worse.

As showed in Figure 9.2.1.2.7-2 in case of expanding the guard band to 5MHz, possible to achieve > 25dB attenuation and < 3dB insertion loss. And the in-band deviation is 1.5dB. If it is implemented after the 1st stage LNA, the insertion loss is not a problem as the effect on receiver sensitivity is small. But this result is not including the influence of temperature dependence and individual variation.

Figure 9.2.1.2.7-2: Simulation result of blocking filter(guardband=5MHz)

To use analogue filter, temperature dependence and individual variation should be considered. Since individual variation relates to the manufacturing process, it will not be discussed here, but it is possible to investigate the impact of temperature dependence.

Generally, TCF (Temperature Coefficients of Frequency) of conventional SAW filter is about -40 ppm/℃, and TCF of BAW filter is -20 to 30 ppm/℃. Of course, the filter which improved TCF is exist. For example, TCF = -8 ppm/℃ was presented for a high-performance filter. But considering versatility, at least 20 ppm/℃ TCF should be considered. Hence, how much the filter frequency shift was analysed for the best TCF of -8ppm/℃ and the general case of -20ppm/℃. The temperature range was set as -40 to +85℃.

Figure 9.2.1.2.7-3: Temperature dependence of blocking filter.

Table 9.2.1.2.7-2: Temperature dependence of insertion loss and attenuation.

|  | TCF = -8 [ppm/℃] |  | TCF = -20 [ppm/℃] |  |
| --- | --- | --- | --- | --- |
| Temperature | -40℃ | +85℃ | -40℃ | +85℃ |
| Insertion loss | < 4.9 dB | < 5.2dB | < 24.1dB | < 20.5dB |
| Attenuation | > 11. 5dB | > 12.6dB | > 3.5 dB | > 3.2dB |

Presented as the above, even if TCF = -8 ppm/℃, insertion loss degrades to 5dB and attenuation degrades to 12dB. And in case of TCF = -20 ppm/℃, it is not possible to expect filter function. Furthermore, the above result does not include the device individual variation. Thus, it is difficult to suppress interference of DL Tx signal by implementing BPF in RF front end.

#### <End of TP>

References

- R4-2317010, “Draft TR 38.858”, CMCC, October 2023.

- R4-2318472, “Feasibility of filtering for FR1 BS in SBFD”, Murata Manufacturing Co Ltd.