3GPP TSG-RAN WG4 Meeting #109	R4-2318479

Chicago, US, November 13 – 17, 2023

Agenda item:	8.21.3

Source: 		NTT DOCOMO, INC.

Title: 	Interoperability and testability aspects discussions for NR AI/ML

Document for:	Discussion

- Introduction

From RAN4#106-bis, RAN4 has intensively discussed AI/ML for NR air interface. Although RAN4 reached some consensuses, there are still many open items. The previous agreements and open items are summarized in [1]. This contribution proposes test encoder/decoder requirements for option 4 and provides opinions for comparison table between each option.

- Discussion

  - Test encoder/decoder option 4

Agreement:

- Who builds the decoder?

  - TE vendor should be able to develop  the decoder just based on the specifications

    - FFS what needs to be specified, RAN4 might specify some high level parameters for the decoder (e.g. parameters related to processing complexity, model structure, etc)

        - FFS exactly which parameters are needed

    - Test repeatability should be ensured (variation among TE vendor implementations should be bound)

    - Other vendors should also be able to develop such a decoder and which can deliver similar performance within the same bounds as with TE vendors

      - FFS how similar the performance has to be among possible implementations

Companies are invited to bring further inputs for the following questions:

- Is there a standardized data set for this decoder?

- Will decoder be shared with DUT vendors and infra vendors?

In previous meeting, RAN4 reached following agreements:

For Option 4, the decoder is developed by each TE vendor and test repeatability should be ensured. In order to bound the output variation within certain range, at least data set for this decoder should be standardized. If data set is standardized and other vendors can develop a decoder which can deliver similar performance, DUT vendors and infra vendors can prepare the reference decoder and performance is similar, thus the decoder developed by TE vendor is not needed to be shared with them.

Proposal 1: The data set used for the decoder training should be standardized to bound the output variation within certain range and ensure the repeatability.

Proposal 2: If data set is standardized and other vendors can develop a decoder which can deliver similar performance, DUT vendors and infra vendors can prepare the reference decoder and performance is similar, thus the decoder developed by TE vendor is not needed to be shared with them.

  - Test encoder/decoder options comparison table

Proposal 3: For some issues, we provide our opinion in the table below (yellow highlighted):

|  | Option 1: DUT provides decoder | Option 2: Decoder not from DUT and Spec | Option 3: Full decoder specification in standard | Option 4: partially specified decoder |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | It should be specified to bound the output variation within certain range and ensure the repeatability |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type between DUT and decoder provider (source of training data should be consistent with the collaboration type) | No collaboration is defined because DUT vendor develops both encoder and decoder, or this can be included in all the Types | Type 1 or 3. It depends on source of decoder training data. | No collaboration is defined because decoder model is fully specified, or this can be included in all the Types | Type 3 according to the assumption that training data set is standardized and decoder model is not needed to be shared. |
| Test decoder performance verification procedure at TE and/or DUT | TE side verification is needed. Since the decoder is developed by DUT, it should be verified at TE | TE side verification is needed. Since the decoder is developed by DUT, it should be verified at TE | Not needed because decoder model is fully specified | Not needed because TE develops the decoder |
| Feasibility of test decoder verification procedure |  |  |  |  |
| Number of test per test configuration/setup (propagation condition, CSI configuration etc excluding decoder/network side model configuration) |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Probably low. It depends on the training data | Probably high. It depends on the training data | Low. It is difficult to reflect the real deployment to standardized model | Probably low. . It depends on the training data |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) |  |  |  |  |
| Specification Effort (e.g. test decoder) |  |  |  |  |
| Confidentiality/ IP issues |  |  |  |  |
| Applicability to different scenarios/conditions/ configurations | It depends on generalization test procedure | It depends on generalization test procedure | It depends on generalization test procedure | It depends on generalization test procedure |
| Complexity of actual testing procedure for the ecosystem |  |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented | Yes, DUT vendor can update decoder model | Yes, decoder implementer can update decoder model | No, it needs discussions how to specify | Yes, TE vendor can update decoder model |
| Relationship with reference decoder/encoder for defining requirement | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] |
| Whether model transfer/delivery is needed during the test procedure | Not needed | Not needed | Not needed | Not needed |

- Conclusion

In this contribution, we proposed our views on interoperability and testability aspects discussions for NR AI/ML.

Proposal 1: The data set used for the decoder training should be standardized to bound the output variation within certain range and ensure the repeatability.

Proposal 2: If data set is standardized and other vendors can develop a decoder which can deliver similar performance, DUT vendors and infra vendors can prepare the reference decoder and performance is similar, thus the decoder developed by TE vendor is not needed to be shared with them.

Proposal 3: For some issues, we provide our opinion in the table below (yellow highlighted):

|  | Option 1: DUT provides decoder | Option 2: Decoder not from DUT and Spec | Option 3: Full decoder specification in standard | Option 4: partially specified decoder |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | It should be specified to bound the output variation within certain range and ensure the repeatability |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type between DUT and decoder provider (source of training data should be consistent with the collaboration type) | No collaboration is defined because DUT vendor develops both encoder and decoder, or this can be included in all the Types | Type 1 or 3. It depends on source of decoder training data. | No collaboration is defined because decoder model is fully specified, or this can be included in all the Types | Type 3 according to the assumption that training data set is standardized and decoder model is not needed to be shared. |
| Test decoder performance verification procedure at TE and/or DUT | TE side verification is needed. Since the decoder is developed by DUT, it should be verified at TE | TE side verification is needed. Since the decoder is developed by DUT, it should be verified at TE | Not needed because decoder model is fully specified | Not needed because TE develops the decoder |
| Feasibility of test decoder verification procedure |  |  |  |  |
| Number of test per test configuration/setup (propagation condition, CSI configuration etc excluding decoder/network side model configuration) |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Probably low. It depends on the training data | Probably high. It depends on the training data | Low. It is difficult to reflect the real deployment to standardized model | Probably low. . It depends on the training data |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) |  |  |  |  |
| Specification Effort (e.g. test decoder) |  |  |  |  |
| Confidentiality/ IP issues |  |  |  |  |
| Applicability to different scenarios/conditions/ configurations | It depends on generalization test procedure | It depends on generalization test procedure | It depends on generalization test procedure | It depends on generalization test procedure |
| Complexity of actual testing procedure for the ecosystem |  |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented | Yes, DUT vendor can update decoder model | Yes, decoder implementer can update decoder model | No, it needs discussions how to specify | Yes, TE vendor can update decoder model |
| Relationship with reference decoder/encoder for defining requirement | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] | Reference encoder/decoder is not needed to be considered according to the agreements in ad-hoc session in #108 [2] |
| Whether model transfer/delivery is needed during the test procedure | Not needed | Not needed | Not needed | Not needed |

References

[1] 3GPP, R4-2317631, Qualcomm Incorporated, “WF on RAN4 requirements for NR AI/ML”

[2] 3GPP, R4-2314740, Qualcomm Incorporated, “WF on RAN4 requirements for NR AI/ML”