# 3GPP TSG-RAN WG4 Meeting # 109				R4-2318498

# Chicago, US, November 13 – 17, 2023

Agenda item:	10

Source: 	MediaTek

Title: 	Rel-18 RAN4 UE feature list for NR

Document for:	Discussion

- Introduction

In this paper, we provide our reviews on the Rel-18 RAN4 UE feature list on selective items.

- Proposed UE feature list

The detail discussions are provided in the discussion paper of individual WIs. Here, we only provide the collected inputs among WIs.

# NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_ENDC_RF_FR1_enh2 | 27-y | Network query filter | UE supports network query filter such as band(s), MSD types, power class, frequency range, minimum lower-MSD class, top-K largest MSD class and etc for low MSD reporting | 27-x [lower-MSD capability] | Yes | No | Network cannot apply the query filter | per BC | No | FR1 only | N.A |  | Optional with UE capability |

# NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Support of requirements for multiRx_DL | - Supports simultaneous reception of PDCCH/PDSCH and RS for L1 measurement with different QCL Type-D on overlapping OFDM symbols, and - Supports simultaneous measurement of 2 RS for L1 measurements different QCL Type-D on overlapping OFDM symbol(s), and - Supports requirements for dual TCI state switch in TS38.133 Note: Above 3 bullets belong to one single UE capability | 16-2c | Yes | No | UE does not follow the requirements | [Per band] | TDD only | FR2-1 only |  |  | Optional with capability signalling |
| 30. NR_FR2_multiRX_DL | 30-2 | Fast beam sweeping for L1 measurement | - Supports beam sweeping factor reduction for SSB-based layer 1 measurement. |  | No | N/A | UE’s L1 measurement delay is not reduced | Per band | TDD only | FR2-1 only |  | Candidate values: {2,4,6} | Optional without capability signalling |
| 30. NR_FR2_multiRX_DL | 30-3 | Joint demodulation processing of multiple RX panels | - Supports joint demodulation processing of multiple RX panels | 16-2c [ and 30-1] | Yes | N/A | In scenarios with cross-talk between RX panels demodulation performance is worse | Per band | TDD only | FR2-1 only |  |  | Optional with capability signalling |

# NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | Enhanced L3 report after SCell activation command | Support network triggered L3 measurement report upon SCell activation command. |  | Yes | No | UE cannot support network triggered L3 measurement report upon SCell activation command | Per UE | [No] | [No] | N/A |  | Optional with UE capability |
|  | 31-2 | Reduced beam sweeping factor | - Value X1 for the reduce beam sweeping factor for L3 cell search during SCell activation - Value X2 for the reduce beam sweeping factor for SSB-based L1 measurements during SCell activation |  | Yes | No | The beam sweeping factor is 8 for both cases | Per band | TDD only | FR2 only | N/A | Candidate values for X1 ={1, 2, 4, 6} Candidate values for X2 ={0, 1, 2, 3, 4, 5, ,6 ,7} | Optional with UE capability |
|  | 31-3 | Enhanced SCell activation delay | - Support of using SSB periodicity instead of SMTC periodicity during SCell activation - Perform L1-RSRP measurements with the delay assuming non-DRX during SCell activation even DRX is configured |  | Yes | No | UE will still use SMTC periodicity and follow the L1-RSRP measurement delay with DRX if DRX configured during SCell activation | Per UE | [No] | [No] | N/A |  | Optional with UE capability |

# NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gap with Pre-MG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Table 9.1.x-1 of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-2 | 2 Pre-MG configuration with simultaneous activation/deactivation | Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | Network should not configure 2 Pre-MG in the same FR | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-3] | Dynamic collision | Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap or Pre-MG | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-4 | Concurrent gap with NCSG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in Table 9.1.y-1 of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-5] | 2 NCSG configuration | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-6 | Inter-RAT EUTRAN measurement with RS on UE active BWP | Support inter-RAT EUTRAN measurements with CRS contained within UE’s active DL BWP |  | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only |  |  | Optional with capability signalling |
|  | [32-7] | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | 32-6 | Yes | No | Undefined UE behavior about when to measurement and scheduling restriction | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-8] | simultaneousRxDataCRS-DiffNumerology | Support concurrent inter-RAT measurement on EUTRAN cell with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-6 | Yes | No | scheduling restriction is applicable | Per UE | No | No |  |  | Optional with capability signalling |

# NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 | non-collocated NR intra-band non-contiguous CA | Indicates the UE supports NR TDD intra-band non-collocated non-contiguous CA with NR CA MRTD according to Table 7.6.4-2 in 38.133 in FR1 and inter-band RF requirements (i.e., Type 2 UE) | - | Yes | N/A | UE supports intra-band non-contiguous NR CA with MRTD<3us according to Table 7.6.4-1 in 38.133 in FR1 and intra-band RF requirements (i.e., Type 1 UE) | [Per BC] | TDD only | FR1 only | N/A | - | Optional with UE capability signalling |

# NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | 36-1 | Support of requirements for advanced receiver in MU-MIMO scenarios | - Supports new DCI field for network assistance signalling without modulation order blind detection of co-scheduled UEs |  | Yes | N/A | UE does not follow the requirements (Fallback to Rel-17 MU-MIMO requirements) | [Per FSPC] |  | FR1 only |  |  | Optional with capability signalling |
| 36. NR_demod_enh3 | 36-2 | Support of co-scheduled UE modulation order blind detection | - Supports modulation order blind detection of co-scheduled UEs | 36-1 | Yes | N/A | UE does not follow the requirements that require modulation order blind detection of co-scheduled UEs | [Per FSPC] |  | FR1 only |  | Candidate value options: - {Supported, Not supported} - {All supported, Partial support, Not supported | Optional with capability signalling |

# NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-x | Preferred switching band pairs | Support the indication of UE’s preferred (switched-from, switched-to) switching band pairs for parallel UL transmission switching for a band combination consisting of four different bands | 38-1 [Rel-18 Tx switching] | Yes | No | Network can only assume the maximum switch period | Per BC | No | FR1 only | N.A |  | Optional with capability signalling |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 | Simultaneous L1-RSRP measurements for multiple cells with RTD > CP | - Capability of simultaneous L1-RSRP measurements for more than 1 Cells with RTD > CP on the same OFDM symbol in one FR1 frequency layer - Capability of L1-RSRP measurements for more than 1 Cells with RTD > CP in one FR2 frequency layer Note: Duplication with component 1 of 45-1 and 45-1a in RAN1 feature list. Suggest to handle in RAN4 feature list | 45-1, or 45-1a from RAN1 Rel-18 feature list | Yes | No | [NW does not know which requirements UE will follow] | Per UE | No | Yes | N/A | Note: the max number of cells is up to 39-2 | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-2 | Number of [neighbour] cells to be measured per frequency layer | - Number of [neighbour] cells to be measured for L1-RSRP per frequency layer per intra-freq layer - Number of neighbour cells to be measured for L1-RSRP per frequency layer per inter-freq layer Note: Duplication with component 3 of 45-1 and 45-1a in RAN1 feature list. Suggest to handle in RAN4 feature list | 45-1, or 45-1a from RAN1 Rel-18 feature list | Yes | No | NW does not know the max number of cells UE can measure | Per band | No | [No] | N/A | Candidate values: 2, 3, [… TBD] | Mandatory with capability signaling if UE supports 45-1, or 45-1a |
| 39. NR_Mob_enh2 | 39-3 | Interruption on DL symbols due to PDCCH- ordered RACH | Capability on whether UE will cause interruption on DL symbols on serving cells due to PDCCH-ordered RACH | 45-5 | Yes | No | UE will not cause interruptions on DL symbols on serving cells due to PDCCH-ordered RACH | Per band pair (between the target band for RACH transmission and band under UE’s current band combo) per band combination | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-4 | Early ASN.1 decoding and validity check before cell switch | Capability of early ASN.1 decoding and validity check before cell switch to [reduce or skip] Texecution delay (refer to TS 38.133). Note: Additional conditions to apply the capability is FFS in RAN4. Additional components may be introduced to this FG later | [Component 6 of 45-3 or 45-4] | Yes | No | Texecution delay will not be [reduced or skipped] | [Per UE] | No | No | N/A | [Candidate values: 0 or TBD] | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-5] | Shorter UE processing time during cell switch | Capability of reduced TLTM, processing delay (refer to TS 38.133). Note: Additional conditions to apply the capability is FFS in RAN4. Additional components may be introduced to this FG later | [Component 6 of 45-3 or 45-4] | Yes | No | TLTM, processing delay will not be reduced | [Per UE] | No | No | N/A | Candidate values: TBD | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-6] | Using L3 measurement results in L1-RSRP measurement report | Capability of using L3 measurement results in L1 measurement report Note: Additional conditions to apply the capability is FFS in RAN4. | 45-1, or 45-1a | [Yes or No] | No | UE will not use L3 measurement results in L1 measurement report | [Per UE] | No | [Yes] | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-7] | Measurement validation during connection setup/resume | Support of measurement validation during connection setup/resume |  | [Yes] | N/A | UE does not support measurement validation during connection setup/resume | [Per-UE] | No | [Yes] | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-8] | Enhanced measurement during RRC connection setup/resume | Support of enhanced measurement during RRC connection setup/resume |  | [Yes] | N/A | UE does not support enhanced measurement during RRC connection setup/resume | [Per-UE] | No | [Yes] | N/A |  | Optional with capability signaling |

# NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh | [40-1] | Mechanical beam sweeping delay | For mechanical beam steering UE, to indicate the beam switching delay from one satellite to another in Ka band | TBD | Yes | No |  | Per Band (Ka band only) | FDD only (Ka band only) | TBD | N.A | Candidate values are TBD | Optional with UE capability signalling |

# Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. # Netw_Energy_NR | 42-1 | Scell without SSB for FR1 inter-band collocated CA | Support of Scell without SSB for FR1 inter-band collocated CA |  | Yes | No | UE can not support this feature | Per BC | No | FR1 only | N.A |  | Optional with UE capability |

# NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. # NR_DualTxRx_MUSIM | 43-1 | Requirements for MUSIM gaps | Support of requirements of MUSIM gap, including priority indication to network, collision handling between MUSIM gap and measurement gap and among MUSIM gaps |  | Yes | No | UE cannot meet the requirement for MUSIM gap | Per UE | No | No | N/A |  | Optional with UE capability |
|  | 43-2 | Keep solution | Support the indication of keeping all MUSIM gaps upon colliding among MUSIM gaps | 43-1 | Yes | No | UE follows priority rule upon colliding among MUSIM gaps | Per UE | No | No | N/A |  | Optional with UE capability |

- Discussions

Regarding 39-1 and 39-2 for NR_Mob_enh2, we understand similar issues were also discussed in RAN1 UE feature list. FG 39-1 has some overlap with component 1 of 45-1 and 45-1a in RAN1 feature list, while FG 39-2 has some overlap with component 3 of 45-1 and 45-1a in RAN1 feature list. However, in our view, these 2 FGs have larger impact in RAN4 RRM requirements, rather than in RAN1 spec. Corresponding open issues were captured in previous agreed WF [1]. Therefore, it seems more suitable to capture these 2 FGs in RAN4 UE feature list. Such that RAN4 can immediately update the FG detail based on new agreements.

Proposal 1: In the LS to RAN1 and RAN2, request RAN1 to remove component 1 of 45-1 and 45-1a and component 3 of 45-1 and 45-1a from RAN1 feature list. Instead, RAN4 will handle them in RAN4’s UE feature list.

- Summary

In this paper, we provide our suggestion of UE feature list on selective WIs. We also have the following proposal:

Proposal 1: In the LS to RAN1 and RAN2, request RAN1 to remove component 1 of 45-1 and 45-1a and component 3 of 45-1 and 45-1a from RAN1 feature list. Instead, RAN4 will handle them in RAN4’s UE feature list.

- Reference

[1] R4-2317330, WF on NR mobility enhancements (part 1), MediaTek inc.