3GPP TSG-RAN WG4 Meeting #109	R4-2318508

# Chicago, US, November 13 – 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on BS conformance testing requirement of network energy savings

Agenda Item:	8.34.2

Document for:	Discussion

Introduction

At the RAN#99 meeting, the revised WID of Network Energy Savings for NR was approved [1]. This contribution discusses the BS conformance testing requirements for NES technologies, with the core requirements almost complete.

Discussion

Time alignment error

The following agreement is taken from the WF [2]:

Issue 1-1: TAE

- Agree on Alternative #1:

  - Do not specify the BS TAE requirements of SSB-less operation for FR1 co-located inter-band CA.

  - Define the side condition of RTD to ensure UE performance in RRM part.

As it has been agreed that there are no additional BS TAE requirements to be defined in the RF session, it follows that no conformance test for TAE is necessary.

Proposal 1: No additional BS TAE conformance testing is needed for SSB-less operation for FR1 co-located inter-band CA.

Modulation quality

For BS supporting network energy savings, the modulation quality including EVM value of each carrier for different modulation schemes on PDSCH in [3,4] shall be met. No additional test configuration or requirements are needed.

Proposal 2: No additional BS EVM conformance testing is needed for network energy savings.

Transmit ON/OFF transient time

The Cell DTX allows the transmitter to be partially or completely activated or deactivated. Unlike TDD TX/RX ON-OFF, there is no additional interference from TX to RX during the DTX activate/ non-activate switch process. However, the transmitter could be optimized to improve the reconfiguration performance, otherwise there may be a negative impact on user experience and energy-saving effects.

Proposal 3: No additional Transmit ON/OFF transient time conformance testing is needed for DTX.

Conformance testing for spatial and power domains techniques

Spatial domains techniques mean selectively enable or disable individual or multiple antenna elements that are linked to a logical antenna port. Power domains techniques is reducing power when needed. Similar with Cell DTX, spatial and power domains techniques only occur during the transmitter period or frequency band without causing any additional interference.

Proposal 4: No additional conformance testing is needed for spatial and power domains techniques.

Conclusions

In this contribution, we discuss BS conformance testing requirements for energy savings techniques. Our proposals are as follows:

Proposal 1: No additional BS TAE conformance testing is needed for SSB-less operation for FR1 co-located inter-band CA.

Proposal 2: No additional BS EVM conformance testing is needed for network energy savings.

Proposal 3: No additional Transmit ON/OFF transient time conformance testing is needed for DTX.

Proposal 4: No additional conformance testing is needed for spatial and power domains techniques.

Reference

[1] RP-230566, WID revision: Network energy savings for NR, Huawei, RAN#99, March 20-23, 2023

[2] R4-2317738, WF on RF feasibility study of Network Energy Savings for NR, Huawei, RAN4#108bis

, Oct., 2023

[3] 3GPP TS 38.141-1 V18.2.0 (2023-06), Base Station (BS) conformance testing

Part 1: Conducted conformance testing (Release 18)

[4] 3GPP TS 38.141-2 V18.2.0 (2023-06), Base Station (BS) conformance testing

Part 2: Radiated conformance testing (Release 18)