3GPP TSG-RAN WG4 # 109	R4-2318551

Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.7.4.2

Source:	MediaTek Inc.

Title:	Simulation results of PDSCH requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#108-bis, the Way Forward document was approved [1]. We provide some simulation results of PDSCH requirements for simulation alignment for the Rel-18 work in this contribution.

# Simulation results

In this chapter we provide some simulation results of PDSCH requirements for simulation alignment.

We have simulated both Single-DCI and Multi-DCI transmission schemes for this document. Multi-DCI transmission scheme is simulated with both fully overlapping and non-overlapping modes.

In all transmission schemes, we have simulated both joint and separate processing receiver assumptions for completeness.

Channel model is TDLA30-75 for 100 MHz/120 kHz. The correlation matrix model follows agreement from the previous meeting [1]. MCS13 is used in all simulations. TX EVM is 6% as agreed. For phase noise modelling we are using Example 1 model from TR38.803 [2] assuming carrier frequency parameters for 29.55 GHz. We have simulated both time and frequency offset pairs (0.25ns, 0Hz) and (-0.0625us, 600Hz) between TRPs. In Single-DCI SDM simulations we have used single port PT-RS configuration because it seems to be feasible.

First, in Figures 1-1 to 1-8 there are simulation curves of Multi-TRP Single-DCI SDM scheme with MCS13 Rank 2 and 4 using both joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 1.

Next, in Figures 2-1 to 2-4 there are simulation curves of Multi-TRP Single-DCI SDM scheme with MCS17 Rank 2 using both joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 2.

Next, in Figures 3-1 to 3-8 there are simulation curves of Multi-TRP Multi-DCI fully overlapping scheme with MCS13 Rank 1+1 and 2+2 using joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 3.

Next, in Figures 4-1 to 4-4 there are simulation curves of Multi-TRP Multi-DCI fully overlapping scheme with MCS17 Rank 1+1 using joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 4.

Next, in Figures 5-1 to 5-8 there are simulation curves of Multi-TRP Multi-DCI non-overlapping scheme with MCS13 Rank 1+1 and 2+2 using joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 5.

Finally, in Figures 6-1 to 6-8 there are simulation curves of Multi-TRP Multi-DCI non-overlapping scheme with MCS17 Rank 1+1 and 2+2 using joint and separate processing receiver assumptions. Simulation results from all these figures are collected to Table 6.

The simulated cross-talk power ratios ρ are 0dB, -3dB, -6dB, -9dB, -12dB and -15dB. Here, ρ in [dB] is defined as 10·log10(ρ).

In all simulations we have not assumed any additional scaling to TRPs as agreed in the previous meeting.

Figure 1-1: Multi-TRP Single-DCI SDM MCS13 Rank 4, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 1-2: Multi-TRP Single-DCI SDM MCS13 Rank 4, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 1-3: Multi-TRP Single-DCI SDM MCS13 Rank 4, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 1-4: Multi-TRP Single-DCI SDM MCS13 Rank 4, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 1-5: Multi-TRP Single-DCI SDM MCS13 Rank 2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 1-6: Multi-TRP Single-DCI SDM MCS13 Rank 2, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 1-7: Multi-TRP Single-DCI SDM MCS13 Rank 2, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 1-8: Multi-TRP Single-DCI SDM MCS13 Rank 2, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 2-1: Multi-TRP Single-DCI SDM MCS17 Rank 2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 2-2: Multi-TRP Single-DCI SDM MCS17 Rank 2, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 2-3: Multi-TRP Single-DCI SDM MCS17 Rank 2, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 2-4: Multi-TRP Single-DCI SDM MCS17 Rank 2, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 3-1: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 2+2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 3-2: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 2+2, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 3-3: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 2+2. joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 3-4: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 2+2. separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 3-5: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 3-6: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 3-7: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 3-8: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 4-1: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 4-2: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 4-3: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 4-4: Multi-TRP Multi-DCI fully overlapping MCS13 Rank 1+1, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 5-1: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 2+2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 5-2: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 2+2, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 5-3: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 2+2. joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 5-4: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 2+2. separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 5-5: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 1+1, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 5-6: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 1+1, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 5-7: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 1+1, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 5-8: Multi-TRP Multi-DCI non-overlapping MCS13 Rank 1+1, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 6-1: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 2+2, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 6-2: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 2+2, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 6-3: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 2+2. joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 6-4: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 2+2. separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 6-5: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 1+1, joint processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 6-6: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 1+1, separate processing receiver assumption,
time offset 0.25us and frequency offset 0Hz

Figure 6-7: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 1+1, joint processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

Figure 6-8: Multi-TRP Multi-DCI non-overlapping MCS17 Rank 1+1, separate processing receiver assumption,
time offset -0.0625us and frequency offset 600Hz

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 14.0dB | 14.0dB | 13.8dB | 13.6dB | 13.4dB | 13.3dB |
| Rank 4 / separate processing 0.25us, 0Hz | N/A | N/A | N/A | 24.0dB | 16.2dB | 14.1dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 14.0dB | 13.9dB | 13.8dB | 13.6dB | 13.4dB | 13.3dB |
| Rank 4 / separate processing -0.0625us, 600Hz | 12.7dB | N/A | N/A | 23.9dB | 16.2dB | 14.1dB |
| Rank 2 / joint processing 0.25us, 0Hz | 7.3dB | 7.2dB | 7.1dB | 7.0dB | 6.9dB | 6.9dB |
| Rank 2 / separate processing 0.25us, 0Hz | 12.8dB | 10.6dB | 9.0dB | 7.7dB | 6.9dB | 6.4dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 7.3dB | 7.2dB | 7.1dB | 7.0dB | 7.0dB | 6.9dB |
| Rank 2 / separate processing -0.0625us, 600Hz | N/A | 10.6dB | 9.0dB | 7.7dB | 6.9dB | 6.4dB |

Table 1: Simulation result collection of Single-DCI SDM scenarios with MCS13
showing SNR needed to achieve 70% of maximum throughput.

| Rank 2 / joint processing 0.25us, 0Hz | 11.1dB | 10.9dB | 10.7dB | 10.5dB | 10.3dB | 10.2dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 2 / separate processing 0.25us, 0Hz | 17.9dB | 15.3dB | 13.5dB | 12.3dB | 11.3dB | 10.5dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 11.1dB | 10.9dB | 10.7dB | 10.5dB | 10.3dB | 10.2dB |
| Rank 2 / separate processing -0.0625us, 600Hz | 17.9 | 15.3dB | 13.5dB | 12.3dB | 11.3dB | 10.6dB |

Table 2: Simulation result collection of Single-DCI SDM scenarios with MCS13
showing SNR needed to achieve 70% of maximum throughput.

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 13.9dB | 13.6dB | 13.1dB | 12.6dB | 12.2dB | 12.0dB |
| Rank 4 / separate processing 0.25us, 0Hz | N/A | N/A | N/A | 17.9dB | 14.0dB | 12.6dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 13.8dB | 13.6dB | 13.1dB | 12.6dB | 12.3dB | 11.9dB |
| Rank 4 / separate processing -0.0625us, 600Hz | N/A | N/A | N/A | 17.6dB | 13.9dB | 12.7dB |
| Rank 2 / joint processing 0.25us, 0Hz | 6.6dB | 6.4dB | 6.1dB | 5.9dB | 5.7dB | 5.6dB |
| Rank 2 / separate processing 0.25us, 0Hz | 12.0dB | 9.8dB | 8.2dB | 6.9dB | 6.2dB | 5.7dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 6.5dB | 6.4dB | 6.1dB | 5.9dB | 5.7dB | 5.6dB |
| Rank 2 / separate processing -0.0625us, 600Hz | 12.0dB | 9.8dB | 8.2dB | 6.9dB | 6.1dB | 5.8dB |

Table 3: Simulation result collection of Multi-DCI fully overlapping scenarios with MCS13
showing SNR needed to achieve 70% of maximum throughput.

| Rank 2 / joint processing 0.25us, 0Hz | 10.7dB | 10.5dB | 10.3dB | 10.0dB | 9.8dB | 9.7dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 2 / separate processing 0.25us, 0Hz | 17.1dB | 14.6dB | 13.0dB | 11.7dB | 10.6dB | 10.0dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 10.7dB | 10.5dB | 10.2dB | 10.0dB | 9.7dB | 9.7dB |
| Rank 2 / separate processing -0.0625us, 600Hz | 17.2dB | 14.6dB | 13.0dB | 11.7dB | 10.7dB | 10.0dB |

Table 4: Simulation result collection of Multi-DCI fully overlapping scenarios with MCS17
showing SNR needed to achieve 70% of maximum throughput.

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 9.8dB | 9.9dB | 10.1dB | 10.4dB | 10.8dB | 11.1dB |
| Rank 4 / separate processing 0.25us, 0Hz | 13.9dB | 12.9dB | 12.1dB | 11.6dB | 11.4dB | 11.3dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 9.8dB | 9.9dB | 10.1dB | 10.5dB | 10.9dB | 11.1dB |
| Rank 4 / separate processing -0.0625us, 600Hz | 13.9dB | 12.8dB | 12.1dB | 11.7dB | 11.4dB | 11.3dB |
| Rank 2 / joint processing 0.25us, 0Hz | 5.1dB | 5.0dB | 5.0dB | 5.0dB | 5.0dB | 5.1dB |
| Rank 2 / joint processing 0.25us, 0Hz | 8.1dB | 6.9dB | 6.2dB | 5.7dB | 5.5dB | 5.4dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 5.1dB | 5.0dB | 5.0dB | 5.0dB | 5.1dB | 5.0dB |
| Rank 2 / separate processing -0.0625us, 600Hz | 8.1dB | 6.9dB | 6.2dB | 5.7dB | 5.5dB | 5.4dB |

Table 5: Simulation result collection of Multi-DCI non-overlapping scenarios with MCS13
showing SNR needed to achieve 70% of maximum throughput.

|  | ρ=0dB | ρ=-3dB | ρ=-6dB | ρ=-9dB | ρ=-12dB | ρ=-15dB |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 4 / joint processing 0.25us, 0Hz | 13.2dB | 13.3dB | 13.8dB | 14.5dB | 15.1dB | 15.5dB |
| Rank 4 / separate processing 0.25us, 0Hz | 17.9dB | 16.8dB | 16.2dB | 15.9dB | 15.7dB | 15.6dB |
| Rank 4 / joint processing -0.0625us, 600Hz | 13.2dB | 13.3dB | 13.8dB | 14.5dB | 15.1dB | 15.6dB |
| Rank 4 / separate processing -0.0625us, 600Hz | 17.9dB | 16.9dB | 16.2dB | 15.9dB | 15.7dB | 15.6dB |
| Rank 2 / joint processing 0.25us, 0Hz | 8.3dB | 8.4dB | 8.5dB | 8.7dB | 8.8dB | 8.9dB |
| Rank 2 / joint processing 0.25us, 0Hz | 11.7dB | 10.5dB | 9.7dB | 9.3dB | 9.0dB | 8.9dB |
| Rank 2 / joint processing -0.0625us, 600Hz | 8.3dB | 8.3dB | 8.5dB | 8.7dB | 8.8dB | 8.9dB |
| Rank 2 / separate processing -0.0625us, 600Hz | 11.7dB | 10.5dB | 9.7dB | 9.3dB | 9.0dB | 8.9dB |

Table 6: Simulation result collection of Multi-DCI non-overlapping scenarios with MCS17
showing SNR needed to achieve 70% of maximum throughput.

From simulation results collection in Tables 1-4, we can do following observations:

Observation #1: In fully overlapping schemes separate processing receiver assumption requires high isolation to work.

Observation #2: In fully overlapping schemes joint processing receiver assumption is robust with all simulated ρ values.

Observation #3: In fully overlapping schemes joint processing receiver assumption performance is better with higher isolation.

From simulation results collection in Tables 5-6, we can do following observations:

Observation #4: In non-overlapping schemes joint processing receiver assumption in low isolation gives diversity gain.

# Conclusion

In this paper we provided simulation results on the PDSCH demodulation requirements of FR2 multipanel RX downlink for performance alignment. The following observations are made:

Observation #1: In fully overlapping schemes separate processing receiver assumption requires high isolation to work.

Observation #2: In fully overlapping schemes joint processing receiver assumption is robust with all simulated ρ values.

Observation #3: In fully overlapping schemes joint processing receiver assumption performance is better with higher isolation.

Observation #4: In non-overlapping schemes joint processing receiver assumption in low isolation gives diversity gain.

# Reference

- R4-2316963, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm

- 3GPP TR 38.803 V14.3.0 (2022-03), “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”