3GPP TSG-RAN WG4 # 109	R4-2318556

Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.11.4

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario

Document for:	Discussion

# Introduction

In RAN4#108-bis, the Way Forward document for demodulation requirements was approved [1]. In RAN4#107 it was agreed that RAN4 should discuss the need for demodulation requirements only for Type-2 UEs and preclude Type-3a/3b and Type-4a/4b UE from the UE demodulation performance requirements discussion. Furthermore, for Type-2 UE requirements are agreed to be defined in NR-CA scenario only. We provide discussion with proposals for the Rel-18 work in this contribution.

# Discussion

In this chapter we discuss agreements of the previous meeting and remaining open items.

In the previous meeting it was agreed to define new Type 2 UE demodulation requirement with NR-CA scenario only. In new requirement we must assume the power difference of 25dB and received time difference of 33us under the assumption that requirements introduced under static channel.

Way forward: Rank/MCS selection

- Interested companies are encouraged to bring the PDSCH simulation results with impairments in RAN4#109 to decide the MCS index pair and corresponding SNR test point pair to define PDSCH NR-CA demodulation requirements.

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| MCS table and MCS index | FFS |
| Test metric | 70% of the maximum throughput |

|  | Rank | MCS table | MCS index |
| --- | --- | --- | --- |
| Carrier with lower power | Rank 1 | MCS table 1 | Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37) |
| Carrier with higher power | Rank 2 | MCS table 2 | Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86) |

Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB.

In the previous meetings it was agreed for NR-CA requirement to measure both PCell and SCell at the same time using 70% of the maximum throughput as the test metric. The channel model agreed to be static channel. For Type 2 UEs both carriers are using 2 receiver antennas as defined for Type-2 UE.

We have simulated static channel 2x2 simulations with all fixed MCS options in MCS tables 1 and 2 with Rank 1 and Rank 2. These results are available in separate simulation results Tdoc [2]. In the previous meeting it was agreed to use configuration combination of PCell Rank 2 with MCS table 2 and SCell Rank 1 with MCS table 1.

For Type 2 UE testing it was agreed to assume 25dB power difference between carriers. Therefore, we need to find suitable MCS pairs to achieve less than 25dB difference. In Table 1 below there are proposed MCS pairs listed. SNR difference of these proposed MCS pairs is less than 25dB for 2 combinations and more than 25dB for 2 combinations.

| SCell MCS index | Rank 1 MCS Table 1 | PCell MCS index | Rank 2 MCS Table 2 | SNR difference |
| --- | --- | --- | --- | --- |
| MCS 4 | -3.0dB | MCS 22 | 21.5dB | 24.5dB |
| MCS 5 | -2.0dB | MCS 23 | 22.9dB | 24.9dB |
| MCS 5 | -2.0dB | MCS 24 | 23.7dB | 25.7dB* |
| MCS 5 | -2.0dB | MCS 25 | 24.7dB | 26.7dB* |

Table 1: Fixed MCS mapping between Rank 1 MCS Table 1 to Rank 2 MCS Table 2.

From Table 1 we get 2 valid proposals with SNR difference < 25dB for test case configuration

- PCell: Rank 2, MCS Table 2, MCS 22 / SCell: Rank 1, MCS Table 1, MCS 4

- PCell: Rank 2, MCS Table 2, MCS 23 / SCell: Rank 1, MCS Table 1, MCS 5

From Table 1 we get 2 invalid proposals with SNR difference > 25dB for test case configuration

- PCell: Rank 2, MCS Table 2, MCS 24 / SCell: Rank 1, MCS Table 1, MCS 6

- PCell: Rank 2, MCS Table 2, MCS 25 / SCell: Rank 1, MCS Table 1, MCS 7

Corresponding SNR requirements per carrier are also listed in Table 1 without impairments. We are fine to either of 2 valid MCS pairs showing SNR difference less than 25dB.

Proposal #1: We propose 2 following test case configurations as possible test candidates

- PCell: Rank 2, MCS Table 2, MCS 22 / SCell: Rank 1, MCS Table 1, MCS 4

- PCell: Rank 2, MCS Table 2, MCS 23 / SCell: Rank 1, MCS Table 1, MCS 5

# Conclusion

In this paper we provided the view on the UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario. The following observations and proposals are made:

Proposal #1: We propose 2 following test case configurations as possible test candidates

- PCell: Rank 2, MCS Table 2, MCS 22 / SCell: Rank 1, MCS Table 1, MCS 4

- PCell: Rank 2, MCS Table 2, MCS 23 / SCell: Rank 1, MCS Table 1, MCS 5

# Reference

- R4-2316917, “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson

- R4-2318557, “Simulation results of UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario”, MediaTek Inc.