3GPP TSG-RAN WG4 # 109	R4-2318559

Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.18.1.2

Source:	MediaTek Inc.

Title:	Simulation results of Advanced receiver to cancel inter-user interference for MU-MIMO

Document for:	Discussion

# Introduction

In RAN#99, the status report of Rel-18 WI of NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined [1].  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. This work is split into two phases where the first phase studies the performance gain, reference receiver assumption, interference modelling, testability, required signalling overhead, as well as impact on other WGs. The initial receiver candidates are E-MMSE-IRC and R-ML. In the second phase it is expected to specify PDSCH demodulation requirements under MU-MIMO scenario with advanced receiver.

# Simulation parameters and results

In this chapter we are going to discuss simulation parameters in Chapter 2.1, simulation results in Chapter 2.2 and simulation analysis in Chapter 2.3.

# 2.1 Simulation parameters

In this chapter we are going to show new simulation results with XP antenna correlation with short analysis.

We have provided simulation results of 4Rx UE in MCS13 2+2 and MCS17 2+2 scenarios. Also, we are assuming the same scrambling ID for DMRS sequence for all UEs. For Modulation order of the co-scheduled UE, we have simulated all options of QPSK, 16-QAM and 64-QAM to give more complete summary of results. We have simulated only single co-scheduled UE scenarios for this meeting.

We have simulated MCS13 with XP medium and XP high correlation antenna correlation, and MCS17 with XP medium antenna correlation, by using orthogonal precoding. We have assumed full bandwidth allocation of all UEs.

Furthermore, we have assumed the same QCL assumption in all DMRS ports. We have assumed all needed co-scheduled UE information as known. The evaluation metric is SNR at 70% of maximum throughput.

In this document we compare performance of 3 receiver types as agreed in WF [2], namely

- MMSE-IRC (Rel-17 reference receiver)

- E-MMSE-IRC (Rel-18 advanced receiver)

- R-ML (Rel-18 advanced receiver)

# 2.2 Simulation results

Simulation configuration of target UE rank 2 using MCS13 with co-scheduled UE rank 2 simulations in 4RX are shown in Table 2-1. The corresponding simulation results with random orthogonal precoding are shown in Table 2-2 and Figure 2-1.

Simulation configuration of target UE rank 2 using MCS17 with co-scheduled UE rank 2 simulations in 4RX are shown in Table 2-3. The corresponding simulation results with random orthogonal precoding are shown in Table 2-4 and Figure 2-2.

Table 2-1: Test configurations for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE in 4RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank2 + CoUE Rank2 | FDD: 10/15 | 16QAM, 0.48 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal | TDLA30-10 | 4 x 4 XP Medium and XP High | 70 | Test dependent |

Table 2-2: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-XP medium | 12.2dB | 14.2dB | 14.2dB | 2.0dB | 0.0dB |
| coUE:16-QAM TDLA-XP medium | 13.8dB | 14.2dB | 14.2dB | 0.4dB | 0.0dB |
| coUE:64-QAM TDLA-XP medium | 14.2dB | 14.2dB | 14.2dB | 0.0dB | 0.0dB |
| coUE:QPSK TDLA-XP high | 19.3dB | 23.5dB | 23.5dB | 4.2dB | 0.0dB |
| coUE:16-QAM TDLA-XP high | 21.0dB | 23.5dB | 23.5dB | 2.5dB | 0.0dB |
| coUE:64-QAM TDLA-XP high | 22.5dB | 23.5dB | 23.5dB | 1.0dB | 0.0dB |

Figure 2-1: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with orthogonal precoding.

| TDLA30-10 XP Medium correlation | TDLA30-10 XP High correlation |
| --- | --- |

Table 2-3: Test configurations for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE in 4RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank2 + CoUE Rank2 | FDD: 10/15 | 64QAM, 0.43 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal | TDLA30-10 | 4 x 4 XP Medium | 70 | Test dependent |

Table 2-4: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-XP Medium | 16.8dB | 18.7dB | 18.7dB | 1.9dB | 0.0dB |
| coUE:16-QAM TDLA-XP Medium | 18.5dB | 18.7dB | 18.7dB | 0.2dB | 0.0dB |
| coUE:64-QAM TDLA-XP Medium | 19.6dB | 18.7dB | 18.7dB | -0.9dB | 0.0dB |

Figure 2-2: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with orthogonal precoding.

TDLA30-10 XP Medium correlation

# 2.3 Simulation analysis

We can do several observations from these simulations.

Observations from MCS13 2+2 scenario:

Observation #1: In MCS13 2+2 scenario with orthogonal random precoding and XP medium antenna correlation E-MMSE-IRC gain over MMSE-IRC is 0.0dB.

Observation #2: In MCS13 2+2 scenario with orthogonal random precoding and XP high antenna correlation E-MMSE-IRC gain over MMSE-IRC is 0.0dB.

Observation #3: In MCS13 2+2 scenario with orthogonal random precoding and XP medium antenna correlation R-ML gain over MMSE-IRC varies between 0.0dB and 2.0dB depending on co-scheduled UE modulation order.

Observation #4: In MCS13 2+2 scenario with orthogonal random precoding and XP high antenna correlation R-ML gain over MMSE-IRC varies between 1.0dB and 4.2dB depending on co-scheduled UE modulation order.

Observations from MCS17 2+2 scenario:

Observation #5: In MCS17 2+2 scenario with orthogonal random precoding and XP medium antenna correlation E-MMSE-IRC gain over MMSE-IRC is 0.0dB.

Observation #6: In MCS17 2+2 scenario with orthogonal random precoding and XP medium antenna correlation R-ML gain over MMSE-IRC varies between -0.9dB and 1.9dB depending on co-scheduled UE modulation order.

Common observations:

Observation #7: E-MMSE-IRC gain over MMSE-IRC does not depend on modulation order of co-scheduled UE.

Observation #8: R-ML gain over MMSE-IRC increases when modulation order of co-scheduled UE decreases.

# Conclusion

In this paper we provided the view on the advanced receiver to cancel inter-user interference for MU-MIMO. The following observations and proposals are made:

Observation #1: In MCS13 2+2 scenario with orthogonal random precoding and XP medium antenna correlation E-MMSE-IRC gain over MMSE-IRC is 0.0dB.

Observation #2: In MCS13 2+2 scenario with orthogonal random precoding and XP high antenna correlation E-MMSE-IRC gain over MMSE-IRC is 0.0dB.

Observation #3: In MCS13 2+2 scenario with orthogonal random precoding and XP medium antenna correlation R-ML gain over MMSE-IRC varies between 0.0dB and 2.0dB depending on co-scheduled UE modulation order.

Observation #4: In MCS13 2+2 scenario with orthogonal random precoding and XP high antenna correlation R-ML gain over MMSE-IRC varies between 1.0dB and 4.2dB depending on co-scheduled UE modulation order.

Observation #5: In MCS17 2+2 scenario with orthogonal random precoding and XP medium antenna correlation E-MMSE-IRC gain over MMSE-IRC is 0.0dB.

Observation #6: In MCS17 2+2 scenario with orthogonal random precoding and XP medium antenna correlation R-ML gain over MMSE-IRC varies between -0.9dB and 1.9dB depending on co-scheduled UE modulation order.

Observation #7: E-MMSE-IRC gain over MMSE-IRC does not depend on modulation order of co-scheduled UE.

Observation #8: R-ML gain over MMSE-IRC increases when modulation order of co-scheduled UE decreases.

# Reference

- RP-232250, “Status report for NR demodulation requirement evolution”, China Telecom

- R4-2316915, “WF on advanced receiver for MU-MIMO scenario”, China Telecom