3GPP TSG-RAN WG4 Meeting # 109	R4-2318583

# Chicago, US, November 13 – 17, 2023

Agenda item:	8.29.2.1

Source:	Apple

Title:	On RRM requirements for TDCP measurement

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108bis RRM impacts for R18 MIMO evolution were discussed and way forward [1] was agreed.  In this contribution we present our views on RRM requirements for TDCP measurement.

# 2. Discussion

In [1] the following agreements were made for feasibility of introducing accuracy requirements for TDCP :

Issue 2-1-1: Channel model for TDCP for feasibility study Agreement: Channel model for TDCP for feasibility study - Agree to use TDL as baseline - Other channel model (e.g., simplified/modified TDL, or CDL) can be considered if it finds not feasible with TDL channel. Issue 2-1-3: How to achieve ideal value of TDCP reporting? Agreement: How to achieve ideal value of TDCP reporting for the feasibility study - As starting point, for TDL channel, ideal value can be calculated for each delay value and over different doppler spread (fmax). - Other options (if any) can be considered. Issue 2-1-4: Is it feasible to define TDCP accuracy requirement for TDCP? Agreement: Other essential parameters for the feasibility study of defining TDCP accuracy requirement - Duration between TRS symbols - Doppler spread fmax for TDL - Doppler can be additionally considered if CDL is to be considered. - SNR - Number of averaging samples: one shot as baseline, other UE implementation not precluded for the feasibility study. - Channel bandwidth - Reference channel estimation algorithm - Further study on feasibility study on TDCP accuracy requirements based on existing evaluation results and additional evaluation results in RAN4#109 meeting. Make a conclusion of feasibility study in RAN4#109 meeting. - Draft CR on TDCP is expected to be submitted in the next meeting, and whether it can be agreeable depending on the conclusion of feasibility study. Understanding of other parameters: - Wideband measurement based on RAN1 definition

Based on the agreements in previous meeting, the feasibility study and evaluation are limited to TDL channel model. Also, the ideal or genie value is only for a specific channel model. The accuracy requirements specified in RRM are typically agnostic to propagation channel condition and applicable to a range of SNR, although the test case might be defined for a certain channel model and SNR. If in case it is determined that it is feasible to define inaccuracy requirements for TDCP measurement based on certain channel model- TDL in this case, such accuracy requirements would only be for that channel model and not applicable in real deployment. Such a requirement is inadequate and not useful, as UE meeting the accuracy requirement in the test setup wouldn’t ensure accuracy in actual deployment.

- The accuracy requirements for RRM are agnostic to propagation channel condition and applicable to a range of SNR.

- The feasibility study of defining accuracy requirements for TDCP measurement is limited to TDL channel model.

- Defining an accuracy requirement for TDCP measurement in a certain channel model is inadequate and not useful in real deployment.

Another issue is with testability. We typically have performance tests for accuracy in static channel condition, since the measurement could change based on the fading channel and we cannot determine if the error is due to instantaneous channel condition or measurement error.

- Testability issue with defining such accuracy requirements as static channel conditions are used for accuracy requirements and TDCP measurement accuracy would need a TDL channel.

- Using a fading channel for accuracy requirement test might lead to false errors, as the measurement can change with channel conditions.

Based on the above observations, we don’t think it is feasible to define accuracy requirements for TDCP measurement and propose not to introduce any accuracy requirements for TDCP measurement.

It was agreed to study feasibility based on some simulation in TDL channel. The simulation parameters for the feasibility study are captured below:

Link level simulation assumptions for evaluating TRS based TDCP measurements in NR Parameter; Value. Delay (between TRS symbols); 1slot. Channel model; TDL-A, delay spread=30ns. Doppler Spread; 10, 30, 75, 100, 200, 300. SNR; 5:5:20. Number of averaging samples:; one shot as baseline, 4 samples. Channel BW; 10MHz. SCS; 30KHz as baseline, 15KHz 2nd priority. Reference Channel estimation; LS CE for TRS as baseline, MMSE CE as 2nd priority. Correlation matrix and antenna configuration; 1x2 Low. Simulation results to be considered: - CDF curves of (including the 90% and 50% and 10% CDF points) for estimated TDCP. TDCP calculation reference: ct,∆t=n=0N-1hnt+∆t∙hn*tnhnt+∆t2∙nhnt2

The simulation assumptions for the feasibility are provided in the table below:

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1slot |
| Channel model | TDLA, delay spread=30ns TDLB, delay spread=30ns |
| Doppler Spread | 10, 30, 75, 100, 200, 300 |
| SNR | 5:5:20 |
| Number of samples | One shot measurement |
| Channel BW | 10MHz |
| SCS | 30KHz |
| Reference Channel estimation | LS CE for TRS |
| Correlation matrix and antenna configuration | 1x2 Low |

TDLA-30

TDLB-100

Companies were suggested to provide the median, 10,90 percentile points of the TDCP measurement. In the tables below we provide the unquantized TDCP amplitude statistics.

Table 1: Statistics for TDLA-30 with 30KHz SCS

| Doppler | SNR | 10 Perc | 90 Perc | Median |
| --- | --- | --- | --- | --- |
| 10 | 5 | 0.5037 | 0.8272 | 0.7028 |
|  | 10 | 0.7367 | 0.9337 | 0.8715 |
|  | 15 | 0.8901 | 0.9754 | 0.9516 |
|  | 20 | 0.9572 | 0.9903 | 0.9809 |
| 75 | 5 | 0.4766 | 0.8193 | 0.6613 |
|  | 10 | 0.6874 | 0.9295 | 0.8465 |
|  | 15 | 0.8476 | 0.9727 | 0.9375 |
|  | 20 | 0.9279 | 0.9876 | 0.9712 |
| 100 | 5 | 0.4577 | 0.8085 | 0.654 |
|  | 10 | 0.6763 | 0.9251 | 0.8375 |
|  | 15 | 0.8306 | 0.9696 | 0.9293 |
|  | 20 | 0.9028 | 0.9846 | 0.964 |
| 200 | 5 | 0.4273 | 0.7923 | 0.6182 |
|  | 10 | 0.6138 | 0.9119 | 0.8002 |
|  | 15 | 0.7195 | 0.9596 | 0.895 |
|  | 20 | 0.7657 | 0.9769 | 0.9322 |
| 300 | 5 | 0.386 | 0.7702 | 0.5976 |
|  | 10 | 0.553 | 0.89 | 0.7648 |
|  | 15 | 0.65 | 0.9425 | 0.8555 |
|  | 20 | 0.6928 | 0.9617 | 0.8913 |

Table 2: Statistics for TDLB-100 with 30KHz SCS

| Doppler | SNR | 10 Perc | 90 Perc | Median |
| --- | --- | --- | --- | --- |
| 10 | 5 | 0.5939 | 0.8165 | 0.7253 |
|  | 10 | 0.8113 | 0.9303 | 0.8875 |
|  | 15 | 0.9250 | 0.9742 | 0.9575 |
|  | 20 | 0.9714 | 0.9902 | 0.9830 |
| 30 | 5 | 0.5882 | 0.8158 | 0.7148 |
|  | 10 | 0.8067 | 0.9293 | 0.8821 |
|  | 15 | 0.9214 | 0.9739 | 0.9545 |
|  | 20 | 0.9659 | 0.9907 | 0.9797 |
| 75 | 5 | 0.5655 | 0.8073 | 0.7057 |
|  | 10 | 0.7801 | 0.9225 | 0.8726 |
|  | 15 | 0.8970 | 0.9673 | 0.9449 |
|  | 20 | 0.9436 | 0.9853 | 0.9713 |
| 100 | 5 | 0.5628 | 0.8000 | 0.6984 |
|  | 10 | 0.7689 | 0.9175 | 0.8628 |
|  | 15 | 0.8833 | 0.9629 | 0.9366 |
|  | 20 | 0.9256 | 0.9802 | 0.9632 |
| 200 | 5 | 0.5078 | 0.7740 | 0.6609 |
|  | 10 | 0.6781 | 0.8897 | 0.8148 |
|  | 15 | 0.7672 | 0.9372 | 0.8848 |
|  | 20 | 0.8035 | 0.9556 | 0.9099 |

Based on the results, we observe a large dependence on SNR , channel model and Doppler on the TDCP measurement. Requirements if defined would need to be for a specific SNR, channel model and Doppler. Such accuracy requirements are not useful.

- Simulation results show that the TDCP measurement is very sensitive to SNR and depends on channel model and Doppler.

- It is not feasible to define accuracy requirements for TDCP measurement that would be applicable to a range of SNR, and all channel conditions.

- Do not define accuracy requirements for TDCP measurement.

# 3. Conclusion

In this paper, we provide our views on open issues on RRM requirements for TDCP measurement. Our observations and proposals are captured below:

- The accuracy requirements for RRM are agnostic to propagation channel condition and applicable to a range of SNR.

- The feasibility study of defining accuracy requirements for TDCP measurement is limited to TDL channel model.

- Defining an accuracy requirement for TDCP measurement in a certain channel model is inadequate and not useful in real deployment.

- Testability issue with defining such accuracy requirements as static channel conditions are used for accuracy requirements and TDCP measurement accuracy would need a TDL channel.

- Using a fading channel for accuracy requirement test might lead to false errors, as the measurement can change with channel conditions.

- Simulation results show that the TDCP measurement is very sensitive to SNR and depends on channel model and Doppler.

- It is not feasible to define accuracy requirements for TDCP measurement that would be applicable to a range of SNR, and all channel conditions.

- Do not define accuracy requirements for TDCP measurement.

# Reference

- R4-2317371, “WF on R18 NR MIMO RRM requirement”, Samsung